| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0380-v1 |
| --- | --- |

| Title: | CE5: Per-context CABAC initialization with double-windows (test A1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Hilmi Egilmez, Yung-Hsuan Chao, Marta Karczewicz, Vadim Seregin 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | asaid@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports simulation results for test CE5.2.1.0. This tool extends the arithmetic coding implementation in JEM, which employs for estimating binary symbol probabilities a “parallel double window” with fixed window parameters for all contexts, by allowing different pairs of window parameters optimized for each coding context. With the BMS configuration the average coding gains are 1.04%, 1.00%, and 0.92% on AI, RA, and LDP tests, respectively, and with the VTM configuration the average gains are 0.97%, 0.88%, and 0.64%.

# Introduction

The CABAC probability estimation method of this experiment was proposed in JVET-G0112, and extends the method described in [6]. Maintaining the original notation, U[k] and V[k] represent estimated probabilities scaled by 215 and converted to integers. These probability estimates are updated using the recursive equations

bin = 1    →     U[k+1] = U[k] + ((215 – U[k]) >> a),     V[k+1] = V[k] + ((215 – V[k]) >> b),

bin = 0    →              U[k+1] = U[k] – (U[k] >> a),                V[k+1] = V[k]  – (V[k]) >> b),

P[k] = (U[k] + V[k]) >> 1.

where P[k] represents the probability value used for binary arithmetic coding. The two positive integer window shift parameters a and b can be different for each context and are store in a table.

Differently from JVET-G0112, where 4 bits are used for each of the parameters a and b, in this implementation they are together stored with 4 bits with a parameter c, such that

a = c & 3,    b = a + 1 + (c >> 2)

Since the value of c for each context are never modified, in custom hardware implementation they can be stored in less expensive and power-efficient read-only memory (ROM).

# Results

The method was implemented using the CABAC engine of JVET-K0383, and a multiplication table with 16×16×8 bits (since it has about 0.01% loss compared to the JEM table with 512×64×9 bits).

Table 1.1: Comparison of the proposed method using the 16x16x8 table on BMS1.0 with VTM configuration against (left) BMS 1.0 with VTM configuration and (right) BMS1.0 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.07% | -0.68% | -0.67% | 102% | 101% | 13.14% | 46.80% | 42.07% | 12% | 48% |
| Class A2 | -1.12% | -0.52% | -0.37% | 103% | 102% | 10.86% | 22.31% | 17.23% | 11% | 54% |
| Class B | -0.94% | -0.27% | -0.23% | 103% | 103% | 9.96% | 18.30% | 23.71% | 10% | 56% |
| Class C | -0.88% | -0.51% | -0.54% | 104% | 105% | 11.62% | 18.98% | 24.34% | 10% | 67% |
| Class E | -0.91% | -0.37% | -0.29% | 104% | 103% | 14.64% | 19.80% | 23.78% | 12% | 55% |
| Overall | -0.97% | -0.45% | -0.41% | 103% | 103% | 11.79% | 24.12% | 25.84% | 11% | 56% |
| Class D | -0.83% | -0.49% | -0.49% | 103% | 104% | 9.61% | 14.88% | 17.33% | 9% | 69% |
| Class F (optional) | -0.77% | 0.13% | 0.01% | 104% | 104% | 10.90% | 19.93% | 21.00% | 11% | 64% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.90% | -0.71% | -0.62% | 103% | 100% | 19.06% | 45.84% | 48.75% | 26% | 41% |
| Class A2 | -0.92% | -0.03% | -0.02% | 102% | 101% | 23.27% | 29.03% | 20.19% | 30% | 41% |
| Class B | -0.89% | 0.25% | -0.14% | 102% | 101% | 17.42% | 29.83% | 31.82% | 26% | 45% |
| Class C | -0.83% | -0.16% | -0.39% | 102% | 102% | 13.88% | 20.14% | 23.55% | 26% | 51% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.88% | -0.11% | -0.28% | 102% | 101% | 17.98% | 30.29% | 30.68% | 27% | 45% |
| Class D | -0.65% | -0.06% | -0.34% | 102% | 102% | 15.09% | 17.82% | 19.87% | 25% | 50% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.76% | 2.17% | 2.09% | 102% | 103% | 11.25% | 14.36% | 15.35% | 27% | 53% |
| Class C | -0.71% | 0.33% | 0.14% | 102% | 102% | 9.38% | 9.90% | 10.87% | 30% | 60% |
| Class E | -0.34% | 0.88% | 0.48% | 101% | 100% | 12.33% | 11.47% | 14.40% | 30% | 61% |
| Overall | -0.64% | 1.23% | 1.04% | 102% | 102% | 10.90% | 12.15% | 13.62% | 29% | 57% |
| Class D | -0.52% | 1.46% | 1.11% | 101% | 100% | 11.87% | 12.14% | 12.23% | 34% | 63% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 1.2: Comparison of the proposed method using the 16x16x8 table on BMS1.0 with BMS configuration against (left) BMS 1.1 with VTM configuration and (right) BMS1.0 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -13.50% | -32.15% | -30.26% | 904% | 207% | -1.09% | -1.00% | -0.83% | 104% | 98% |
| Class A2 | -11.85% | -18.62% | -15.14% | 992% | 190% | -1.19% | -0.70% | -0.57% | 104% | 100% |
| Class B | -10.83% | -15.69% | -19.57% | 1012% | 182% | -1.03% | -0.03% | -0.24% | 103% | 99% |
| Class C | -11.91% | -16.35% | -19.95% | 1114% | 162% | -0.95% | -0.27% | -0.35% | 103% | 103% |
| Class E | -14.42% | -16.68% | -19.45% | 855% | 187% | -0.97% | 0.29% | 0.11% | 102% | 99% |
| Overall | -12.28% | -19.23% | -20.68% | 983% | 183% | -1.04% | -0.30% | -0.36% | 103% | 100% |
| Class D | -10.29% | -13.07% | -15.12% | 1232% | 156% | -0.87% | 0.10% | -0.08% | 104% | 103% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -17.63% | -31.39% | -33.24% | 405% | 239% | -1.04% | -0.86% | -0.78% | 102% | 98% |
| Class A2 | -20.38% | -21.95% | -16.66% | 356% | 250% | -1.04% | -0.18% | -0.15% | 104% | 102% |
| Class B | -16.40% | -22.44% | -24.01% | 396% | 223% | -1.05% | 0.37% | 0.03% | 102% | 99% |
| Class C | -13.66% | -16.77% | -19.78% | 404% | 201% | -0.87% | 0.04% | -0.54% | 102% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -16.71% | -22.62% | -23.26% | 391% | 225% | -1.00% | -0.08% | -0.32% | 102% | 100% |
| Class D | -14.08% | -15.31% | -17.17% | 421% | 205% | -0.74% | -0.30% | -0.41% | 102% | 101% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -10.04% | -8.37% | -9.42% | 345% | 170% | -0.91% | 0.18% | 0.05% | 102% | 101% |
| Class E | -12.26% | -9.46% | -11.19% | 333% | 165% | -1.10% | -0.07% | 1.08% | 100% | 100% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -11.18% | -7.23% | -8.33% | 304% | 161% | -0.56% | 2.30% | 1.32% | 102% | 102% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

# Additional Results

In the following results, the method was implemented using the CABAC engine of JVET-K0383 using a multiplication table with 8×8×8 bits.

Table 1.3: Comparison of the proposed method using the 8x8x8 table on BMS1.0 with VTM configuration against (left) BMS 1.0 with VTM configuration and (right) BMS1.0 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.05% | -0.66% | -0.65% | 102% | 100% | 13.17% | 46.83% | 42.10% | 12% | 47% |
| Class A2 | -1.10% | -0.50% | -0.35% | 103% | 103% | 10.88% | 22.33% | 17.25% | 11% | 54% |
| Class B | -0.92% | -0.25% | -0.21% | 103% | 102% | 9.98% | 18.32% | 23.74% | 10% | 56% |
| Class C | -0.86% | -0.49% | -0.52% | 104% | 105% | 11.64% | 19.00% | 24.36% | 10% | 67% |
| Class E | -0.89% | -0.35% | -0.27% | 104% | 105% | 14.66% | 19.82% | 23.81% | 12% | 55% |
| Overall | -0.95% | -0.43% | -0.39% | 103% | 103% | 11.81% | 24.14% | 25.87% | 11% | 56% |
| Class D | -0.81% | -0.47% | -0.47% | 103% | 106% | 9.64% | 14.91% | 17.35% | 9% | 70% |
| Class F (optional) | -0.75% | 0.15% | 0.03% | 104% | 105% | 10.92% | 19.95% | 21.02% | 11% | 64% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.88% | -0.69% | -0.59% | 103% | 100% | 19.08% | 45.87% | 48.78% | 26% | 41% |
| Class A2 | -0.90% | -0.01% | 0.00% | 102% | 100% | 23.30% | 29.06% | 20.22% | 30% | 41% |
| Class B | -0.87% | 0.27% | -0.11% | 102% | 101% | 17.45% | 29.86% | 31.85% | 26% | 45% |
| Class C | -0.81% | -0.13% | -0.36% | 103% | 102% | 13.91% | 20.17% | 23.58% | 26% | 51% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.86% | -0.09% | -0.25% | 102% | 101% | 18.00% | 30.32% | 30.71% | 27% | 45% |
| Class D | -0.62% | -0.03% | -0.31% | 103% | 104% | 15.12% | 17.84% | 19.90% | 25% | 51% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.74% | 2.20% | 2.12% | 102% | 102% | 11.28% | 14.39% | 15.38% | 28% | 53% |
| Class C | -0.68% | 0.35% | 0.17% | 102% | 101% | 9.41% | 9.92% | 10.90% | 30% | 60% |
| Class E | -0.31% | 0.92% | 0.51% | 101% | 98% | 12.37% | 11.51% | 14.44% | 30% | 59% |
| Overall | -0.61% | 1.26% | 1.07% | 102% | 101% | 10.93% | 12.18% | 13.65% | 29% | 57% |
| Class D | -0.49% | 1.49% | 1.14% | 102% | 100% | 11.91% | 12.18% | 12.27% | 34% | 63% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 1.4 Comparison of the proposed method using the 8x8x8 table on BMS1.0 with VTM configuration against (left) BMS 1.0 with VTM configuration and (right) BMS1.0 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -13.48% | -32.14% | -30.24% | 903% | 207% | -1.07% | -0.98% | -0.81% | 103% | 98% |
| Class A2 | -11.83% | -18.60% | -15.12% | 995% | 193% | -1.17% | -0.68% | -0.55% | 104% | 102% |
| Class B | -10.81% | -15.67% | -19.55% | 1012% | 179% | -1.01% | -0.01% | -0.22% | 103% | 98% |
| Class C | -11.88% | -16.33% | -19.93% | 1114% | 162% | -0.93% | -0.25% | -0.33% | 103% | 103% |
| Class E | -14.39% | -16.65% | -19.43% | 855% | 188% | -0.94% | 0.32% | 0.13% | 102% | 100% |
| Overall | -12.26% | -19.22% | -20.66% | 983% | 183% | -1.02% | -0.28% | -0.34% | 103% | 100% |
| Class D | -10.27% | -13.04% | -15.10% | 1225% | 156% | -0.84% | 0.13% | -0.05% | 103% | 103% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -17.61% | -31.37% | -33.22% | 405% | 239% | -1.01% | -0.84% | -0.75% | 102% | 98% |
| Class A2 | -20.36% | -21.93% | -16.64% | 356% | 248% | -1.02% | -0.16% | -0.12% | 104% | 101% |
| Class B | -16.38% | -22.42% | -23.99% | 397% | 225% | -1.02% | 0.39% | 0.06% | 102% | 100% |
| Class C | -13.64% | -16.75% | -19.75% | 403% | 201% | -0.85% | 0.06% | -0.51% | 102% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -16.69% | -22.60% | -23.24% | 391% | 225% | -0.97% | -0.05% | -0.29% | 102% | 100% |
| Class D | -14.05% | -15.28% | -17.15% | 421% | 205% | -0.71% | -0.27% | -0.39% | 102% | 101% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -11.50% | -9.13% | -9.87% | 379% | 190% | -0.87% | 1.61% | 1.73% | 102% | 99% |
| Class C | -10.02% | -8.34% | -9.40% | 345% | 169% | -0.89% | 0.21% | 0.08% | 102% | 100% |
| Class E | -12.22% | -9.43% | -11.16% | 338% | 169% | -1.07% | -0.03% | 1.12% | 101% | 102% |
| Overall | -11.19% | -8.94% | -10.03% | 357% | 178% | -0.92% | 0.73% | 1.03% | 102% | 100% |
| Class D | -11.16% | -7.20% | -8.30% | 303% | 161% | -0.53% | 2.33% | 1.35% | 102% | 102% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

# References

- J. Chen and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1),” JVET-J1002, 9th meeting, San Diego, CA, USA, April 2018.

- T. Nguyen and A. Said, “Core Experiment 5: Arithmetic Coding Engine,” JVET-J1025, 9th meeting, San Diego, CA, USA, April 2018.

- J.Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said, M. Karczewicz, H. Egilmez, V. Seregin, L. Zhang, and X. Zhao, “EE2: Arithmetic coding with context-dependent double-window adaptation response,” JVET-H0061, 8th Meeting, Macao, China, Oct. 2017.

- A. Said, M. Karczewicz, L. Zhang, V. Seregin, and X. Zhao, “Arithmetic coding with context-dependent double-window adaptation response,” JVET-G0112, 7th Meeting, Torino, Italy, July 2017.

- A. Alshin, E. Alshina, and J.-H. Park, “High precision probability estimation for CABAC,” in Proc. IEEE Visual Commun. Image Process. Conf., Kuching, Malaysia, Nov. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).