| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0382-v3 |
| --- | --- |

| Title: | CE2-related: CTU Based Adaptive Loop Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Akshay Gadde, Nan Hu, Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | gadde@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this contribution, additional mode for the adaptive loop filter in BMS is proposed. In this mode, selection of the set of filters is done for each CTU. Test results reportedly show 3.10%, 4.96%, 4.31% luma gain for AI, RA, and LB configurations respectively comparing to VTM-1.0 anchor. Additional results with low-delay ALF encoder, where the filters used for the current picture are derived from the previous coded pictures, show 3.05%, 4.72%, 4.16% luma gain for AI, RA, and LB configurations comparing to VTM-1.0 anchor.

# Introduction

BMS includes an in-loop adaptive loop filter (ALF). For the luma component, each 2x2 block in the picture is categorized into one out of 25 classes based on its directionality and quantized value of activity. Thus, each picture can use a set of 25 filters, one per class. The current picture can use a set of filters signalled for the previously coded pictures (temporal prediction filters) or new set of filters can be signalled. To reduce the number of bits required to represent the filter coefficients, different classes can be merged.

In addition, 16 fixed filters are assigned to each class. To indicate the usage of the fixed filter, a flag for each class is signaled and if required, the index of the chosen fixed filter. Even when the fixed filter is selected for a given class, the coefficients of the adaptive filter f(k,l) can still be sent for this class, in this case the coefficients of the filter, which will be applied to the reconstructed image, are derived as a sum of both sets of coefficients. Number of classes can share the same coefficients f(k,l) signaled in the bitstream even if different fixed filters were chosen for them.

In this contribution additional ALF mode is proposed. The largest filter shape used in the proposed ALF is 7x7 diamond support filters used in BMS and CE2.4.1.4. The 2x2 block classification in the proposed ALF is the same as in BMS. In this mode selected set of filters is signalled for each CTU instead of picture.

# Technology description

As in BMS, a new set of filters can be signalled for the current picture. Fixed filters may also be used to predict a new set of filters for a picture. However, instead of signaling an index of the fixed filter per class, in this mode, index of one out 16 sets of 25 fixed filters is signaled.

Each CTB has the following options:

- Use the set of filters signalled for the current picture

- Use temporal filter set derived from the previous pictures

- Use a set of fixed filters from 16 available sets (only for luma)

- No filter applied

The signaling of ALF decision per luma CTB is done as follows. First a use_alf_flag is signaled. use_alf_flag equal to 1 indicates that the CTB is filtered, otherwise CTB is not filtered. If use_alf_flag is equal to 1, then use_latest_tlayer_filter_flag is sent. If use_latest_tlayer_filter_flag is equal to 1, then the latest available filter set derived from the picture with the closet temporal ID less than or equal to the current picture’s temporal ID is used. Note that if use_latest_tlayer_filter_flag is equal to 1 and a filter set derived from the current picture is available then it will be used. Otherwise, temporal_filter_flag is sent. If temporal_filter_flag is equal to 1 then one of the other filter sets from previously coded pictures will be used to filter the CTB. The candidate list of the temporal filters sets is constructed the same way as in BMS. The index of filter set used to filter the CTB, temporal_filter_idx, is signaled using truncated binary coding. If temporal_filter_flag is equal to 0, then one of the 16 sets of fixed filters is used to filter the CTB. Each of the 16 sets of fixed filters has 25 filters, one per class. The index of the fixed filter used, fixed_filter_idx is signaled using truncated binary code.

For chroma components, each CTU has similar options except fixed filters. For each chroma CTB, a flag use_alf_chroma_flag is sent. If it equals 1 and if a filter is sent for the current picture then that filter is used to filter the chroma CTB. If use_alf_chroma_flag equals 1 and no filter is sent for the current picture, then the latest available filter derived from a previous picture with the closet temporal ID less than or equal to the current picture’s temporal ID is used. Otherwise, the chroma CTB is not filtered. If no chroma filter is sent for the current picture and no chroma filter from previously coded pictures is available, then use_alf_chroma_flag is set to 0 without signaling.

To enable low delay encoder with proposed CTU level signaling mode for ALF, the filter set derived from the current picture is not used to filter that picture. Instead, it is stored to be used to filter next pictures.

# Simulation results

The next table shows the results for proposed CTU ALF signaling mode. The anchor is BMS-1.0 in VTM configuration.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.28% | -3.25% | -3.50% | 101% | 135% |
| Class A2 | -2.90% | -3.73% | -3.69% | 100% | 126% |
| Class B | -2.67% | -2.40% | -3.03% | 100% | 130% |
| Class C | -3.12% | -2.83% | -3.62% | 100% | 122% |
| Class E | -4.81% | -6.78% | -7.70% | 100% | 129% |
| Overall | -3.10% | -3.59% | -4.13% | 100% | 128% |
| Class D | -2.41% | -2.95% | -3.49% | 100% | 116% |
| Class F (optional) |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.92% | -2.58% | -2.36% | 105% | 137% |
| Class A2 | -6.07% | -1.71% | -1.87% | 107% | 139% |
| Class B | -5.48% | -4.49% | -3.08% | 107% | 135% |
| Class C | -4.27% | -2.05% | -2.58% | 105% | 130% |
| Class E |  |  |  |  |  |
| Overall | -4.96% | -2.90% | -2.56% | 106% | 135% |
| Class D | -4.84% | -2.51% | -2.98% | 106% | 130% |
| Class F (optional) |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.32% | -3.77% | -3.03% | 106% | 134% |
| Class C | -4.03% | -2.21% | -2.26% | 104% | 129% |
| Class E | -4.65% | -4.79% | -4.61% | 116% | 143% |
| Overall | -4.31% | -3.50% | -3.17% | 108% | 134% |
| Class D | -3.63% | -1.53% | -1.48% | 102% | 134% |
| Class F (optional) |  |  |  |  |  |

Low-delay encoder results for the proposed ALF scheme are summarized in the next table. The anchor is BMS-1.0 in VTM configuration.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.25% | -3.12% | -3.36% | 103% | 132% |
| Class A2 | -2.88% | -3.62% | -3.57% | 101% | 124% |
| Class B | -2.61% | -2.28% | -2.89% | 101% | 128% |
| Class C | -3.04% | -2.72% | -3.47% | 101% | 121% |
| Class E | -4.76% | -6.63% | -7.56% | 102% | 129% |
| Overall | -3.05% | -3.47% | -3.99% | 101% | 127% |
| Class D | -2.34% | -2.71% | -3.19% | 101% | 114% |
| Class F (optional) |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.74% | -2.74% | -2.58% | 105% | 136% |
| Class A2 | -5.89% | -1.73% | -2.18% | 106% | 142% |
| Class B | -5.17% | -4.46% | -3.01% | 107% | 136% |
| Class C | -4.02% | -2.46% | -2.95% | 105% | 132% |
| Class E |  |  |  |  |  |
| Overall | -4.72% | -3.04% | -2.75% | 106% | 136% |
| Class D | -4.71% | -2.72% | -3.10% | 107% | 134% |
| Class F (optional) |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.19% | -3.71% | -3.04% | 106% | 135% |
| Class C | -3.99% | -2.45% | -2.44% | 104% | 133% |
| Class E | -4.35% | -4.76% | -4.59% | 116% | 130% |
| Overall | -4.16% | -3.55% | -3.23% | 108% | 133% |
| Class D | -3.54% | -1.97% | -1.50% | 102% | 134% |
| Class F (optional) |  |  |  |  |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).