| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0383-v2 |
| --- | --- |

| Title: | CE5: Binary Arithmetic Coding Range Update with Small Table or Short Multiplications (test B1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Hilmi Egilmez, Yung-Hsuan Chao, Marta Karczewicz, Vadim Seregin 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | asaid@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the simulation results of CE5-3.1.0. It proposes the method of JVET-H0061 to efficiently compute the range sub-interval computation required for arithmetic coding using very small tables, or directly with hardware for multiplying integers with small number of bits. Tables of sizes 16×16, and 8×8, with each table element requiring 8 bits were tested, and compared to the JEM 512x64 table (CABAC engine 1 in

# Introduction

This proposal is applicable to the extended precision binary arithmetic coding used in JEM, where range and scaled probabilities are defined as in Fig. 1.

Fig. 1 – Interval range and scaled probabilities in high-accuracy arithmetic coding

Symbol P, with 0 < P ≤ 214, is used to represent the integer with a scaled version of the estimated least-probable symbol (LPS) probability. The interval range (an integer) is represented by symbol R, which should be divided into integers RL and RM  such that

RL / R  ≈ 2-15 P,       RL + RM = R.

# Proposed solution

As described in JVET-H0061, in this proposal the probabilities are scaled using the integer function

z(P) = floor(log2(P))

This number can be computed very efficiently with “bit-scan-reverse” assembler instructions, simple hardware, or table look-up, using the same table employed for renormalization of the arithmetic coding interval. In fact, this type of operation was chosen because is already commonly supported efficiently by CABAC software and hardware implementations, for the renormalization operation.

For highest accuracy it is proposed to use two indexes of 4 bits for the table, defined by

m(P) = (P >> (z(P) – 4)) & 15,      n(R) = (R >> 4) & 15,

use byte-size table elements according to

TN [m, n] = ( (33 + 2×m) × (33+2×n) ) >> 4,

and use

RL = TN [m(P), n(R)] >> (13 – z(P)).

Note that z(P) ≤ 13 because we always have P < 214. If the probability P has been replaced by 215 – 1 – P to guarantee this condition, then the following can be used

RM = R – TN [m(P), n(R)] >> (13 – z(P)).

If indexes m and n have binary representations {b3,b2,b1,b0}2, and {c3,c2,c1,c0}2, where bn and cn represent the binary number of the n-th bit, then we can define the computation of the table entries as

TN [m, n] = TN [{b3,b2,b1,b0}2, {c3,c2,c1,c0}2] = ({1,b3,b2,b1,b0,1}2 × {1,c3,c2,c1,c0,1}2) >> 4,

which means that the value of RL can be computed using table look-up and bit shifts, or alternatively, using the product of two 6-bit integers with special form, plus bit shifts, as indicated in the figure below.

Since we have

0 ≤ m ≤ 15,    0 ≤ n ≤ 15,    68 ≤ TN ≤ 248,

the total number of bits required for storing the table is

16 × 16 × 8 = 2,048 bits.

Alternatively, a version using two indexes of 3 bits for the table was implemented in a similar manner:  byte-size table elements are defined according to

TN [m, n] = ( (17 + 2×m) × (17+2×n) ) >> 2,

and range is updated using

RL = TN [m(P), n(R)] >> (13 – z(P)).

This table requires

8 × 8 × 8 = 512 bits.

and corresponds to product on 5 bit-integers, but with only 3 bits that are variable.

# Simulation results

Table 1.1: Results for the proposed 16×16×8 table on CABAC engine 1, against (left) BMS 1.0 with VTM configuration and (right) BMS1.0 with BMS configuration, using the original 512×64×9 bits table of CABAC engine 1.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 + CABAC Engine 1 |  |  |  |  | Over BMS-1.0 + CABAC engine 1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.06% | 100% | 100% | 0.01% | -0.05% | -0.05% | 100% | 99% |
| Class A2 | 0.01% | 0.01% | -0.02% | 100% | 100% | 0.01% | 0.01% | 0.03% | 100% | 97% |
| Class B | 0.00% | 0.02% | 0.02% | 100% | 100% | 0.02% | -0.01% | -0.04% | 100% | 98% |
| Class C | 0.01% | -0.08% | 0.02% | 99% | 99% | 0.01% | 0.03% | 0.06% | 100% | 100% |
| Class E | -0.01% | -0.08% | 0.00% | 100% | 99% | 0.02% | 0.01% | -0.06% | 100% | 99% |
| Overall | 0.00% | -0.02% | 0.02% | 100% | 100% | 0.02% | 0.00% | -0.01% | 100% | 99% |
| Class D | 0.02% | -0.03% | 0.02% | 100% | 100% | 0.01% | 0.08% | 0.24% | 100% | 100% |
| Class F (optional) | 0.00% | 0.04% | 0.09% | 99% | 98% | 0.04% | -0.04% | 0.06% | 89% | 94% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 + CABAC Engine 1 |  |  |  |  | Over BMS-1.0 + CABAC engine 1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.12% | 0.02% | 101% | 101% |  |  |  |  |  |
| Class A2 | 0.06% | -0.04% | 0.05% | 100% | 100% |  |  |  |  |  |
| Class B | 0.02% | 0.05% | -0.04% | 100% | 100% | 0.00% | 0.17% | -0.04% | 100% | 100% |
| Class C | 0.02% | 0.09% | 0.10% | 100% | 99% | 0.02% | -0.01% | -0.08% | 99% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.03% | 0.05% | 0.02% | 100% | 100% |  |  |  |  |  |
| Class D | 0.03% | 0.18% | 0.16% | 99% | 98% | 0.01% | 0.06% | 0.05% | 100% | 99% |
| Class F (optional) | 0.02% | 0.06% | -0.09% | 100% | 99% | 0.03% | 0.05% | -0.04% | 103% | 102% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 + CABAC Engine 1 |  |  |  |  | Over BMS-1.0 + CABAC engine 1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.01% | 0.29% | 0.14% | 92% | 90% |  |  |  |  |  |
| Class C | 0.01% | -0.06% | -0.23% | 100% | 102% | -0.02% | 0.13% | -0.18% | 101% | 101% |
| Class E | 0.13% | 0.72% | 0.28% | 103% | 105% | -0.09% | 0.07% | 0.14% | 101% | 104% |
| Overall | 0.03% | 0.28% | 0.05% | 97% | 98% |  |  |  |  |  |
| Class D | 0.08% | 0.38% | 0.36% | 101% | 100% | -0.06% | -0.11% | -0.33% | 100% | 99% |
| Class F (optional) | -0.11% | -0.28% | -0.42% | 100% | 100% | 0.03% | 0.58% | 0.11% | 94% | 96% |

Table 1.2: Results for the proposed 8×8×8 table on CABAC engine 1, against (left) BMS 1.0 with VTM configuration and (right) BMS1.0 with BMS configuration, using the original 512×64×9 bits table of CABAC engine 1.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 + CABAC engine 1 |  |  |  |  | Over BMS + CABAC engine 1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.02% | 0.08% | 100% | 100% | 0.04% | -0.02% | -0.03% | 100% | 100% |
| Class A2 | 0.03% | 0.03% | 0.00% | 100% | 101% | 0.03% | 0.04% | 0.05% | 100% | 97% |
| Class B | 0.02% | 0.04% | 0.04% | 100% | 101% | 0.05% | 0.01% | -0.02% | 100% | 99% |
| Class C | 0.04% | -0.06% | 0.04% | 99% | 99% | 0.04% | 0.06% | 0.09% | 100% | 99% |
| Class E | 0.02% | -0.05% | 0.02% | 100% | 98% | 0.05% | 0.04% | -0.03% | 100% | 99% |
| Overall | 0.03% | 0.00% | 0.04% | 100% | 100% | 0.04% | 0.03% | 0.01% | 100% | 99% |
| Class D | 0.04% | -0.01% | 0.04% | 100% | 99% | 0.04% | 0.11% | 0.27% | 100% | 100% |
| Class F (optional) | 0.02% | 0.06% | 0.11% | 100% | 99% | 0.07% | -0.01% | 0.09% | 95% | 96% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 + CABAC engine 1 |  |  |  |  | Over BMS + CABAC engine 1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.15% | 0.05% | 101% | 101% |  |  |  |  |  |
| Class A2 | 0.09% | -0.01% | 0.07% | 100% | 101% |  |  |  |  |  |
| Class B | 0.05% | 0.08% | -0.01% | 100% | 100% | 0.03% | 0.20% | -0.01% | 100% | 100% |
| Class C | 0.05% | 0.11% | 0.12% | 100% | 99% | 0.05% | 0.01% | -0.05% | 99% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.06% | 0.08% | 0.05% | 100% | 100% |  |  |  |  |  |
| Class D | 0.06% | 0.20% | 0.18% | 99% | 98% | 0.04% | 0.09% | 0.08% | 100% | 100% |
| Class F (optional) | 0.05% | 0.08% | -0.07% | 102% | 100% | 0.06% | 0.08% | -0.01% | 103% | 102% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 + CABAC engine 1 |  |  |  |  | Over BMS + CABAC engine 1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.03% | 0.32% | 0.17% | 91% | 90% |  |  |  |  |  |
| Class C | 0.04% | -0.03% | -0.20% | 101% | 102% | 0.01% | 0.16% | -0.15% | 101% | 102% |
| Class E | 0.17% | 0.76% | 0.31% | 103% | 104% | -0.05% | 0.11% | 0.18% | 101% | 104% |
| Overall | 0.07% | 0.31% | 0.08% | 97% | 97% |  |  |  |  |  |
| Class D | 0.11% | 0.41% | 0.39% | 100% | 100% | -0.03% | -0.08% | -0.30% | 100% | 100% |
| Class F (optional) | -0.08% | -0.25% | -0.39% | 104% | 98% | 0.07% | 0.61% | 0.14% | 92% | 93% |

Table 1.3: Results for the proposed 8×8×8 table on BMS1.0 with VTM configuration against (left) BMS 1.0 with VTM configuration and (right) BMS1.0 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.82% | -0.91% | -1.17% | 100% | 100% | 13.42% | 46.46% | 41.32% | 11% | 48% |
| Class A2 | -0.95% | -0.77% | -0.90% | 102% | 103% | 11.04% | 22.00% | 16.61% | 11% | 54% |
| Class B | -0.73% | -0.85% | -0.73% | 101% | 99% | 10.19% | 17.66% | 23.17% | 10% | 54% |
| Class C | -0.66% | -0.91% | -0.69% | 101% | 102% | 11.87% | 18.52% | 24.18% | 9% | 65% |
| Class E | -0.55% | -1.14% | -0.84% | 101% | 103% | 15.05% | 18.94% | 23.18% | 12% | 54% |
| Overall | -0.74% | -0.91% | -0.84% | 101% | 101% | 12.05% | 23.59% | 25.33% | 11% | 55% |
| Class D | -0.51% | -0.83% | -0.77% | 101% | 102% | 9.96% | 14.54% | 16.97% | 9% | 68% |
| Class F (optional) | -0.41% | -0.35% | -0.19% | 102% | 104% | 11.30% | 19.21% | 20.66% | 11% | 63% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.48% | -0.78% | -1.11% | 102% | 101% | 19.55% | 45.80% | 47.95% | 26% | 41% |
| Class A2 | -0.42% | -0.48% | -0.58% | 102% | 101% | 23.89% | 28.43% | 19.50% | 30% | 41% |
| Class B | -0.34% | -0.52% | -0.69% | 101% | 101% | 18.05% | 28.83% | 31.08% | 26% | 45% |
| Class C | -0.30% | -0.79% | -0.60% | 101% | 100% | 14.49% | 19.30% | 23.26% | 26% | 50% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.37% | -0.64% | -0.73% | 101% | 101% | 18.57% | 29.60% | 30.05% | 27% | 45% |
| Class D | -0.02% | -0.39% | -0.49% | 101% | 102% | 15.83% | 17.42% | 19.70% | 25% | 50% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.40% | 0.53% | 0.28% | 101% | 101% | 11.63% | 12.47% | 13.23% | 27% | 53% |
| Class C | -0.26% | -0.73% | -0.34% | 100% | 100% | 9.88% | 8.70% | 10.27% | 30% | 59% |
| Class E | 0.15% | -0.39% | 0.24% | 100% | 102% | 12.88% | 10.06% | 14.04% | 30% | 61% |
| Overall | -0.21% | -0.12% | 0.06% | 101% | 101% | 11.36% | 10.61% | 12.45% | 29% | 57% |
| Class D | 0.07% | 0.17% | 0.39% | 101% | 100% | 12.54% | 10.57% | 11.32% | 34% | 63% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 1.4: Results for the proposed 16x16x8 table on BMS1.0 with BMS configuration against (left) BMS 1.1 with VTM configuration and (right) BMS1.0 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -13.26% | -32.57% | -30.87% | 896% | 204% | -0.82% | -1.69% | -1.81% | 103% | 97% |
| Class A2 | -11.67% | -18.81% | -15.63% | 971% | 183% | -1.00% | -0.97% | -1.20% | 102% | 97% |
| Class B | -10.63% | -16.25% | -20.07% | 995% | 180% | -0.81% | -0.72% | -0.90% | 101% | 98% |
| Class C | -11.70% | -16.61% | -20.21% | 1102% | 161% | -0.72% | -0.62% | -0.73% | 102% | 103% |
| Class E | -14.08% | -17.47% | -20.00% | 848% | 187% | -0.58% | -0.72% | -0.61% | 102% | 99% |
| Overall | -12.06% | -19.68% | -21.15% | 970% | 181% | -0.79% | -0.90% | -1.02% | 102% | 99% |
| Class D | -10.03% | -13.37% | -15.36% | 1214% | 156% | -0.58% | -0.25% | -0.36% | 103% | 103% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -17.15% | -31.49% | -33.56% | 400% | 241% | -0.48% | -1.10% | -1.35% | 101% | 99% |
| Class A2 | -19.96% | -22.13% | -17.05% | 354% | 250% | -0.53% | -0.46% | -0.64% | 103% | 102% |
| Class B | -15.85% | -23.06% | -24.52% | 392% | 223% | -0.41% | -0.48% | -0.65% | 101% | 99% |
| Class C | -13.16% | -17.32% | -20.09% | 400% | 198% | -0.31% | -0.68% | -0.94% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -16.21% | -23.03% | -23.65% | 388% | 225% | -0.42% | -0.65% | -0.86% | 102% | 100% |
| Class D | -13.42% | -15.51% | -17.31% | 417% | 202% | 0.01% | -0.56% | -0.56% | 101% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -11.03% | -10.33% | -10.65% | 374% | 189% | -0.36% | 0.20% | 0.79% | 101% | 98% |
| Class C | -9.43% | -8.66% | -9.98% | 341% | 169% | -0.25% | -0.11% | -0.55% | 101% | 100% |
| Class E | -11.46% | -9.87% | -13.22% | 334% | 167% | -0.23% | -0.59% | -1.22% | 100% | 101% |
| Overall | -10.61% | -9.66% | -11.07% | 353% | 177% | -0.29% | -0.10% | -0.16% | 101% | 100% |
| Class D | -10.55% | -8.21% | -9.30% | 301% | 157% | 0.14% | 1.06% | 0.12% | 101% | 99% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 1.5: Results for the proposed 8x8x8 table on BMS1.0 with VTM configuration against (left) BMS 1.0 with VTM configuration and (right) BMS1.0 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.80% | -0.89% | -1.15% | 101% | 101% | 13.44% | 46.49% | 41.35% | 12% | 48% |
| Class A2 | -0.93% | -0.75% | -0.87% | 102% | 104% | 11.06% | 22.03% | 16.63% | 11% | 55% |
| Class B | -0.71% | -0.83% | -0.71% | 101% | 101% | 10.22% | 17.68% | 23.20% | 10% | 55% |
| Class C | -0.64% | -0.89% | -0.67% | 102% | 102% | 11.89% | 18.55% | 24.20% | 9% | 65% |
| Class E | -0.53% | -1.11% | -0.81% | 101% | 101% | 15.08% | 18.97% | 23.21% | 12% | 53% |
| Overall | -0.71% | -0.89% | -0.82% | 101% | 102% | 12.08% | 23.62% | 25.35% | 11% | 56% |
| Class D | -0.49% | -0.81% | -0.75% | 101% | 102% | 9.98% | 14.57% | 17.00% | 9% | 67% |
| Class F (optional) | -0.39% | -0.33% | -0.17% | 103% | 105% | 11.32% | 19.23% | 20.69% | 11% | 64% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | -0.75% | -1.09% | 102% | 101% | 19.59% | 45.83% | 47.99% | 26% | 41% |
| Class A2 | -0.39% | -0.45% | -0.56% | 102% | 102% | 23.93% | 28.47% | 19.53% | 30% | 41% |
| Class B | -0.31% | -0.49% | -0.66% | 101% | 101% | 18.09% | 28.86% | 31.11% | 26% | 45% |
| Class C | -0.27% | -0.77% | -0.57% | 101% | 100% | 14.52% | 19.33% | 23.29% | 26% | 50% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.35% | -0.61% | -0.70% | 102% | 101% | 18.60% | 29.64% | 30.08% | 27% | 45% |
| Class D | 0.01% | -0.36% | -0.46% | 101% | 102% | 15.85% | 17.45% | 19.73% | 25% | 50% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.37% | 0.56% | 0.32% | 101% | 101% | 11.67% | 12.51% | 13.27% | 27% | 53% |
| Class C | -0.23% | -0.71% | -0.32% | 101% | 99% | 9.91% | 8.73% | 10.30% | 30% | 59% |
| Class E | 0.19% | -0.35% | 0.28% | 101% | 100% | 12.93% | 10.10% | 14.08% | 30% | 60% |
| Overall | -0.18% | -0.09% | 0.10% | 101% | 100% | 11.40% | 10.65% | 12.48% | 29% | 56% |
| Class D | 0.10% | 0.20% | 0.42% | 101% | 100% | 12.58% | 10.60% | 11.36% | 34% | 63% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 1.6: Results for the proposed 8x8x8 table on BMS1.0 with BMS configuration against (left) BMS 1.1 with VTM configuration and (right) BMS1.0 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -13.24% | -32.55% | -30.85% | 899% | 206% | -0.80% | -1.66% | -1.78% | 103% | 98% |
| Class A2 | -11.65% | -18.79% | -15.61% | 975% | 184% | -0.97% | -0.94% | -1.18% | 102% | 97% |
| Class B | -10.60% | -16.22% | -20.05% | 998% | 180% | -0.78% | -0.69% | -0.87% | 101% | 99% |
| Class C | -11.68% | -16.59% | -20.18% | 1101% | 160% | -0.69% | -0.59% | -0.70% | 102% | 102% |
| Class E | -14.05% | -17.45% | -19.97% | 847% | 186% | -0.55% | -0.68% | -0.58% | 101% | 98% |
| Overall | -12.03% | -19.66% | -21.13% | 972% | 181% | -0.76% | -0.87% | -0.99% | 102% | 99% |
| Class D | -10.00% | -13.34% | -15.33% | 1215% | 155% | -0.54% | -0.21% | -0.33% | 103% | 103% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -17.13% | -31.47% | -33.54% | 401% | 240% | -0.45% | -1.07% | -1.32% | 101% | 98% |
| Class A2 | -19.93% | -22.10% | -17.02% | 353% | 249% | -0.50% | -0.43% | -0.61% | 103% | 102% |
| Class B | -15.82% | -23.04% | -24.49% | 393% | 224% | -0.38% | -0.45% | -0.62% | 101% | 100% |
| Class C | -13.14% | -17.30% | -20.07% | 400% | 199% | -0.28% | -0.66% | -0.91% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -16.19% | -23.01% | -23.63% | 388% | 225% | -0.39% | -0.62% | -0.83% | 102% | 100% |
| Class D | -13.39% | -15.49% | -17.29% | 416% | 203% | 0.04% | -0.53% | -0.53% | 101% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -11.00% | -10.30% | -10.63% | 375% | 189% | -0.32% | 0.24% | 0.83% | 101% | 98% |
| Class C | -9.41% | -8.64% | -9.96% | 341% | 171% | -0.22% | -0.08% | -0.52% | 101% | 101% |
| Class E | -11.42% | -9.84% | -13.18% | 333% | 168% | -0.19% | -0.55% | -1.19% | 99% | 102% |
| Overall | -10.58% | -9.63% | -11.04% | 352% | 178% | -0.25% | -0.07% | -0.13% | 101% | 100% |
| Class D | -10.53% | -8.18% | -9.27% | 301% | 157% | 0.17% | 1.09% | 0.15% | 101% | 99% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

# References

- J. Chen and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1),” JVET-J1002, 9th meeting, San Diego, CA, USA, April 2018.

- T. Nguyen and A. Said, “Core Experiment 5: Arithmetic Coding Engine,” JVET-J1025, 9th meeting, San Diego, CA, USA, April 2018.

- J.Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said, T. Hsieh, R. Joshi, V. Seregin, and M. Karczewicz, “Binary arithmetic coding with small table or short multiplications,” JVET-H0061, 5th Meeting, Geneva, CH, Jan. 2017.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).