| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0384-v2 |
| --- | --- |

| Title: | CE2.1.3: In-loop bilateral filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akshay Gadde, Vadim Seregin, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | gadde@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports simulation results of CE2.1.3. In the proposed bilateral filter, the intensity difference between current sample and one of its neighboring samples is replaced by a representative intensity difference between two windows covering current sample and the neighboring sample. The results show that the proposed bilateral filter provides 0.29%, 0.79% and 0.63% luma gain for VTM configuration and 0.33%, 0.54%, and 0.62% luma gain for BMS configuration, in AI, RA, and LB coding conditions respectively.

# Introduction

The bilateral filter is an edge-preserving filter that takes into account the geometric distances between pixels as well as the intensity differences between them. By considering intensity differences between pixels, the bilateral filter prevents blurring of edges in an image during filtering. JEM’s bilateral filter is a 5-tap bilateral filter which is applied on the reconstructed block when the transform has non-zero coefficients and QP is larger than 17. The strength of the filter is controlled by QP and the center weight of the filter.

# Technology description

The proposed bilateral filter is always applied to luma blocks with non-zero transform coefficients and slice quantization parameter larger than 17. Therefore, there is no need to signal the usage of the bilateral filter. Bilateral filter, if applied, is performed on decoded samples right after the inverse transform. In addition, the filter parameters, i.e., weights are explicitly derived from the coded information.

The filtering process is defined as:

P0,  0'=P0, 0+k=1KWkPk, 0-P0, 0×Pk, 0-P0, 0,

where P0,  0 is the intensity of the current sample and P0, 0' is the modified intensity of the current sample, Pk,  0 and Wk are the intensity and weighting parameter for the k-th neighboring sample, respectively. An example of one current sample and its four neighboring samples (i.e., K=4) is depicted in Figure 1.

Figure 1: Neighboring samples utilized in bilateral filter.

More specifically, the weight Wk(x) associated with the k-th neighboring sample is defined as follows:

Wk(x)=Distancek× Rangek(x)

wherein

Distancek=e- 100002σd21+4*e- 100002σd2,

Rangekx=e– x28*QP – 17*(QP – 17)

and σd is dependent on the coded mode and coding block sizes. The described filtering process is applied to intra-coded blocks, and inter-coded blocks when TU is further split, to enable parallel processing.

To further improve the coding performance, for inter-coded blocks when TU is not split, the intensity difference between current sample and one of its neighboring samples is replaced by a representative intensity difference between two windows covering current sample and the neighboring sample. Therefore, the equation of filtering process is revised to:

P0,  0'=P0, 0+k=1NWk1Mm=-M/2M/2abs(Pk, m-P0, m)×(Pk, 0-P0, 0)

wherein Pk, m and P0, m represent the m-th sample value within the windows centered at  Pk,  0 and P0,  0, respectively. In this proposal, the window size is set to 3×3. An example of two windows covering P2,  0 and P0,  0 are depicted in Figure 2.

Figure 2: Windows covering two samples utilized in weight calculation.

# Simulation results

Tables below show the gain of proposed bilateral filter on top of BMS-1.0 with VTM and BMS configurations for all intra (AI), random access (RA) and low-delay B (LDB) configurations respectively.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | 0.06% | 0.05% | 105% | 109% | -0.10% | -0.26% | -0.25% | 105% | 106% |
| Class A2 | -0.26% | -0.05% | 0.09% | 104% | 111% | -0.36% | -0.23% | 0.07% | 105% | 105% |
| Class B | -0.27% | 0.07% | -0.13% | 104% | 107% | -0.23% | -0.16% | -0.30% | 105% | 105% |
| Class C | -0.37% | -0.06% | -0.23% | 105% | 110% | -0.57% | -0.28% | -0.29% | 106% | 108% |
| Class E | -0.30% | -0.52% | -0.48% | 105% | 111% | -0.37% | -0.37% | -0.36% | 105% | 105% |
| Overall | -0.29% | -0.08% | -0.15% | 105% | 109% | -0.33% | -0.25% | -0.24% | 105% | 106% |
| Class D | -0.39% | -0.07% | -0.20% | 105% | 112% | -0.40% | 0.09% | -0.14% | 107% | 110% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.51% | -0.19% | -0.24% | 104% | 103% | -0.26% | -0.25% | -0.16% | 106% | 102% |
| Class A2 | -0.87% | -0.53% | -0.36% | 104% | 105% | -0.64% | -0.49% | -0.14% | 108% | 104% |
| Class B | -1.00% | -0.15% | -0.25% | 106% | 107% | -0.60% | -0.26% | -0.36% | 106% | 102% |
| Class C | -0.66% | -0.16% | -0.28% | 106% | 110% | -0.61% | -0.16% | -0.37% | 107% | 106% |
| Overall | -0.79% | -0.24% | -0.28% | 105% | 107% | -0.54% | -0.27% | -0.28% | 107% | 103% |
| Class D | -0.59% | -0.04% | -0.35% | 105% | 118% | -0.52% | -0.17% | -0.11% | 107% | 109% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | -0.73% | 0.39% | 0.54% | 106% | 109% | -0.45% | 0.14% | 0.66% | 107% | 103% |
| Class C | -0.58% | 0.32% | 0.46% | 106% | 114% | -0.73% | 0.59% | 0.26% | 107% | 105% |
| Class E | -0.51% | 0.11% | 0.18% | 105% | 103% | -0.74% | -0.22% | 0.08% | 104% | 103% |
| Overall | -0.63% | 0.30% | 0.42% | 105% | 109% | -0.62% | 0.20% | 0.38% | 106% | 104% |
| Class D | -0.65% | 0.30% | -0.17% | 106% | 119% | -0.75% | 1.21% | 0.41% | 106% | 110% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).