| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0384-v2 |
| --- | --- |

| Title: | CE2.1.3: In-loop bilateral filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akshay Gadde, Vadim Seregin, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | gadde@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports simulation results of CE2.1.3. In the proposed bilateral filter, the intensity difference between current sample and one of its neighboring samples is replaced by a representative intensity difference between two windows covering current sample and the neighboring sample. The results show that the proposed bilateral filter provides 0.29%, 0.79% and 0.63% luma gain for VTM configuration and 0.33%, 0.54%, and 0.62% luma gain for BMS configuration, in AI, RA, and LB coding conditions respectively.

# Introduction

The bilateral filter is an edge-preserving filter that takes into account the geometric distances between pixels as well as the intensity differences between them. By considering intensity differences between pixels, the bilateral filter prevents blurring of edges in an image during filtering. JEM’s bilateral filter is a 5-tap bilateral filter which is applied on the reconstructed block when the transform has non-zero coefficients and QP is larger than 17. The strength of the filter is controlled by QP and the center weight of the filter.

# Technology description

The proposed bilateral filter is always applied to luma blocks with non-zero transform coefficients and slice quantization parameter larger than 17. Therefore, there is no need to signal the usage of the bilateral filter. Bilateral filter, if applied, is performed on decoded samples right after the inverse transform. In addition, the filter parameters, i.e., weights are explicitly derived from the coded information.

The filtering process is defined as:

P0,  0'=P0, 0+k=1KWkPk, 0-P0, 0×Pk, 0-P0, 0,

where P0,  0 is the intensity of the current sample and P0, 0' is the modified intensity of the current sample, Pk,  0 and Wk are the intensity and weighting parameter for the k-th neighboring sample, respectively. An example of one current sample and its four neighboring samples (i.e., K=4) is depicted in Figure 1.

Figure 1: Neighboring samples utilized in bilateral filter.

More specifically, the weight Wk(x) associated with the k-th neighboring sample is defined as follows:

Wk(x)=Distancek× Rangek(x)

wherein

Distancek=e- 100002σd21+4*e- 100002σd2,

Rangekx=e– x28*QP – 17*(QP – 17)

and σd is dependent on the coded mode and coding block sizes. The described filtering process is applied to intra-coded blocks, and inter-coded blocks when TU is further split, to enable parallel processing.

To further improve the coding performance, for inter-coded blocks when TU is not split, the intensity difference between current sample and one of its neighboring samples is replaced by a representative intensity difference between two windows covering current sample and the neighboring sample. Therefore, the equation of filtering process is revised to:

P0,  0'=P0, 0+k=1NWk1Mm=-M/2M/2abs(Pk, m-P0, m)×(Pk, 0-P0, 0)

wherein Pk, m and P0, m represent the m-th sample value within the windows centered at  Pk,  0 and P0,  0, respectively. In this proposal, the window size is set to 3×3. An example of two windows covering P2,  0 and P0,  0 are depicted in Figure 2.

Figure 2: Windows covering two samples utilized in weight calculation.

# Storage and complexity

The filtering process in the proposed bilateral filter is implemented using a sliding window method similar to JEM/BMS-1.0. For example, the value W3×P3,0-P0,0 computed during filtering of center pixel P0,0 in Figure 1, is stored and is reused for filtering of P3,0 with a negative sign. The term W4×P4,0-P0,0 can similarly be reused for filtering of P4,0 with a negative sign. Therefore, we only need to compute two new terms for filtering each pixel. The filtering of inter-coded blocks, which uses the sum of absolute differences between 3x3 windows surrounding the pixels, is also implemented using a similar sliding window method.

In order to reduce the number of calculations, the proposed bilateral filter has been implemented using a look-up-table (LUT) which stores pre-computed filter-weights. The σd parameter in the expression for Distancek depends on the coding mode and size of the block as explained in Table 1.

Table 1 Value of σd for different block sizes and coding modes

| Min(block width, block height) | Inter mode | Intra mode |
| --- | --- | --- |
| 4 | 62 | 82 |
| 8 | 52 | 72 |
| other | 32 | 52 |

Therefore, the number of precomputed values of Distancek is 5 (since the number of unique values of σd is 5).  Rangekx depends on the QP and the pixel difference x. The bilateral filter is applied for QPs ranging from 18 to 51. For each QP, if the pixel difference is larger than a certain threshold, then Rangekx is set to 0. The thresholds for each of the 34 QPs are given below:

max_diff[34] = {11, 21, 30, 40, 49, 59, 68, 78  87, 97, 106, 116, 125, 135, 145, 154, 164,

173, 183, 192, 202, 211, 221, 230, 240, 249,259, 268, 278, 288, 297, 307, 316, 326}.

Therefore, the number of precomputed Rangekx values is 5759. The total number of values in the LUT equals the total number of products Wk(x)=Distancek× Rangek(x), which is 28795. Each of these values has 18 bits precision.

Note that the proposed bilateral filter is division-free. We do not need to normalize the sum P0,0+k=1KWkPk,0-P0,0×Pk,0-P0,0 by the sum of weights. This is in contrast to the bilateral filter in JEM/BMS-1.0, where filtering of each pixel requires normalization by the sum of weights. In JEM/BMS-1.0, this normalization (division by sum of weights) is done using two LUTs, each of size 325.

The bilateral filter in JEM/BMS-1.0 also uses an LUT to compute the range weights based on the pixel differences and QPs. If pixel difference is larger than certain threshold, then the weight is set to 0. The thresholds for each of the 34 QPs in JEM/BMS-1.0 bilateral filter are max_diff[34] = {6, 12, 18, 23, 29, 35, 41, 46, 52, 58, 64, 69, 75, 81, 87, 92, 98, 104, 110, 115, 121, 127, 133, 138, 144, 150, 156, 161, 167, 173, 179, 184, 190, 196}. Therefore, the number of precomputed range weights are 3468.

The number of entries in the LUTs of the proposed bilateral filter and JEM/BMS-1.0 bilateral filter is summarized in Table 2.

Table 2 Number of entries in LUTs used in proposed and JEM/BMS-1.0 bilateral filter

|  | LUT for weights | LUTs for division |
| --- | --- | --- |
| Proposed | 28795 | - |
| JEM/BMS-1.0 | 3468 | 325 x 2 |

The reduction in the total number of entries in the LUTs used in JEM/BMS-1.0 bilateral filter comes at the cost of increased computation. The number of addition, multiplication and bit-shit operations used in the filtering of one pixel in proposed bilateral filter (not considering the 3x3 window based bilateral filter used to filter inter blocks) and the bilateral filter in JEM/BMS-1.0 are summarized in Table 3. (The numbers in Table 3 assume a sliding window implementation explained earlier).

Table 3 Number of operations used in filtering of one pixel

|  | Proposed | JEM/BMS-1.0 |
| --- | --- | --- |
| #additions | 7 | 8 |
| #multiplications | 2 | 3 |
| #bitshifts | 1 | 2 |

The LUT size in the proposed bilateral filter can be reduced using the idea presented in JVET-K0274, where LUT for range weights is stored only for certain QP values. LUT for other QPs can be obtained with the stored LUTs using bit-shifts.

# Simulation results

Tables below show the gain of proposed bilateral filter on top of BMS-1.0 with VTM and BMS configurations for all intra (AI), random access (RA) and low-delay B (LDB) configurations respectively.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | 0.06% | 0.05% | 105% | 109% | -0.10% | -0.26% | -0.25% | 105% | 106% |
| Class A2 | -0.26% | -0.05% | 0.09% | 104% | 111% | -0.36% | -0.23% | 0.07% | 105% | 105% |
| Class B | -0.27% | 0.07% | -0.13% | 104% | 107% | -0.23% | -0.16% | -0.30% | 105% | 105% |
| Class C | -0.37% | -0.06% | -0.23% | 105% | 110% | -0.57% | -0.28% | -0.29% | 106% | 108% |
| Class E | -0.30% | -0.52% | -0.48% | 105% | 111% | -0.37% | -0.37% | -0.36% | 105% | 105% |
| Overall | -0.29% | -0.08% | -0.15% | 105% | 109% | -0.33% | -0.25% | -0.24% | 105% | 106% |
| Class D | -0.39% | -0.07% | -0.20% | 105% | 112% | -0.40% | 0.09% | -0.14% | 107% | 110% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.51% | -0.19% | -0.24% | 104% | 103% | -0.26% | -0.25% | -0.16% | 106% | 102% |
| Class A2 | -0.87% | -0.53% | -0.36% | 104% | 105% | -0.64% | -0.49% | -0.14% | 108% | 104% |
| Class B | -1.00% | -0.15% | -0.25% | 106% | 107% | -0.60% | -0.26% | -0.36% | 106% | 102% |
| Class C | -0.66% | -0.16% | -0.28% | 106% | 110% | -0.61% | -0.16% | -0.37% | 107% | 106% |
| Overall | -0.79% | -0.24% | -0.28% | 105% | 107% | -0.54% | -0.27% | -0.28% | 107% | 103% |
| Class D | -0.59% | -0.04% | -0.35% | 105% | 118% | -0.52% | -0.17% | -0.11% | 107% | 109% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | -0.73% | 0.39% | 0.54% | 106% | 109% | -0.45% | 0.14% | 0.66% | 107% | 103% |
| Class C | -0.58% | 0.32% | 0.46% | 106% | 114% | -0.73% | 0.59% | 0.26% | 107% | 105% |
| Class E | -0.51% | 0.11% | 0.18% | 105% | 103% | -0.74% | -0.22% | 0.08% | 104% | 103% |
| Overall | -0.63% | 0.30% | 0.42% | 105% | 109% | -0.62% | 0.20% | 0.38% | 106% | 104% |
| Class D | -0.65% | 0.30% | -0.17% | 106% | 119% | -0.75% | 1.21% | 0.41% | 106% | 110% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).