| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0385-v1 |
| --- | --- |

| Title: | CE5-related: Context State Memory Reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Hilmi Egilmez, Yung-Hsuan Chao, Marta Karczewicz, Vadim Seregin 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | asaid@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes a method to reduce the amount of memory needed for each context to be used for binary arithmetic coding. This is accomplished by using the approach of using a single adaptation window per context. With the BMS configuration the average coding gains are x.xx%, and x.xx% on AI and RA tests, respectively, and with the VTM configuration the average gains are x.xx% and x.xx%.

# Introduction

The CABAC probability estimation method of this experiment was proposed in JVET-G0112 and JVET-H0061, utilizes two windows that are defined per context. However, this requires maintaining two different 15-bit integers with probability estimations, corresponding to a context state defined by 30 bits per context. This proposal reduces the memory requirements by using a single estimation window, but that can vary for each state.

Defining P[k] as the estimated probability for a context scaled by 215 and converted to integers, the single-window probability estimation is implemented using the recursive equations

bin = 1    →     P[k+1] = P[k] + ((215 – P[k]) >> a),

bin = 0    →              P[k+1] = P[k] – (P[k] >> a),

where >> represent binary right shifts, and positive integer window shift parameter a can be different for each context. Since 2 ≤ a ≤ 9, a table containing the parameters for all the contexts requires only 3 bits per entry. Since the table with values of parameters a for each context is never modified, in custom hardware implementation all parameters can be stored in less expensive and power-efficient read-only memory (ROM).

# Results

The method was implemented using the CABAC engine of JVET-K0383 using a multiplication table with 16×16×8 bits (since it has about 0.01% loss compared to the JEM arithmetic coding multiplication table with 512×64×9 bits).

The following tables (I to IV) present the simulation results, using the Common Test Conditions defined in JVET-H1010. Since the simulations were run on a heterogenous cluster the running times were not measured consistently.

Table I – Simulation results using proposed method on BMS1.0 with VTM configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.72% | -0.11% | -0.26% | 104% | 111% | 13.55% | 47.59% | 42.64% | 12% | 53% |
| Class A2 | -0.78% | 0.08% | -0.01% | 103% | 116% | 11.24% | 23.06% | 17.64% | 11% | 61% |
| Class B | -0.59% | 0.15% | 0.34% | 93% | 109% | 10.36% | 18.78% | 24.40% | 9% | 60% |
| Class C | -0.56% | -0.03% | 0.03% | 92% | 99% | 11.99% | 19.59% | 25.02% | 9% | 63% |
| Class E | -0.52% | 0.23% | 0.51% | 88% | 99% | 15.09% | 20.52% | 24.72% | 11% | 52% |
| Overall | -0.62% | 0.07% | 0.14% | 95% | 107% | 12.19% | 24.77% | 26.50% | 10% | 58% |
| Class D | -0.56% | 0.15% | 0.24% | 92% | 100% | 9.91% | 15.64% | 18.17% | 8% | 66% |
| Class F (optional) | -0.32% | 0.63% | 0.65% | 103% | 105% | 11.40% | 20.49% | 21.77% | 11% | 64% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -0.12% | -0.15% | 111% | 111% | 19.56% | 46.74% | 49.43% | 28% | 46% |
| Class A2 | -0.50% | 0.49% | 0.44% | 104% | 107% | 23.80% | 29.78% | 20.74% | 30% | 44% |
| Class B | -0.44% | 0.72% | 0.63% | 116% | 115% | 17.97% | 30.42% | 32.83% | 30% | 51% |
| Class C | -0.45% | 0.20% | 0.37% | 94% | 97% | 14.33% | 20.55% | 24.55% | 24% | 49% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.47% | 0.37% | 0.37% | 106% | 108% | 18.48% | 30.93% | 31.52% | 28% | 48% |
| Class D | -0.37% | 0.24% | 0.30% | 85% | 95% | 15.43% | 18.15% | 20.67% | 21% | 47% |
| Class F (optional) | -0.20% | 0.78% | 0.82% | 82% | 95% | 12.34% | 21.61% | 21.49% | 18% | 57% |

Table II – Simulation results using proposed method on BMS1.0 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -11.68% | -16.18% | -19.68% | 984% | 154% | -0.69% | -0.01% | 0.02% | 91% | 98% |
| Class E | -14.15% | -16.68% | -19.16% | 679% | 170% | -0.66% | 0.28% | 0.48% | 81% | 90% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -10.12% | -13.07% | -15.09% | 982% | 143% | -0.68% | 0.10% | -0.01% | 83% | 95% |
| Class F (optional) | -10.80% | -15.56% | -16.64% | 737% | 151% | -0.46% | 0.65% | 0.54% | 80% | 92% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -16.06% | -22.12% | -23.50% | 408% | 245% | -0.64% | 0.75% | 0.69% | 105% | 109% |
| Class C | -13.39% | -16.49% | -19.27% | 351% | 185% | -0.56% | 0.38% | 0.10% | 89% | 93% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -13.90% | -14.90% | -16.67% | 376% | 192% | -0.54% | 0.18% | 0.22% | 91% | 95% |
| Class F (optional) | -11.38% | -16.03% | -16.02% | 381% | 154% | -0.33% | 0.80% | 0.92% | 84% | 92% |

The proposed method was also tested with the technique of context initialization from previous frames (CIPF), described in JVET-K0379.

Table III – Simulation results using proposed method combined with CIPF, on BMS1.0 and using VTM configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.59% | 0.15% | 0.34% | 118% | 116% | 10.36% | 18.78% | 24.40% | 12% | 63% |
| Class C | -0.56% | -0.03% | 0.03% | 101% | 102% | 11.99% | 19.59% | 25.02% | 9% | 65% |
| Class E | -0.52% | 0.23% | 0.51% | 104% | 104% | 15.09% | 20.52% | 24.72% | 12% | 55% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.56% | 0.15% | 0.24% | 99% | 99% | 9.91% | 15.64% | 18.17% | 8% | 66% |
| Class F (optional) | -0.32% | 0.63% | 0.65% | 102% | 104% | 11.40% | 20.49% | 21.77% | 11% | 63% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.57% | -0.14% | -0.24% | 107% | 110% | 19.46% | 46.73% | 49.29% | 27% | 45% |
| Class A2 | -0.54% | 0.49% | 0.35% | 105% | 108% | 23.75% | 29.77% | 20.65% | 31% | 44% |
| Class B | -0.58% | 0.77% | 0.65% | 112% | 114% | 17.80% | 30.51% | 32.86% | 29% | 51% |
| Class C | -0.64% | 0.10% | 0.32% | 103% | 101% | 14.12% | 20.46% | 24.52% | 26% | 51% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.59% | 0.35% | 0.32% | 107% | 108% | 18.34% | 30.93% | 31.48% | 28% | 48% |
| Class D | -0.88% | 0.36% | 0.35% | 91% | 97% | 14.82% | 18.32% | 20.76% | 22% | 48% |
| Class F (optional) | -0.33% | 0.78% | 0.79% | 82% | 95% | 12.20% | 21.60% | 21.44% | 18% | 56% |

Table IV – Simulation results using proposed method combined with CIPF, on BMS1.0 and using BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -11.68% | -16.18% | -19.68% | 848% | 145% | -0.69% | -0.01% | 0.02% | 79% | 92% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -10.12% | -13.07% | -15.09% | 1016% | 131% | -0.68% | 0.10% | -0.01% | 86% | 87% |
| Class F (optional) | -10.80% | -15.56% | -16.64% | 750% | 149% | -0.46% | 0.65% | 0.54% | 81% | 91% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -16.19% | -22.24% | -23.63% | 455% | 263% | -0.79% | 0.60% | 0.54% | 118% | 117% |
| Class C | -13.64% | -16.73% | -19.49% | 393% | 199% | -0.85% | 0.09% | -0.19% | 99% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -14.41% | -15.16% | -17.00% | 420% | 205% | -1.12% | -0.14% | -0.17% | 102% | 101% |
| Class F (optional) | -11.57% | -16.16% | -16.16% | 383% | 155% | -0.55% | 0.63% | 0.74% | 85% | 93% |

# References

- J. Chen and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1),” JVET-J1002, 9th meeting, San Diego, CA, USA, April 2018.

- J.Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said, M. Karczewicz, H. Egilmez, V. Seregin, L. Zhang, and X. Zhao, “EE2: Arithmetic coding with context-dependent double-window adaptation response,” JVET-H0061, 8th Meeting, Macao, China, Oct. 2017.

- A. Said, M. Karczewicz, L. Zhang, V. Seregin, and X. Zhao, “Arithmetic coding with context-dependent double-window adaptation response,” JVET-G0112, 7th Meeting, Torino, Italy, July 2017.

- A. Alshin, E. Alshina, and J.-H. Park, “High precision probability estimation for CABAC,” in Proc. IEEE Visual Commun. Image Process. Conf., Kuching, Malaysia, Nov. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).