| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0385-v2 |
| --- | --- |

| Title: | CE5-related: Context State Memory Reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Hilmi Egilmez, Yung-Hsuan Chao, Marta Karczewicz, Vadim Seregin 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | asaid@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes a method to reduce the amount of random-access memory (RAM) and read-only memory (ROM) needed for binary arithmetic coding tools tested in CE 5.1 and CE 5.1A (JVET-K381 and JVET-K380). This is accomplished by the approach of using a single adaptation window per context. The proposed method reduces the RAM and ROM memory requirements by 15 bits and 1 bit per-context, respectively. The proposed method provides average BD-rate gains of 0.8%, 0.8% and 0.9% for AI, RA and LD over BMS, respectively, and the gains over VTM are 0.6% for AI, RA and LD coding.

# Introduction

In CE 5.1 (JVET-K381) and CE5.1A (JVET-K380), the CABAC probability estimation method utilizes two windows that are defined per-context. However, this requires maintaining two separate 15-bit integers to store probability estimates, which are updated using the following recursive equations:

bin = 1    →     U[k+1] = U[k] + ((215 – U[k]) >> a),     V[k+1] = V[k] + ((215 – V[k]) >> b),

bin = 0    →              U[k+1] = U[k] – (U[k] >> a),                V[k+1] = V[k]  – (V[k]) >> b),

where >> denotes binary right shifts. U[k] and V[k] represent the two probability estimates obtained using the window shift parameters a and b. At state k, the probability P[k] used for binary arithmetic coding is estimated as:

P[k] = (U[k] + V[k]) >> 1.

So, the CABAC probability estimation in CEs 5.1 and 5.1A requires 30 bits of random-access memory (RAM) for storing U[k] and V[k]. Also, a 4-bit read-only memory (ROM) is needed to specify window shift parameters for each of the 359 context models in BMS1.0.

# Proposed method

This proposal reduces the memory requirement in CE 5.1 (JVET-K381) and CE5.1A (JVET-K380) by using a single window-based probability estimation. So, the CABAC probability estimation requires 15 bits of RAM to store probability estimates at each state instead of 30 bits of RAM.

The single-window probability estimation is implemented using the following recursive equations:

bin = 1    →     P[k+1] = P[k] + ((215 – P[k]) >> a),

bin = 0    →              P[k+1] = P[k] – (P[k] >> a),

where a denotes window shift parameter which can be different for each context. Since the range of parameters are restricted to 2 ≤ a ≤ 9, the proposed method requires 3 bits of ROM per-context. The main advantage of this proposal is the reduction in RAM memory, which is reduced from 30 bits to 15 bits per-context as compared to the CEs 5.1 and 5.1A.

# Results

The method was implemented using the multiplication table with 16×16×8 bits, since the same table is also used in CE 5.1 and CE 5.1A.

In the following, the tables present the simulation results using the Common Test Conditions defined in JVET-H1010. Since the simulations of the proposed methods were run on a heterogenous cluster the running times were not measured consistently.

Tables 1 and 2 compare the results of CE5.1 and the proposed method under BMS and VTM configurations, respectively. To fairly compare both methods, the tool for context initialization from previous frames (CIPF), described in JVET-K0379, is also enabled with the proposed method, since it is also enabled in CE5.1.

In addition, Tables 3 and 4 compare the results of CE5.1A (testing the double-window based probability estimation) and the proposed method under BMS and VTM configurations, respectively.

Table 1: Comparison of CE5.1 and proposed method with CIPF enabled on BMS-1.0.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE5.1 Over BMS-1.0 |  |  |  |  | Proposed method with CIPF Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.09% | -1.00% | -0.83% | 102% | 95% | -0.83% | -0.54% | -0.61% | 103% | 114% |
| Class A2 | -1.19% | -0.70% | -0.57% | 102% | 96% | -0.93% | -0.14% | -0.29% | 94% | 108% |
| Class B | -1.03% | -0.03% | -0.24% | 103% | 98% | -0.75% | 0.15% | 0.23% | 94% | 110% |
| Class C | -0.95% | -0.27% | -0.35% | 104% | 103% | -0.69% | -0.01% | 0.02% | 79% | 92% |
| Class E | -0.97% | 0.29% | 0.11% | 103% | 100% | -0.66% | 0.28% | 0.48% | 83% | 90% |
| Overall | -1.04% | -0.30% | -0.36% | 103% | 99% | -0.77% | -0.03% | 0.00% | 90% | 102% |
| Class D | -0.87% | 0.10% | -0.08% | 103% | 103% | -0.68% | 0.10% | -0.01% | 86% | 87% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | CE5.1 Over BMS-1.0 |  |  |  |  | Proposed method with CIPF Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.19% | -1.10% | -0.94% | 102% | 98% | -0.76% | -0.44% | -0.29% | 127% | 122% |
| Class A2 | -1.11% | -0.28% | -0.20% | 104% | 102% | -0.67% | 0.51% | 0.21% | 128% | 125% |
| Class B | -1.20% | 0.34% | -0.03% | 103% | 100% | -0.79% | 0.60% | 0.54% | 118% | 117% |
| Class C | -1.18% | -0.17% | -0.77% | 102% | 101% | -0.85% | 0.09% | -0.19% | 99% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.17% | -0.21% | -0.44% | 103% | 100% | -0.78% | 0.24% | 0.11% | 116% | 115% |
| Class D | -1.39% | -0.37% | -0.52% | 103% | 102% | -1.12% | -0.14% | -0.17% | 102% | 101% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | CE5.1 Over BMS-1.0 |  |  |  |  | Proposed method with CIPF Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.03% | 1.35% | 1.79% | 390% | 208% | -0.60% | 1.23% | 2.35% | 105% | 108% |
| Class C | -1.18% | 0.32% | -0.13% | 336% | 165% | -0.84% | 0.31% | 0.13% | 99% | 97% |
| Class E | -1.62% | 1.06% | 3.06% | 289% | 156% | -1.40% | 0.50% | 0.96% | 86% | 94% |
| Overall | -1.23% | 0.93% | 1.47% | 344% | 179% | -0.88% | 0.74% | 1.26% | 98% | 101% |
| Class D | -1.53% | 2.31% | 0.63% | 276% | 151% | -1.25% | 2.15% | 1.77% | 92% | 95% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 2: Comparison of CE5.1 and proposed method with CIPF enabled on VTM-1.0.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE5.1 Over VTM-1.0 |  |  |  |  | Proposed method with CIPF Over VTM -1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.07% | -0.68% | -0.67% | 103% | 103% | -0.72% | -0.11% | -0.26% | 127% | 121% |
| Class A2 | -1.12% | -0.52% | -0.37% | 104% | 103% | -0.78% | 0.08% | -0.01% | 123% | 124% |
| Class B | -0.94% | -0.27% | -0.23% | 103% | 102% | -0.59% | 0.15% | 0.34% | 118% | 116% |
| Class C | -0.88% | -0.51% | -0.54% | 103% | 105% | -0.56% | -0.03% | 0.03% | 101% | 102% |
| Class E | -0.91% | -0.37% | -0.29% | 103% | 104% | -0.52% | 0.23% | 0.51% | 104% | 104% |
| Overall | -0.97% | -0.45% | -0.41% | 103% | 103% | -0.62% | 0.07% | 0.14% | 114% | 113% |
| Class D | -0.83% | -0.49% | -0.49% | 103% | 105% | -0.56% | 0.15% | 0.24% | 99% | 99% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | CE5.1 Over VTM-1.0 |  |  |  |  | Proposed method with CIPF Over VTM -1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.01% | -0.78% | -0.65% | 101% | 100% | -0.57% | -0.14% | -0.24% | 107% | 110% |
| Class A2 | -0.98% | -0.13% | -0.05% | 103% | 99% | -0.54% | 0.49% | 0.35% | 105% | 108% |
| Class B | -1.03% | 0.28% | -0.08% | 103% | 102% | -0.58% | 0.77% | 0.65% | 112% | 114% |
| Class C | -1.03% | -0.20% | -0.43% | 102% | 101% | -0.64% | 0.10% | 0.32% | 103% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.02% | -0.14% | -0.28% | 102% | 101% | -0.59% | 0.35% | 0.32% | 107% | 108% |
| Class D | -1.19% | 0.09% | -0.47% | 103% | 104% | -0.88% | 0.36% | 0.35% | 91% | 97% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | CE5.1 Over VTM-1.0 |  |  |  |  | Proposed method with CIPF Over VTM -1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.91% | 1.44% | 1.76% | 101% | 103% | -0.45% | 1.34% | 2.45% | 104% | 115% |
| Class C | -0.95% | -0.13% | -0.10% | 103% | 102% | -0.60% | 0.08% | 0.41% | 99% | 100% |
| Class E | -0.67% | 2.02% | 1.62% | 103% | 102% | -0.69% | 1.22% | 1.79% | 86% | 90% |
| Overall | -0.87% | 1.06% | 1.11% | 102% | 103% | -0.56% | 0.89% | 1.61% | 98% | 103% |
| Class D | -1.36% | 1.61% | 1.03% | 103% | 102% | -1.11% | 1.75% | 2.23% | 86% | 94% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 3: Comparison of CE5.1A and proposed method on BMS-1.0.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE5.1A Over BMS-1.0 |  |  |  |  | Proposed method Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.09% | -1.00% | -0.83% | 102% | 95% | -0.83% | -0.54% | -0.61% | 103% | 114% |
| Class A2 | -1.19% | -0.70% | -0.57% | 102% | 96% | -0.93% | -0.14% | -0.29% | 94% | 108% |
| Class B | -1.03% | -0.03% | -0.24% | 103% | 98% | -0.75% | 0.15% | 0.23% | 94% | 110% |
| Class C | -0.95% | -0.27% | -0.35% | 104% | 103% | -0.69% | -0.01% | 0.02% | 79% | 92% |
| Class E | -0.97% | 0.29% | 0.11% | 103% | 100% | -0.66% | 0.28% | 0.48% | 83% | 90% |
| Overall | -1.04% | -0.30% | -0.36% | 103% | 99% | -0.77% | -0.03% | 0.00% | 90% | 102% |
| Class D | -0.87% | 0.10% | -0.08% | 103% | 103% | -0.68% | 0.10% | -0.01% | 86% | 87% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | CE5.1A Over BMS-1.0 |  |  |  |  | Proposed method Over BMS-1.0. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.04% | -0.86% | -0.78% | 102% | 98% | -0.64% | -0.25% | -0.16% | 126% | 122% |
| Class A2 | -1.04% | -0.18% | -0.15% | 104% | 102% | -0.62% | 0.62% | 0.29% | 128% | 124% |
| Class B | -1.05% | 0.37% | 0.03% | 102% | 99% | -0.64% | 0.75% | 0.69% | 105% | 109% |
| Class C | -0.87% | 0.04% | -0.54% | 102% | 101% | -0.56% | 0.38% | 0.10% | 89% | 93% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.00% | -0.08% | -0.32% | 102% | 100% | -0.62% | 0.42% | 0.28% | 109% | 110% |
| Class D | -0.74% | -0.30% | -0.41% | 102% | 101% | -0.54% | 0.18% | 0.22% | 91% | 95% |
| Class F (optional) |  |  |  |  |  | -0.33% | 0.80% | 0.92% | 84% | 92% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | CE5.1A Over BMS-1.0 |  |  |  |  | Proposed method Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.89% | 1.58% | 1.70% | 102% | 98% | -0.50% | 1.96% | 2.86% | 106% | 110% |
| Class C | -0.91% | 0.18% | 0.05% | 102% | 101% | -0.59% | 0.55% | 0.23% | 93% | 97% |
| Class E | -1.10% | -0.07% | 1.08% | 100% | 100% | -0.87% | 0.02% | 0.68% | 92% | 94% |
| Overall | -0.95% | 0.70% | 1.00% | 102% | 99% | -0.62% | 1.01% | 1.44% | 98% | 101% |
| Class D | -0.56% | 2.30% | 1.32% | 102% | 102% | -0.38% | 2.35% | 1.84% | 88% | 92% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 4: Comparison of CE5.1A and proposed method enabled on VTM-1.0.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE5.1A Over VTM-1.0 |  |  |  |  | Proposed method Over VTM -1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.07% | -0.68% | -0.67% | 103% | 103% | -0.72% | -0.11% | -0.26% | 127% | 121% |
| Class A2 | -1.12% | -0.52% | -0.37% | 104% | 103% | -0.78% | 0.08% | -0.01% | 123% | 124% |
| Class B | -0.94% | -0.27% | -0.23% | 103% | 102% | -0.59% | 0.15% | 0.34% | 118% | 116% |
| Class C | -0.88% | -0.51% | -0.54% | 103% | 105% | -0.56% | -0.03% | 0.03% | 101% | 102% |
| Class E | -0.91% | -0.37% | -0.29% | 103% | 104% | -0.52% | 0.23% | 0.51% | 104% | 104% |
| Overall | -0.97% | -0.45% | -0.41% | 103% | 103% | -0.62% | 0.07% | 0.14% | 114% | 113% |
| Class D | -0.83% | -0.49% | -0.49% | 103% | 105% | -0.56% | 0.15% | 0.24% | 99% | 99% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | CE5.1A Over VTM-1.0 |  |  |  |  | Proposed method Over VTM -1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.90% | -0.71% | -0.62% | 103% | 100% | -0.49% | -0.12% | -0.15% | 111% | 111% |
| Class A2 | -0.92% | -0.03% | -0.02% | 102% | 101% | -0.50% | 0.49% | 0.44% | 104% | 107% |
| Class B | -0.89% | 0.25% | -0.14% | 102% | 101% | -0.44% | 0.72% | 0.63% | 116% | 115% |
| Class C | -0.83% | -0.16% | -0.39% | 102% | 102% | -0.45% | 0.20% | 0.37% | 94% | 97% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.88% | -0.11% | -0.28% | 102% | 101% | -0.47% | 0.37% | 0.37% | 106% | 108% |
| Class D | -0.65% | -0.06% | -0.34% | 102% | 102% | -0.37% | 0.24% | 0.30% | 85% | 95% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | CE5.1A Over VTM-1.0 |  |  |  |  | Proposed method Over VTM -1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.76% | 2.17% | 2.09% | 102% | 103% | -0.35% | 2.32% | 2.89% | 120% | 117% |
| Class C | -0.71% | 0.33% | 0.14% | 102% | 102% | -0.35% | 0.64% | 0.80% | 92% | 98% |
| Class E | -0.34% | 0.88% | 0.48% | 101% | 100% | -0.25% | 0.27% | 1.22% | 88% | 93% |
| Overall | -0.64% | 1.23% | 1.04% | 102% | 102% | -0.33% | 1.25% | 1.77% | 102% | 104% |
| Class D | -0.52% | 1.46% | 1.11% | 101% | 100% | -0.30% | 1.96% | 1.94% | 81% | 92% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

# Conclusion

This contribution proposes a method that reduces the memory requirements of the CABAC engines tested in CEs 5.1 and 5.1A by 15 bits for RAM and 1 bit for ROM per-context. The proposed single window solution leads to a BD-rate loss about 0.4% for AI, RA and LD coding as compared to the results in CE5.1 and CE5.1A.

# References

- J. Chen and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1),” JVET-J1002, 9th meeting, San Diego, CA, USA, April 2018.

- J.Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said, M. Karczewicz, H. Egilmez, V. Seregin, L. Zhang, and X. Zhao, “EE2: Arithmetic coding with context-dependent double-window adaptation response,” JVET-H0061, 8th Meeting, Macao, China, Oct. 2017.

- A. Said, M. Karczewicz, L. Zhang, V. Seregin, and X. Zhao, “Arithmetic coding with context-dependent double-window adaptation response,” JVET-G0112, 7th Meeting, Torino, Italy, July 2017.

- A. Alshin, E. Alshina, and J.-H. Park, “High precision probability estimation for CABAC,” in Proc. IEEE Visual Commun. Image Process. Conf., Kuching, Malaysia, Nov. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).