| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0387-v1 |
| --- | --- |

| Title: | CE13: Rotated Sphere Projection (RSP) in JVET-J0024 and JVET-J0025 (Test 8) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Anubhav Singh Akhil Konda Chirag Pujara | Email: | anubhav.23@samsung.com akhil.konda@samsung.com chirag.p@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution reports simulation results of CE13 test 8.1, 8.2 and 8.3. Rotated Sphere Projection (RSP) implementation over 360Lib6.0 merged with BMS1.0rc3 has been tested. The results are reported for Base RSP code, Base with rotation and Base with rotation, inactive region filling and blending.  Results show that RSP has luma component BD-Rate gains of approximately -10.71%, -11.49% and -10.95% for 8.1, 8.2 and 8.3 respectively for BMS configurations and -10.37%, -11.66% and -11.10% for VTM configuration over PERP, under 360 Common Test Condition (CTC) for E2EWSPSNR.

# Introduction

The simulations were performed using 360 video CTC using the code uploaded at https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE13/CE13-8/. The simulations were run with RSP implementation over 360Lib6.0 merged with BMS1.0rc3 software. For BMS results are generated using config file encoder_randomaccess_bms.cfg while for VTM results encoder_randomaccess_vtm.cfg is used.

The tests performed are as follows:

- 8.1 RSP: Rotated sphere projection in 3x2 packing arrangement. Refer to JVET-H1004 [1] for further details.

- 8.2 RSP + rotation: Rotation is implemented over base RSP. An algorithm to compute best rotation angle is implemented as described in CE13 document [JVET-J1033] and the results are compiled for both BMS and VTM.

- 8.3 RSP + rotation + inactive region filling + blending: and Third test implements the rotation, filling of inactive pixels in RSP frame and blending for recreated ERP. Figure 1 shows the representation of different components of RSP.

Figure 1. Representative image of RSP lobe, showing different regions

# Experimental Results

Snapshot of experimental results is as follows,

- 8.1 RSP:

- 8.2 RSP + rotation:

- 8.3 RSP + rotation + inactive region filling + blending:

# Observations

Rotation provides gain of around 1-1.3% (for BMS and VTM).

Inactive filling and padding, reduces the objective slightly but helps in improving the subjective quality. The gain reduces by ~0.5% for Luma component.

# Cross check

The cross-check for CE13-8.1 has been performed by Yule Sun from Zhejiang Univ. The cross checkers reported the result of cross checks are matching with the provided test results.

The cross check for CE13-8.2 and CE13-8.3 has been performed by Dr. Bang of ETRI.

# References

[1] Philippe Hanhart, Jian-Liang Lin, “Description of Core Experiment 13: Projection formats,” JVET-J1033, April 2018.

[2] P. Hanhart, J. Boyce, K. Choi, “Common test conditions and evaluation procedures for 360° video,” JVET-J1012, April 2018.

# Patent rights declaration(s)

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).