| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0388-v4 |
| --- | --- |

| Title: | CE2-related: Improvement on the implementation of adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yiming Li, Zhenzhong Chen Wuhan University Xiang Li, Shan Liu Tencent | Tel: Email: | zzchen@whu.edu.cn shanl@tencent.com |
| Source: | Tencent, Wuhan University |  |  |

_____________________________

- Abstract

This contribution presents some implementation problems about adaptive loop filter (ALF) in the current BMS1.1 software. To solve the problems, a numerical method based on LDLT decomposition is suggested, which can be used to replace the current Cholesky factorization method.

Introduction

In the process of ALF, Wiener filter is obtained by solving the equation: (1), where T, c, and v denote the auto-correlation matrix of the to-be-filtered samples, the filter coefficients, and the cross-correlation vector of the to-be-filtered and the original samples, respectively [1]. To solve this linear equation system and finally get the Wiener filter , some decomposition methods (e.g., LU decomposition, LLT decomposition, LDLT decomposition) should be used. In the current implementation, Cholesky decomposition, or called LLT decomposition is chosen, while it should be mentioned that the general algorithm for Cholesky factorization is for positive-definite matrix, which is not guaranteed in the equation. In (1), T is an auto-correlation matrix, that is positive semidefinite instead of positive definite. If a matrix is non-positive-definite, the solution of Cholesky factorization is invalid number, and the performance may be decreased.

To make sure the equation can be solved, in the current implementation, if it is found that the matrix is positive semidefinite, a small value is added, to make the new matrix become non-singular and positive-definite. The equation is represented as NewT = T + 0.0001*E, where E is the unit matrix. Although this change makes the matrix positive-definite, it is also an ill-conditioned matrix, which enlarges the error of the results. Moreover, due to the computational accuracy for the decomposition of this ill-conditioned matrix, the failure case of the factorization also exists.

In JEM 7.1, the failure case makes the solution of Wiener filter  be invalid values, that may even cause an infinite loop as mentioned in Ticket #59. In the current implementation of BMS1.1, if it is found the factorization is also failed after processing NewT = T + 0.0001*E, the solution of Wiener filter will be set to 0. Thus, the problem of infinite loop may not be triggered currently, but the wrong solution may decrease the performance.

In this contribution, we suggest using LDLT decomposition as an alternative since it has the similar characteristics compared with Cholesky factorization, e.g., numerical stability, the algorithm complexity. Besides, it is noted, LDLT is for positive semidefinite.

Solution

- Using LDLT decomposition to replace the current Cholesky factorization.

- In the current BMS 1.1, there is a bug in the function of QuantizeIntegerFilterPP(), as mentioned in Ticket #59, where “count” is not added in the loop, which may cause the infinite loop. Although it will not be triggered since the solution will not be invalid values in BMS1.1 or in our proposed method, we suggest to fix it for avoiding the potential problems as well.

Results

*The coding time is not very accurate. The encoding/decoding time should be very similar as BMS1.1.

Conclusions

In this contribution, we represent the problem of the implementation of ALF. We suggested to use LDLT factorization and fix the bug of potential infinite loop in the software.

Patent rights declaration(s)

Wuhan University and Tencent America LLC do not have any current or pending patent rights relating to the technology described in this contribution.

Reference

- C. Y. Tsai, C. Y. Chen, T. Yamakage, et al. Adaptive loop filtering for video coding. IEEE Journal of Selected Topics in Signal Processing, 2013, 7(6): 934-945.

- Appendix

One of the failure case of the Cholesky factorization:

Original Matrix T (before T + 0.0001*E):

Original Matrix v:

Solved Wiener filter c in JEM 7.1 is:

Which also makes quantCoeffSum in QuantizeIntegerFilterPP() becomes an invalid number(-2147483648), that cause a endless loop if count is not added in the loop.