| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0390-v4 |
| --- | --- |

| Title: | Rate Control for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yiming Li, Zhenzhong Chen Wuhan University Xiang Li, Shan Liu Tencent | Tel: Email: | zzhen@whu.edu.cn shanl@tencent.com |
| Source: | Tencent, Wuhan University |  |  |

_____________________________

- Abstract

This contribution presents some improvements based on the current R-lambda rate control algorithm. With the proposed algorithm, when targeting the VTM1.1 anchor bit rate, there is 2.94% (luma) and 9.03% (luma) coding efficiency improvement in low delay B and random access configuration when compared with the original rate control algorithm. The proposed algorithm includes three parts. One is splitting the skip and non-skip area when the parameters are updated in picture level. Another modification is changing the updating strategy of the rate-distortion parameters. Besides, in the current software, the GOP size is 16 in random access configuration, while the rate control method only support GOP size equals to 8. We fixed this problem as well in this contribution.

Introduction

The R-lambda rate control algorithm for HEVC, JCTVC-K0103 [1] and M0036 [2], were introduced and adopted in HM at JCT-VC. This algorithm is also integrated in the current VTM/BMS software. Based on the R-lambda rate control algorithm, we further implemented some parts based on VTM 1.1. The proposed algorithm includes three parts. One is splitting the skip and non-skip area when the parameters are updated in picture level. Another modification is changing the updating strategy of the rate-distortion parameters. Besides, in the current software, the GOP size is 16 in random access configuration, while the rate control method only support GOP size equals to 8. We fixed this problem as well in this contribution.

Solution

The modifications in this proposal are listed.

- A bug fix:

The total bits for each CTU is incorrect:

#if !ENABLE_WPP_PARALLELISM

Int actualBits = int(cs.fracBits >> SCALE_BITS);

#if BUG_FIX

actualBits -= (Int)m_uiPicTotalBits;

#endif

#endif

if ( pCfg->getUseRateCtrl() )

{

…

pRateCtrl->getRCPic()->updateAfterCTU( pRateCtrl->getRCPic()->getLCUCoded(), actualBits, actualQP, actualLambda, pcSlice->getSliceType() == I_SLICE ? 0 : pCfg->getLCULevelRC() );

…

}

where “actualBits” should be the actual bits for each CTU. While in the current implementation, it incorrectly records the total bits of previous CTUs. Thus, in this bug fix, we add

actualBits -= (Int)m_uiPicTotalBits

to fix it.

- Splitting the skip and non-skip area when the parameters are updated in picture level

In the high-resolution video, because most of the CTU are the skipped block, the average bpp is very small, and the slope of the rate-distortion (R-D) curve is thus very flat. Although the flat curve is indeed the actual average R-D curve of this current frame, it has a bad influence on the non-skip CTU. In the current rate control framework, the CTU level lambda and QP is clipped by the frame level parameters for avoiding the quality fluctuation. Although the frame level R-D curve has a flat slope, the R-D curve of non-skip CTU may be different, i.e., the lambda and QP value of this non-skip CTU may be incorrectly clipped by the frame level parameters.

To solve the problem, we propose to split the skip and non-skip CTU, and only using the average bits and lambda for these CTUs as the data for updating frame level parameters.

The default picture level updating strategy is:

Step1: collecting the average bits and lambda

Step2: Using the average bits to calculate the bits per pixel (bpp)

Step3: Using the obtained bpp and lambda to update the parameters of rate-distortion model

When allocating the target bits for each frame, the target bits is calculated by:

The proposed picture level updating strategy is:

Step1: collecting the average bits and lambda of non-skip CTUs

Step2: Using the average bits to calculate the bits per pixel (bpp) in the non-skip CTU area

Step3: Using the obtained bpp and lambda to update the parameters of rate-distortion model

When allocating the target bits for each frame, the target bits is calculated by:

- Changing the updating strategy of the rate-distortion parameters.

In the default updating strategy, rate-distortion parameters are updated by the least square method. The updating speed is controlled by another parameter. While, we do not need the R-D parameters to be smooth, our target is a stable and accurate R-D model. Thus, we choose to update the RD parameters only concerning the previous one or two frames. In this contribution, we only use the previous one frame in the same level to generate the updated parameters.

The updating strategy is based on the RD model:

As lambda is the slope of the line tangent to the R-D curve,

When , ,  are obtained after encoding, two variables C, K can be estimated with the above two equations. Thus, we can use the updated C and K as the new R-D parameters.

- Extend rate control to support GOP 16 in random access configuration

Current GOP size in random access configuration is 16, while the default rate control only support GOP=8 for random access configuration. A temporary solution in VTM1.1 is setting the bits ratio of no-referenced frames as 2 and other ratios as 10, which may have a large quality decrease since it doesn’t have a appropriate ratio for each layer. Thus, we extend the GOP size supporting in rate control.

The main problem is the target bits allocation strategy. The ratio of bits for each layer in our contribution follows the document of JCT-VC W0062 and JCT-VC X0038, where the QP offset value of each level is indicated. Following the QP offset value, and according to the relations of QP and lambda, we can obtain the ratios of lambda in different levels. And then the target bits of each frame are calculated by:

This allocation scheme only works when the initial QP is estimated. When encoding the first two GOP, the initial QP is not acquired, we simply use a predefined bits ratio table for each layer.

Results

The experiments were based on VTM1.1 and were carried out on HPC cluster in Linux platform. Due to the precision of floats, in Windows platform there may be 0.01% performance difference. Compared with the HM RC method, the proposed full strategy has 2.94% (luma) and 9.03% (luma) coding efficiency improvement in low delay B and random access configuration, as shown in Table 1. The performance of the proposed method comparing to fix QP method is also shown in Table 2.

Table 1 Proposed full strategy vs. HM RC in VTM (with bug fixed)

|  | Low delay B Main10 |  |  |  |  |  | Random Access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  | -7.97% | -12.32% | -11.18% | 99% | 99% |
| Class A2 |  |  |  |  |  |  |  | -12.91% | -14.83% | -15.17% | 93% | 95% |
| Class B |  | -3.30% | -1.77% | -1.73% | 101% | 98% |  | -9.45% | -12.82% | -12.88% | 94% | 94% |
| Class C |  | -1.06% | -0.02% | 0.79% | 101% | 101% |  | -6.41% | -7.66% | -7.67% | 98% | 97% |
| Class E |  | -4.86% | -3.60% | -3.08% | 101% | 98% |  |  |  |  |  |  |
| Overall |  | -2.94% | -1.65% | -1.23% | 101% | 99% |  | -9.03% | -11.75% | -11.61% | 96% | 96% |
| Class D |  | -0.22% | 1.67% | 1.60% | 102% | 103% |  | -8.25% | -6.82% | -7.35% | 94% | 95% |

Table 2 Proposed full strategy vs. FixQP

|  | Low delay B Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  | 12.74% | 8.48% | 8.10% | 98% | 96% |
| Class A2 |  |  |  |  |  | 5.06% | 6.03% | 5.76% | 99% | 96% |
| Class B | -2.60% | -6.34% | -7.72% | 102% | 97% | 3.13% | 3.86% | 4.36% | 102% | 98% |
| Class C | -1.78% | -4.69% | -3.99% | 100% | 98% | 3.34% | 1.14% | 1.99% | 99% | 96% |
| Class E | 1.35% | 1.59% | 2.08% | 99% | 102% |  |  |  |  |  |
| Overall | -1.34% | -3.81% | -4.03% | 101% | 99% | 5.49% | 4.49% | 4.76% | 100% | 97% |
| Class D | 0.07% | 0.20% | 0.84% | 101% | 100% | 4.36% | 12.16% | 12.21% | 99% | 97% |

In Table 3 and Table 4, they show the performance change with/without bug fixed. The main reason of the performance decrease is due to the non-hierarchical structure of RC in RA configuration (GOP size=16), which is improved when using the proposed GOP extension method (Table 5)

Table 3 HM RC (w/o bug fixed) in VTM 1.1 vs. FixQP

|  | Low delay B Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  | 22.37% | 34.74% | 34.42% | 99% | 98% |
| Class A2 |  |  |  |  |  | 21.63% | 28.81% | 30.25% | 104% | 105% |
| Class B | 0.52% | 1.84% | 1.32% | 99% | 97% | 7.30% | 20.61% | 19.15% | 108% | 108% |
| Class C | 0.05% | 0.57% | 0.76% | 99% | 97% | 5.37% | 9.13% | 9.56% | 103% | 100% |
| Class E | 21.51% | 38.01% | 35.88% | 96% | 99% |  |  |  |  |  |
| Overall | 5.61% | 10.46% | 9.77% | 98% | 97% | 12.67% | 22.01% | 21.87% | 104% | 103% |
| Class D | 0.80% | 5.41% | 5.37% | 99% | 96% | 8.85% | 17.42% | 18.23% | 103% | 99% |

Table 4 HM RC in VTM 1.1(with bug fixed) vs. FixQP

|  | Low delay B Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  | 22.66% | 24.35% | 22.27% | 99% | 97% |
| Class A2 |  |  |  |  |  | 21.20% | 24.93% | 25.25% | 106% | 101% |
| Class B | 0.42% | -4.94% | -6.40% | 100% | 99% | 13.76% | 18.54% | 18.19% | 108% | 105% |
| Class C | -0.75% | -4.67% | -4.74% | 99% | 96% | 10.60% | 9.34% | 10.26% | 100% | 99% |
| Class E | 6.48% | 5.42% | 5.38% | 98% | 105% |  |  |  |  |  |
| Overall | 1.54% | -2.26% | -2.90% | 99% | 99% | 16.18% | 18.53% | 18.30% | 104% | 101% |
| Class D | 0.27% | -1.47% | -0.77% | 99% | 97% | 14.37% | 20.46% | 20.68% | 106% | 102% |

Table 5 RA configuration: Proposed with only GOP size Extend + Bug fix vs. FixQP

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 16.00% | 6.82% | 9.29% | 96% | 96% |
| Class A2 | 6.71% | 5.57% | 5.90% | 99% | 95% |
| Class B | 4.34% | 6.64% | 6.80% | 103% | 100% |
| Class C | 4.37% | 3.24% | 3.57% | 99% | 96% |
| Class E |  |  |  |  |  |
| Overall | 7.15% | 5.56% | 6.26% | 100% | 97% |
| Class D | 5.29% | 13.62% | 13.88% | 100% | 99% |

Conclusions

In this contribution, we represent the improved rate control method. It is proposed to adopt this improved rate control method into VTM.

Patent rights declaration(s)

Wuhan University and Tencent America LLC do not have any current or pending patent rights relating to the technology described in this contribution.

Reference

- B. Li, H. Li, L. Li, and J. Zhang, “Rate control by R-lambda model for HEVC,” JCT-VC K0103, Oct. 2012.

- B. Li, H. Li, and L. Li, “Adaptive bit allocation for R-lambda model rate control in HM,” JCT-VC M0036, Apr. 2013.