| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0391-v2 |
| --- | --- |

| Title: | Dense Residual Convolutional Neural Network based In-Loop Filter for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yingbin Wang, Zhenzhong Chen, Yiming Li Wuhan University Liang Zhao, Shan Liu, Xiang Li Tencent | Tel: Email: | zzhen@whu.edu.cn shanl@tencent.com |
| Source: | Tencent, Wuhan University |  |  |

_____________________________

- Abstract

This contribution provides a dense residual convolutional network based in-loop filter (DRNLF) for VVC. In-loop filters, such as DF (deblocking filter), sample adaptive offset, are employed in VTM for suppressing compression artifacts, which contributes to coding performance improvement. In this contribution, the proposed DRNLF is introduced as an additional filter before SAO. Simulation results report -5.75%, -17.56%	, -18.74% BD-rate savings for luma, and both chroma components compared with VTM1.1 with AI configuration, and -6.11%, -15.85%, -14.36% for RA configuration, and -6.05%, -11.53%, -12.30% for LDB configuration, and -7.14%, -12.16%, -12.72% for LDP configuration.

Introduction

In-loop filtering is a key technology in VVC utilized to reduce compression artifacts and improve coding performance. In VTM, deblocking filter (DF) is applied firstly to reduce the blocking artifacts located at the boundaries of coding blocks and then sample adaptive offset (SAO) further refines the video quality by adaptively adding an predefined offset value to each sample. Both filters contribute to improvement for video quality and coding performance.

Recently, convolutional neural networks (CNNs) have shown great potential in the task of image restoration. Generally, the CNN performs the task of image restoration by learning a mapping from the degraded images to the original one. Similarly, the same process can be applied to remove the compression artifacts. Motivated by the latest advance of CNN, a dense residual convolutional network based in-loop filter (DRNLF) is proposed in this contribution. Researches have shown that the employ of DRNLF provided promising results.

Technical description

Proposed Decoding Scheme

The proposed DRNLF is introduced into VTM as an additional filter between DF and SAO, as shown in Fig. 1. The reconstructed frames are firstly filtered by DF before processed by DRN. Then SAO is employed to further reduce the artifacts in the outputs of DRN adaptively. In this contribution, separated models are trained for different QPs. During the process of in-loop filtering, the optimal one is selected according to the QP of the compressed frame. The decision of whether to apply proposed filter or not is given based the rate-distortion optimization (RDO) on block level. To signal the employ of the proposed filter on luma and both chroma component, additional flags are signaled in the bitstream respectively.

Fig. 1 Proposed decoding scheme in VTM

DRNLF

In this section, the details of the core unit in the proposed DRN are provided firstly and then the whole architecture of the network is described.

Dense Residual Unit

The dense residual unit (DRU) consists of one  convolutional layer, two  convolutional layers and an ReLU between them. Besides, there are two shortcuts in the DRU.  The inner shortcut performs the residual learning by adding up the outputs of the  convolutional layer with the last convolution layer. The outer shortcut directly passes the original input to the next unit. Let  be the input of the DRU, the output of the DRU can be formulated as

(1)

Where  denotes the operation of concatenation,  represents the  convolutional layer, represents the residual learning.

The shortcut in the unit allows the signal to bypass the convolutional layers and directly propagate to the next unit, which enables the subsequent units to explore new features from preceding inputs. The  convolutional layer is served as the bottleneck layer for saving computational resources and reduces the number of the parameters of the network model. No activation layer is appended after the bottleneck layer so that the bottleneck layer simply generates the linear combination of the inputs. Since the input and the target in the task of in-loop filtering are highly similar, most of their residuals between the input and the target are small or zero. The employ of residual learning contributes to faster convergence and easier training.

Fig. 2 Dense Residual Unit

Network architecture

The whole architecture of DRN is shown in Fig. 3. The network mainly consists of series of DRU with some 3x3 convolutional layers. To exploit the correlation between the images channels, the proposed DRN receives a RGB image as input. Apart from the last convolution layer that generates a 3-channel output, other convolutional layers generate the same amount of feature maps (denoted as M).

As described in Sec. 2.2.1, the DRU can directly propagates its input to the subsequent unit through the shortcut. With cascading DRUs in the network, the DRU can receive multi-level features from all preceding units through dense shortcuts. The 1x1 convolutional layer placed at the head of each unit as the bottleneck layer to fuse those multi-level features and save the computational resources as the same time. The global identity skip connection further facilitates the flow of shallow features. Two 3x3 convolution layers are appended in the last of the network to adaptively fuse the preceding feature maps and generate the final reconstructed images. Note that 1x1 convolutional layer is not employed in the first DRU.

Fig. 3 The architecture of DRN.

Simulation results

The proposed network is trained and validated using DIV2K dataset [2]. There are 800 images in its training set and 100 images in its validation set. The models are trained with mini-batch size of 16. The learning rate is initialized as $1 \times 10^{-4}$ and is divided by half every $3 \times 10^{4}$ mini-batch updates. The degraded images are generated by compressing the high-quality images using different QPs with All Intra (AI) configuration. In particular, the QPs of 22, 27, 32 and 37 are utilized in the compression. The network is trained on small patches of  randomly extracted from the training samples. Mean square error (MSE) is adopted as the loss function in training the model. Let  denote the image with compression artifacts, and  denote the corresponding high quality image,  denote the parameters of the proposed DRN. The proposed DRN can be represented by a function  that try to remove the artifacts in  and restore it to . The goal of the training is to estimate the optimal  that minimizes the following MSE loss:

(2)

The proposed DRNLF is implemented on VTM1.1. Table 1-4 show the BD-rate results of the proposed scheme. Note that N and M is set to 8 and 64, respectively. The original VTM1.1 [1] serves as the reference software. The results of the experiments running without GPU are also provided in Table 5. In addition, in order to investigate the potential of the proposed network, the performance of the models with different number of DRUs are compared. Particularly, the performance of the proposed scheme under AI configuration with 2, 8 and 16 DRUs is provided in Fig. 4, where higher performance can be achieved with more DRUs.

Table 1 Experimental Results of the Proposed Scheme (All Intra)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.50% | -20.59% | -18.79% | 112% | 1958% |
| Class A2 | -4.61% | -20.32% | -14.02% | 100% | 1656% |
| Class B | -4.38% | -16.77% | -20.27% | 91% | 2002% |
| Class C | -6.98% | -21.61% | -25.01% | 107% | 1145% |
| Class E | -9.77% | -7.70% | -12.49% | 108% | 1883% |
| Overall | -5.75% | -17.56% | -18.74% | 102% | 1689% |
| Class D | -6.92% | -21.98% | -24.85% | 116% | 1420% |

Table 2 Experimental Results of the Proposed Scheme (Random Access)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.69% | -15.60% | -14.56% | 112% | 4080% |
| Class A2 | -8.02% | -19.74% | -10.37% | 117% | 5585% |
| Class B | -6.09% | -14.85% | -16.56% | 124% | 3539% |
| Class C | -5.77% | -14.36% | -14.43% | 116% | 3067% |
| Class E |  |  |  |  |  |
| Overall | -6.11% | -15.85% | -14.36% | 118% | 3840% |
| Class D | -5.57% | -15.98% | -18.49% | 125% | 2197% |

Table 3 Experimental Results of the Proposed Scheme (Low delay B)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.65% | -11.70% | -13.07% | 108% | 3409% |
| Class C | -5.70% | -15.09% | -13.30% | 112% | 3114% |
| Class E | -8.85% | -6.49% | -9.68% | 110% | 5227% |
| Overall | -6.05% | -11.53% | -12.30% | 110% | 3680% |
| Class D | -5.54% | -13.43% | -15.52% | 113% | 2711% |

Table 4 Experimental Results of the Proposed Scheme (Low delay P)

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -6.77% | -12.32% | -13.23% | 102% | 3091% |
| Class C | -5.81% | -15.48% | -14.13% | 130% | 3143% |
| Class E | -9.55% | -7.46% | -9.99% | 117% | 4783% |
| Overall | -7.14% | -12.16% | -12.72% | 115% | 3467% |
| Class D | -6.23% | -14.12% | -16.40% | 113% | 2427% |

Table 5 Experimental Results of the Proposed Scheme (Without GPU)

|  | All Intra Main10 (CPU) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.50% | -20.59% | -18.78% | 280% | 91396% |
| Class A2 | -4.61% | -20.32% | -14.01% | 282% | 93871% |
| Class B | -4.38% | -16.77% | -20.27% | 256% | 89251% |
| Class C | -6.98% | -21.61% | -25.01% | 228% | 49238% |
| Class E | -9.77% | -7.69% | -12.49% | 330% | 108677% |
| Overall | -5.75% | -17.56% | -18.74% | 268% | 81816% |
| Class D | -6.92% | -21.98% | -24.85% | 227% | 40133% |

Fig.4 Performance of DRNLF with different number of DRUs (All Intra)

Conclusions

Motivated by the latest advances of CNN, this contributes provides a dense residual convolutional network based in-loop filter (DRNLF) for VTM. Simulation results report -5.75%, -17.56%	, -18.74% BD-rate savings for luma, and both chroma components compared with VTM1.1 with AI configuration, and -6.11%, -15.85%, -14.36% for RA configuration, and -6.05%, -11.53%, -12.30% for LDB configuration, and -7.14%, -12.16%, -12.72% for LDP configuration

Patent rights declaration(s)

Wuhan University and Tencent America LLC do not have any current or pending patent rights relating to the technology described in this contribution.

References

- VTM 1.1, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_VTM/tags/VTM-1.1/

- DIV2K, https://data.vision.ee.ethz.ch/cvl/DIV2K/.