| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0394-v1 |
| --- | --- |

| Title: | CE6-related: AMT with only type2/type4 DCT/DST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 takeshi.tsukuba@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In order to reduce the number of core transforms, this contribution proposes a modified AMT with only type2/type4 DCT/DST, which replaces 2^M-pt DCT-5, DCT-8, DST-1 and DST-7 with alternate transforms 2^M-pt DCT-2, DCT-4, DST-2 and DST-4. As a result, the number of core transform reduces 4 from 5. It is also described that matrices of 2^M-pt alternate transforms can be extracted from the matrix of larger 2^N-pt DCT-2 (M<N) based on the property of DCT-2 and relationships of DCT-4/DST-4 and DCT-2/DST-2. It would be useful to further reduce memory usage to store core transforms.

The experiments show that

- average BD-rate gains of Y over VTM are -3.29% for AI, -1.97% for RA, and -1.63% for LB respectively.

- average BD-rate losses of Y over VTM with AMT enabled (--EMT=3 --EMTFast=3) are 0.17% for AI, 0.21% for RA and -0.06% for LB, respectively.

- average BD-rate losses of Y over BMS are 0.17% for AI, 0.16% for RA and -0.19% for LB, respectively.

It indicates that the modified AMT with only type2/type4 DCT/DST achieves similar coding efficiency compared to the existing AMT.

Introduction

Adaptive Multiple core Transforms (AMT) in BMS [1] has a noticeable coding gain but increases implementation burden due to requiring five transforms: DCT-2, DCT-5, DCT-8, DST-1 and DST-7. For example, memory usage to store AMT transform matrices in BMS is up to 508% of VTM’s as shown in Table 1-1.

Table 1-1 Memory Usage for VTM AMT in BMS

| Transform size | # of elements | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (size^2) | # of kernels | Bits per elements | Bits amounts | # of kernels | Bits per elements | Bits amounts |
| 2 | 4 | 1 | 8 | 32 | 1 | 10 | 40 |
| 4 | 16 | 1 | 8 | 128 | 5 | 10 | 800 |
| 8 | 64 | 1 | 8 | 512 | 5 | 10 | 3200 |
| 16 | 256 | 1 | 8 | 2048 | 5 | 10 | 12800 |
| 32 | 1024 | 1 | 8 | 8192 | 5 | 10 | 51200 |
| 64 | 16384 | 1 | 10 | 163840 | 5 | 10 | 819200 |
| Total [bits] |  | 174752 |  |  | 887240 |  |  |
|  |  | (~21.3 Kbytes ) |  |  | (~ 108 Kbytes ) |  |  |
| Over VTM [%] |  | 100 |  |  | 508 |  |  |

Proposal

In this proposal, the following transforms used in AMT are replaced with alternate transforms as shown in Table 2-1. As a result, the number of core transforms reduces four from five.

Table 2-1 Transform replacement

| Replaced transform | Alternate transform |
| --- | --- |
| DCT-5 | DCT-2 |
| DCT-8 | DCT-4 |
| DST-1 | DST-2 |
| DST-7 | DST-4 |

Figure 2-1 shows the lower part of basis vectors of Order-8 DCT-5/DCT-2, DCT-8/DCT-4, DST-7/DST-4, and DST-1/DST-2, respectively. As can be seen, each alternate transform (DCT-2, DCT-8, DST-1, DST-7) seems similar to the corresponding replaced transform (DCT-5, DCT-8, DST-2 and DST-4) so that it is expected that they have similar compression properties.

Figure 2-1 Lower part of basis vectors of Order-8 DCT-5/DCT-2, DCT-8/DCT-4, DST-7/DST-4, and DST-1/ DST-2

Property of DCT-2 and Relationships of DCT-4/DST-4, and DCT-2/DST-2

It is known that the matrix CNII of DCT-2 can be decomposed into a half size of DCT-2 and DCT-4 [3] as below.

- (1)

, where

- PN is a permutation matrix which when applied into input signals: 
x'n= x2n, x'N2+n= x2n+1  (n=0,…,N/2 – 1)

- IN is an identity matrix

- JN is a cross-identity matrix (flip matrix)

- DN is a diagonal odd-sign changing matrix given by cross-identity matrix (flip matrix) 
	DN=diag -1k, k=0, 1,…, N-1

Based on the above mentioned property (1), matrices of 2^M-pt DCT-2/DCT-4 can be directly obtained from the matrix of larger 2^N-pt DCT-2 (M<N) as shown in Figure 2-2.

Figure 2-2 Extracting matrices of 2^M-pt DCT-2/DCT-4 from matrix of 2^N-pt DCT-2

It is also known that the matrix of DST-2/DST-4 is obtained from the corresponding matrix of DCT-2/DCT-4 by flipping and sign changing as below [3].

SNII=JNCNII DN	- (2)

SNIV=JNCNIVDN=DNCNIVJN 	- (3)

By combining of the mentioned property (1), (2) and (3), matrices of 2^M-pt DST-2/DST-4 can be directly obtained from the matrix of larger 2^N-pt DCT-2 (M<N).

Thus, it is considered that the AMT with only type2/type4 DCT/DST enables sharing matrix among all transforms, which would be beneficial to further reduce the memory usage of core transforms.

Simulation results

To evaluate coding performance of the AMT with only type2/type4 DCT/DST, it was integrated on top of BMS-1.1 [1], and two simulations were conducted according to common test condition described in JVET-J1010 [2].

- VTM vs Tested

  - anchor: BMS-1.1 s/w, vtm config

  - additional anchor: BMS-1.1 s/w, vtm config, option: --EMT=3 --EMTFast=3

  - tested: JVET-K0394, vtm config, option: --EMT=3 --EMTFast=3

- BMS vs Tested

  - anchor: BMS-1.1 s/w, bms config

  - tested: JVET-K0394 s/w, bms config

VTM vs Tested

The coding performance over VTM and VTM with AMT enabled is summarized in Table 3-1.

Table 3-1 Results over VTM and VTM with AMT enabled (--EMT=3 –EMTFast=3)

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over VTM-1.1 + AMT |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.94% | -0.86% | -0.80% | 203% | 135% | 0.36% | 0.47% | 0.43% | 100% | 90% |
| Class A2 | -2.88% | -1.31% | -1.29% | 215% | 126% | 0.26% | 0.36% | 0.14% | 101% | 93% |
| Class B | -3.15% | -1.43% | -1.35% | 221% | 125% | 0.33% | 0.01% | 0.06% | 101% | 94% |
| Class C | -3.29% | -1.37% | -1.48% | 223% | 112% | -0.17% | -0.48% | -0.33% | 101% | 98% |
| Class E | -4.32% | -3.10% | -3.03% | 203% | 121% | 0.08% | -0.37% | -0.38% | 101% | 99% |
| Overall | -3.29% | -1.58% | -1.56% | 214% | 123% | 0.17% | -0.03% | -0.02% | 101% | 95% |
| Class D | -3.30% | -1.23% | -1.37% | 234% | 110% | -0.30% | -0.57% | -0.63% | 101% | 99% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over VTM-1.1 + AMT |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.61% | 0.25% | 0.30% | 172% | 119% | 0.30% | 0.18% | 0.15% | 101% | 98% |
| Class A2 | -1.62% | 0.59% | 0.70% | 161% | 112% | 0.44% | 0.45% | 0.23% | 101% | 98% |
| Class B | -2.23% | -0.25% | -0.34% | 169% | 113% | 0.26% | 0.02% | 0.24% | 101% | 98% |
| Class C | -2.17% | -0.18% | -0.43% | 170% | 109% | -0.11% | -0.21% | -0.24% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.97% | 0.04% | -0.03% | 168% | 113% | 0.21% | 0.08% | 0.09% | 101% | 99% |
| Class D | -2.01% | 0.17% | -0.32% | 164% | 106% | -0.20% | -0.34% | -0.17% | 101% | 101% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over VTM-1.1 + AMT |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.93% | 0.52% | 0.56% | 175% | 119% | -0.01% | -0.31% | 0.20% | 101% | 98% |
| Class C | -1.58% | 0.49% | 0.75% | 168% | 112% | -0.28% | -0.46% | -0.18% | 101% | 99% |
| Class E | -1.20% | 1.09% | 1.65% | 153% | 112% | 0.14% | -0.93% | -0.26% | 100% | 102% |
| Overall | -1.63% | 0.65% | 0.90% | 167% | 115% | -0.06% | -0.52% | -0.04% | 101% | 100% |
| Class D | -1.53% | 0.60% | -0.24% | 154% | 105% | -0.29% | -0.11% | -1.09% | 101% | 100% |

BMS vs Tested

The coding performance over BMS is summarized in Table 3-2.

Table 3-2 Results over BMS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.34% | 0.85% | 0.75% | 99% | 92% |
| Class A2 | 0.32% | 0.51% | 0.29% | 100% | 95% |
| Class B | 0.29% | 0.33% | 0.42% | 100% | 95% |
| Class C | -0.13% | -0.10% | -0.19% | 101% | 98% |
| Class E | 0.04% | -0.06% | -0.02% | 101% | 100% |
| Overall | 0.17% | 0.29% | 0.25% | 100% | 96% |
| Class D | -0.15% | -0.23% | -0.27% | 101% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.28% | 0.66% | 0.44% | 98% | 98% |
| Class A2 | 0.37% | 0.81% | 0.28% | 100% | 99% |
| Class B | 0.19% | 0.34% | 0.41% | 100% | 99% |
| Class C | -0.12% | 0.00% | -0.15% | 97% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | 0.40% | 0.24% | 99% | 99% |
| Class D | -0.19% | -0.26% | 0.00% | 96% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | 0.00% | -0.11% | 101% | 99% |
| Class C | -0.34% | -0.16% | -0.27% | 102% | 100% |
| Class E | -0.16% | 0.11% | 0.24% | 100% | 101% |
| Overall | -0.19% | -0.03% | -0.08% | 101% | 100% |
| Class D | -0.28% | -0.26% | -0.36% | 101% | 100% |

Conclusion

In this contribution, a modified AMT with only type2/type4 DCT/DST is proposed, which achieves similar coding efficiency compared to the existing AMT, and has a potential to enable sharing transform matrix among DCT-2 and alternate transforms.

It is recommended to further study the modified AMT with only type2/type4 DCT/DST in CE.

References

[1] BMS, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.1

[2] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, JVET 10th Meeting: San Diego, US, 10–20 Apr. 2018.

[3] V. Britanak, P. Yip, K. R. Rao, “DISCRETE COSINE AND SINE TRANSFORMS - General Properties, Fast Algorithms and Integer Approximations,” Academic Press, 2007.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).