| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0395-v1 |
| --- | --- |

| Title: | CE3 : Inter-color reference prediction (CE3-4.3.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Yoshitaka Kidani Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-5066-9075 ki-kawamura@kddi.com yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution presents results of a core experiment 3-4.3.1 included CE3 (JVET-J1023). BD-rates for chroma (U, V) are (-1.62%, -1.66%), (-2.02%, -1.89%), and (-0.96%, -0.92%) under the CTC AI, RA, and LDB conditions respectively in case of comparing with VTM. In a similar way, BD-rates for chroma (U, V) are of (-0.23%, -0.27%), (-0.22%, -0.31%), and (-0.38%, -0.11%) under the CTC AI, RA, and LDB conditions respectively in case of comparing with BMS.

Introduction

Core Experiment 3 (CE3) related to intra prediction and mode coding is established in 10th meeting at San-Diego on April, 2018 [1]. This contribution is reporting results of CE3-4.3.1 proposed by KDDI in the call for response. The remainder of this contribution is divided into two parts;

- Proposed method in CE3-4.3.1 (Inter-color reference prediction), and

- Experimental results under the CTC.

Inter-color reference prediction

JVET-J0016 [2] is stated as an additional chroma-intra-prediction mode. The reference samples of chroma component is replaced by that of luma component with a linear model. The linear model is borrowed from the existing CCLM tool in BMS. After the refinement of the reference samples, the existing chroma intra-prediction is performed. The prediction mode of chroma is borrowed from the mode of luma.

Experimental Results

The proposed tool was implemented on top of the BMS-1.0 software, which can be found at https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE3/CE3-4.3.1-4.3.2. It is implemented not only proposed method in CE3-4.3.1 but also proposed method in CE3-4.3.2 to the software. We run the software with following parameter settings in TypeDef.h for CE3-4.3.1.

# define J0016_CE3_CCP_NEIGHBOUR 1

# define J0016_CE3_CROSSCOLOR_ORDER 0

In order to verify the performance of our proposed method, we have run simulations according to the test conditions of JVET-J1010 [3]. Each of the results are presented in bellow. In the results, “VTM-1.0” means simulation results by using BMS-1.0 software with VTM configuration files.

Results under the common test conditions

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Objective evaluation

From Table 1 to Table 6 show the experimental results of proposed method. It can be seen that BD-rate for chroma (U, V) are (-1.62%	, -1.66%), (-2.02%, -1.89%), and (-0.96%, -0.92%) for AI, RA and LDB can be observed respectively in case of comparing with VTM-1.0. In the same manner, BD-rate for chroma (U, V) are (-0.23%, -0.27%), (-0.22%, -0.31%), and (-0.38%, -0.11%) for these three cases can be observed, respectively, in case of comparing with BMS-1.0.

Table 1 Experimental results for AI condition compared with VTM-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -3.79% | -3.47% | 111% | 100% |
| Class A2 | 0.04% | -1.46% | -1.07% | 110% | 100% |
| Class B | 0.07% | -1.09% | -1.41% | 110% | 100% |
| Class C | 0.08% | -1.35% | -1.51% | 110% | 101% |
| Class E | 0.12% | -0.88% | -1.09% | 110% | 100% |
| Overall | 0.07% | -1.62% | -1.66% | 110% | 100% |
| Class D | 0.12% | -0.81% | -0.83% | 109% | 101% |

Table 2 Experimental results for RA condition compared with VTM-1.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -4.06% | -3.43% | 103% | 100% |
| Class A2 | 0.02% | -1.53% | -0.82% | 102% | 100% |
| Class B | 0.08% | -1.44% | -1.75% | 102% | 100% |
| Class C | 0.09% | -1.57% | -1.70% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -2.02% | -1.89% | 102% | 100% |
| Class D | 0.14% | -1.08% | -1.41% | 101% | 99% |

Table 3 Experimental results for LDB condition compared with VTM-1.0

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.86% | -0.74% | 101% | 100% |
| Class C | 0.11% | -0.71% | -0.76% | 101% | 100% |
| Class E | 0.10% | -1.47% | -1.44% | 101% | 100% |
| Overall | 0.08% | -0.96% | -0.92% | 101% | 100% |
| Class D | 0.07% | -0.72% | -0.95% | 101% | 100% |

Table 4 Experimental results for AI condition compared with BMS-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.14% | -0.24% | 112% | 98% |
| Class A2 | 0.08% | -0.32% | -0.28% | 111% | 99% |
| Class B | 0.07% | -0.19% | -0.30% | 110% | 100% |
| Class C | 0.09% | -0.25% | -0.24% | 109% | 102% |
| Class E | 0.11% | -0.29% | -0.31% | 110% | 100% |
| Overall | 0.08% | -0.23% | -0.27% | 110% | 100% |
| Class D | 0.10% | -0.11% | -0.20% | 109% | 101% |

Table 5 Experimental results for RA condition compared with BMS-1.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.13% | -0.10% | 102% | 99% |
| Class A2 | 0.05% | -0.34% | -0.20% | 102% | 100% |
| Class B | 0.09% | -0.24% | -0.31% | 102% | 99% |
| Class C | 0.15% | -0.17% | -0.55% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | -0.22% | -0.31% | 102% | 100% |
| Class D | 0.17% | -0.29% | -0.35% | 103% | 100% |

Table 6 Experimental results for LDB condition compared with BMS-1.0

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.17% | 0.27% | 101% | 98% |
| Class C | 0.05% | -0.25% | -0.60% | 101% | 98% |
| Class E | -0.21% | -0.93% | -0.09% | 99% | 97% |
| Overall | -0.03% | -0.38% | -0.11% | 100% | 98% |
| Class D | 0.15% | 0.23% | -0.26% | 101% | 98% |

Conclusion

This contribution presented results of a core experiment CE3-4.3.1 included CE3 (JVET-J1023 [1]). In this contribution, it was presented that inter-color reference prediction as a proposed method. In proposed method, the reference samples of chroma component was replaced by that of luma component with a linear model. BD-rates for chroma (U, V) were (-1.62%, -1.66%), (-2.02%, -1.89%), and (-0.96%, -0.92%) under the CTC AI, RA, and LDB conditions, respectively, in case of comparing with VTM. In a similar way, BD-rates for chroma (U, V) were (-0.23%, -0.27%), (-0.22%, -0.31%), and (-0.38%, -0.11%) under the CTC AI, RA, and LDB conditions, respectively, in case of comparing with BMS.

- Acknowledgment

We would like to thank Huawei for doing crosschecking of our proposal.

References

- G. Auwera, J. Heo and A. Filippov., “Description of Core Experiment 3: Intra Prediction and Mode Coding”, JVET-J1023, San-Diego, US, April 2018.

- K. Kawamura, Y. Kidani and S. Naito, “Description of SDR video coding technology proposal by KDDI”, JVET-J0016, San-Diego, US, April 2018.

- J. Boyce, K. Suehring, X. Li and V. Seregin., “JVET common test conditions and software reference configurations”, JVET-J1010, San-Diego, US, April 2018.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).