| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0396-v1 |
| --- | --- |

| Title: | CE3 : Adaptive inter-residual prediction (CE3-4.3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Yoshitaka Kidani Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-5066-9075 ki-kawamura@kddi.com yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution presents results of a core experiment 3-4.3.2 included CE3 (JVET-J1023). BD-rates for chroma (U, V) are (-2.70%, -3.35%), (-2.67%, -3.37%), and (-0.36%, -1.65%) under the CTC AI, RA, and LDB conditions respectively in case of comparing with VTM. In a similar way, BD-rates for chroma (U, V) are (0.02%, -0.07%), (-0.02%, -0.11%), and (0.36%, 0.20%) under the CTC AI, RA, and LDB conditions respectively in case of comparing with BMS.

Introduction

Core Experiment 3 (CE3) related to intra prediction and mode coding is established in 10th meeting at San-Diego on April, 2018 [1]. This contribution is reporting results of CE3-4.3.2 proposed by KDDI in the call for response. The remainder of this contribution is divided into two parts;

- Proposed method in CE3-4.3.2 (Adaptive inter-residual prediction),

- Experimental results under the CTC.

Adaptive inter-residual prediction [2]

CCLM Cb-to-Cr prediction exists in BMS. In this contribution,  an adaptive inter-residual prediction is introduced as CCLM Cr-to-Cb prediction. Reference component is signaled by an additional flag. The remaining parts like prediction and scaling factor derivation are the same as in BMS. One flag for each chroma CU is added to identify the reference chroma component. This is implemented by adding a weighted reconstructed Cb residual to the original Cr intra prediction to form the final Cr prediction:

The scaling factor  is derived in the same way of CCLM Cb-to-Cr prediction.

When the intra prediction modes other than CCLM luma-to-chroma prediction mode is used for chroma component of a CU, CCLM Cb-to-Cr or Cr-to-Cb prediction is used to enhance the Cr or Cb component prediction.

Experimental Results

Coding conditions

The proposed tool was implemented on top of the BMS-1.0 software, which can be found at https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE3/CE3-4.3.1-4.3.2. It is implemented not only proposed method in CE3-4.3.2 but also proposed method in CE3-4.3.1 to the software. We run the software with following parameter settings in TypeDef.h for CE3-4.3.2.

# define J0016_CE3_CCP_NEIGHBOUR 0

# define J0016_CE3_CROSSCOLOR_ORDER 1

In order to verify the performance of our proposed method, we have run simulations according to the test conditions of JVET-J1010 [3]. Each of the results are presented in bellow. In the results, “VTM-1.0” means simulation results by using BMS-1.0 software with VTM configuration files.

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Objective results

VTM aspects

From Table 1 to Table 3 show the experimental results of proposed method. It can be seen that BD-rate for chroma (U, V) are (-2.70%, -3.35%), (-2.67%, -3.37%), and (-0.36%, -1.65%) for AI, RA and LD can be observed respectively in case of comparing With VTM-1.0.

Table 1 Experimental results for AI condition compared with VTM-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.38% | -4.03% | -3.03% | 129% | 101% |
| Class A2 | -0.55% | -3.23% | -1.06% | 129% | 101% |
| Class B | -0.18% | -1.55% | -2.95% | 128% | 102% |
| Class C | -0.63% | -2.92% | -5.04% | 127% | 104% |
| Class E | -0.42% | -2.47% | -4.40% | 129% | 103% |
| Overall | -0.42% | -2.70% | -3.35% | 128% | 102% |
| Class D | -0.53% | -2.49% | -4.48% | 127% | 106% |

Table 2 Experimental results for RA condition compared with VTM-1.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -4.01% | -2.77% | 106% | 100% |
| Class A2 | -0.19% | -2.85% | -0.56% | 104% | 100% |
| Class B | -0.08% | -2.01% | -4.48% | 7% | 100% |
| Class C | -0.23% | -2.35% | -4.53% | 12% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -2.67% | -3.37% | 24% | 100% |
| Class D | -0.16% | -2.09% | -4.01% | 20% | 100% |

Table 3 Experimental results for LD condition compared with VTM-1.0

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.45% | -0.74% | 103% | 100% |
| Class C | 0.01% | -0.32% | -1.02% | 102% | 100% |
| Class E | 0.00% | -0.28% | -4.02% | 101% | 100% |
| Overall | -0.01% | -0.36% | -1.65% | 102% | 100% |
| Class D | 0.02% | -0.22% | -0.32% | 102% | 101% |

BMS aspects

From BMS aspects, we had mistaken to upload the software which included the bug and it affects BMS simulation results on June 19th when was the deadline of software submission. From Table 4 to Table 6 shows these results and we added the results where the bug was fixed in Appendix of this contribution.

Table 4 Experimental results for AI condition compared with BMS-1.0 but the bug exists

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | -0.13% | 116% | 98% |
| Class A2 | 0.04% | -0.17% | -0.29% | 115% | 98% |
| Class B | 0.01% | 0.06% | -0.03% | 114% | 100% |
| Class C | 0.02% | 0.02% | 0.06% | 113% | 101% |
| Class E | 0.02% | 0.11% | -0.04% | 114% | 99% |
| Overall | 0.02% | 0.02% | -0.07% | 115% | 99% |
| Class D | 0.04% | 0.11% | 0.05% | 113% | 100% |

Table 5 Experimental results for RA condition compared with BMS-1.0 but the bug exists

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.03% | 0.01% | 103% | 99% |
| Class A2 | 0.05% | -0.44% | -0.06% | 103% | 100% |
| Class B | 0.01% | -0.02% | 0.08% | 103% | 99% |
| Class C | 0.09% | 0.18% | -0.31% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | -0.04% | -0.07% | 103% | 99% |
| Class D | 0.04% | -0.08% | -0.02% | 104% | 100% |

Table 6 Experimental results for LDB condition compared with BMS-1.0 but the bug exists

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.12% | 0.13% | 101% | 98% |
| Class C | 0.03% | 0.28% | 0.08% | 102% | 97% |
| Class E | -0.04% | 0.69% | 0.35% | 99% | 97% |
| Overall | 0.00% | 0.32% | 0.17% | 101% | 97% |
| Class D | 0.10% | 0.92% | 0.49% | 102% | 98% |

Conclusion

This contribution presents results of a core experiment CE3-4.3.2 included CE3 (JVET-J1023 [1]). In this contribution, it was presented that the adaptive inter-residual prediction introduces CCLM Cr-to-Cb prediction. In proposed method, reference component was signaled by an additional flag. BD-rates for chroma (U, V) are (-2.70%, -3.35%), (-2.67%, -3.37%), and (-0.36%, -1.65%) under the CTC AI, RA, and LDB conditions respectively in case of comparing VTM. In a similar way, BD-rates for chroma (U, V) are (0.02%, -0.07%), (-0.02%, -0.11%), and (0.36%, 0.20%) under the CTC AI, RA, and LDB conditions respectively in case of comparing BMS with BMS + Proposed method.

- Acknowledgment

We would like to thank Huawei for doing crosschecking of our proposal.

References

- G. Auwera, J. Heo and A. Filippov, “Description of Core Experiment 3: Intra Prediction and Mode Coding”, JVET-J1023, San-Diego, US, April 2018.

- K. Kawamura, Y. Kidani and S. Naito, “Description of SDR video coding technology proposal by KDDI”, JVET-J0016, San-Diego, US, April 2018.

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, San-Diego, US, April 2018.

Appendix

From Table 7 to Table 9 show the results of the proposed method compared to BMS-1.0 with bug-fixed as mention sec. 3.3.2.

Table 7 Experimental results for AI condition compared with BMS-1.0 with bug-fixed

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | -0.13% | 119% | 98% |
| Class A2 | 0.04% | -0.17% | -0.29% | 118% | 98% |
| Class B | 0.01% | 0.06% | -0.03% | 117% | 99% |
| Class C | 0.02% | 0.02% | 0.06% | 116% | 99% |
| Class E | 0.02% | 0.11% | -0.04% | 117% | 99% |
| Overall | 0.02% | 0.02% | -0.07% | 117% | 99% |
| Class D | 0.04% | 0.11% | 0.05% | 115% | 99% |

Table 8 Experimental results for RA condition compared with BMS-1.0 with bug-fixed

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.01% | -0.07% | 105% | 99% |
| Class A2 | 0.03% | -0.34% | -0.11% | 104% | 100% |
| Class B | 0.00% | 0.00% | 0.06% | 104% | 99% |
| Class C | 0.10% | 0.18% | -0.34% | 105% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.02% | -0.11% | 104% | 99% |
| Class D | 0.04% | -0.08% | -0.02% | 105% | 99% |

Table 9 Experimental results for LDB condition compared with BMS-1.0 with bug-fixed

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.17% | 0.16% | 102% | 99% |
| Class C | 0.00% | 0.43% | 0.19% | 102% | 99% |
| Class E | -0.07% | 0.58% | 0.30% | 100% | 98% |
| Overall | -0.02% | 0.36% | 0.20% | 102% | 99% |
| Class D | 0.10% | 0.87% | 0.50% | 102% | 100% |

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).