| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0397-v1 |
| --- | --- |

| Title: | CE6 : Shrink Transform (CE6.1.9) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Yoshitaka Kidani Kyohei Unno Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-5066-9075 ki-kawamura@kddi.com yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution presents results of a core experiment 6.1.9 included CE6 (JVET-J1026). BD-rates for luma are -0.05%, -0.09%, and -0.08% under the CTC AI, RA, and LDB conditions, respectively, in case of comparing with VTM. In a similar way, BD-rates for luma are -0.05%, -0.07%, and -0.16% under the CTC AI, RA, and LDB conditions, respectively, in case of comparing with BMS.

Introduction

Core Experiment 6 (CE6) related to transforms and transform signalling is established in 10th meeting at San-Diego on April, 2018 [1]. This contribution is reporting results of CE6.1.9 proposed by KDDI in the call for response. The remainder of this contribution is divided into two parts;

- Proposed method in CE6.1.9 (Shrink Transform), and

- Experimental results under the CTC.

Shrink Transform [2]

When the transform block size is equal or larger than 64, inverse transform is realized by two steps; inverse transform with half size and up-sampling. The up-sampling ratio is twice with DCT based coefficient, which is the same as in SHVC. Otherwise (the transform block size is less than 64), inverse transform is the same as in the test model. The following aspects are required to evaluate the proposal. Maximum transform size is configured to 64x64 and 128x128. QP range is set as normal (22, 27, 32, and 37) and lower (like 12 or 17) to confirm the effect by the up-sampling filter.

Experimental Results

The proposed tool was implemented on top of the BMS-1.1 software, which can be found at https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE6/CE6-1.9/. In order to verify the performance of our proposed method, we have run simulations according to the test conditions of JVET-J1010 [3]. Each of the results are presented in bellow. In the results, “VTM-1.1” means simulation results by using BMS-1.1 software with VTM configuration files.

Results under the common test conditions

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Objective evaluation

From Table 1 to Table 6 show the experimental results of proposed method. It can be seen that BD-rate for luma are-0.05%, -0.09%, and -0.08% for AI, RA and LD can be observed respectively in case of comparing with VTM-1.1. In the same manner, BD-rate for luma are-0.05%, -0.07%, and -0.16% for these three cases can be observed, respectively, in case of comparing with BMS-1.1.

Table 1 Experimental results for AI condition compared with VTM-1.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.51% | -0.53% | 99% | 99% |
| Class A2 | -0.02% | -0.13% | -0.07% | 101% | 100% |
| Class B | -0.01% | -0.07% | -0.04% | 100% | 100% |
| Class C | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class E | -0.11% | -0.35% | -0.22% | 100% | 99% |
| Overall | -0.05% | -0.19% | -0.15% | 100% | 100% |
| Class D | 0.00% | -0.01% | 0.00% | 100% | 99% |

Table 2 Experimental results for RA condition compared with VTM-1.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.32% | -0.40% | 97% | 100% |
| Class A2 | -0.06% | -0.27% | -0.05% | 100% | 100% |
| Class B | -0.01% | -0.17% | -0.11% | 100% | 100% |
| Class C | -0.02% | -0.02% | -0.04% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.18% | -0.14% | 99% | 100% |
| Class D | -0.01% | -0.02% | 0.03% | 100% | 101% |

Table 3 Experimental results for LDB condition compared with VTM-1.1

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.11% | -0.47% | -0.39% | 100% | 99% |
| Class C | 0.00% | 0.06% | 0.10% | 100% | 100% |
| Class E | -0.16% | -2.08% | -1.82% | 100% | 100% |
| Overall | -0.08% | -0.69% | -0.58% | 100% | 100% |
| Class D | -0.03% | -0.27% | -0.22% | 100% | 101% |

Table 4 Experimental results for AI condition compared with BMS-1.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.49% | -0.35% | 100% | 100% |
| Class A2 | -0.02% | -0.12% | -0.08% | 101% | 100% |
| Class B | -0.02% | -0.07% | -0.03% | 101% | 101% |
| Class C | 0.00% | 0.03% | 0.02% | 100% | 100% |
| Class E | -0.12% | -0.38% | -0.40% | 100% | 100% |
| Overall | -0.05% | -0.18% | -0.14% | 100% | 101% |
| Class D | 0.00% | 0.00% | -0.01% | 100% | 101% |

Table 5 Experimental results for RA condition compared with BMS-1.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.49% | -0.54% | 98% | 100% |
| Class A2 | -0.06% | -0.04% | -0.05% | 101% | 100% |
| Class B | -0.02% | -0.02% | -0.18% | 101% | 100% |
| Class C | 0.01% | -0.01% | 0.03% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.11% | -0.17% | 100% | 100% |
| Class D | 0.00% | -0.09% | -0.02% | 100% | 101% |

Table 6 Experimental results for LDB condition compared with BMS-1.1

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | -0.67% | -0.70% | 100% | 101% |
| Class C | -0.07% | 0.10% | -0.13% | 100% | 100% |
| Class E | -0.27% | -0.93% | -1.44% | 100% | 100% |
| Overall | -0.16% | -0.48% | -0.70% | 100% | 100% |
| Class D | 0.03% | 0.12% | 0.39% | 100% | 101% |

Objective evaluation at low QPs conditions

To verify the effect of the proposed tool for low QP conditions, we conduct additional simulation where QPs are 12, 17, 22 and 27. From Table 1 to Table 6 show the experimental results of proposed method. It can be seen that BD-rate for luma are-0.01%, -0.01%, and -0.01% for AI, RA and LD can be observed respectively in case of comparing VTM-1.1 with VTM-1.1. In the same manner, BD-rate for luma are-0.01%, -0.01%, and -0.04% for these three cases can be observed respectively in case of comparing BMS-1.1 with BMS-1.1.

Table 1 Experimental results for AI condition compared with VTM-1.1 at low QPs conditions

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.06% | 100% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | -0.04% | -0.11% | -0.07% | 100% | 100% |
| Overall | -0.01% | -0.02% | -0.02% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 103% |

Table 2 Experimental results for RA condition compared with VTM-1.1 at low QPs conditions

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.16% | 0.01% | 98% | 100% |
| Class A2 | 0.01% | -0.05% | -0.02% | 100% | 99% |
| Class B | 0.00% | -0.07% | -0.01% | 100% | 100% |
| Class C | 0.00% | 0.02% | 0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | 0.00% | 99% | 100% |
| Class D | 0.00% | -0.02% | -0.02% | 99% | 101% |

Table 3 Experimental results for LD condition compared with VTM-1.1 at low QPs conditions

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.03% | -0.09% | 100% | 100% |
| Class C | -0.01% | 0.02% | 0.03% | 100% | 100% |
| Class E | -0.01% | -0.67% | -0.38% | 100% | 101% |
| Overall | -0.01% | -0.18% | -0.12% | 100% | 100% |
| Class D | -0.02% | -0.06% | 0.08% | 99% | 101% |

Table 4 Experimental results for AI condition compared with BMS-1.1 at low QPs conditions

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.13% | -0.11% | 100% | 101% |
| Class A2 | 0.00% | -0.01% | 0.01% | 101% | 101% |
| Class B | 0.00% | -0.02% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | -0.03% | -0.13% | -0.04% | 101% | 100% |
| Overall | -0.01% | -0.05% | -0.02% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 5 Experimental results for RA condition compared with BMS-1.1 at low QPs conditions

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.01% | -0.01% | 98% | 100% |
| Class A2 | 0.00% | 0.01% | -0.01% | 101% | 100% |
| Class B | 0.01% | -0.05% | 0.00% | 101% | 100% |
| Class C | 0.00% | -0.01% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.00% | 100% | 100% |
| Class D | -0.01% | -0.01% | -0.01% | 100% | 101% |

Table 6 Experimental results for LDB condition compared with BMS-1.1 at low QPs conditions

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.09% | -0.14% | 101% | 101% |
| Class C | -0.03% | 0.03% | -0.04% | 100% | 100% |
| Class E | -0.09% | -0.65% | -0.49% | 101% | 101% |
| Overall | -0.04% | -0.19% | -0.19% | 100% | 100% |
| Class D | 0.01% | -0.05% | -0.12% | 100% | 101% |

Conclusion

This contribution presented results of a core experiment CE6.1.9 included CE6 (JVET-J1026 [1]). In this contribution, it was presented that when the transform block size is equal or larger than 64, inverse transform is realized by two steps; inverse transform with half size and up-sampling as a proposed method. BD-rates for luma were -0.05%, -0.09%, and -0.08% under the CTC AI, RA, and LDB conditions respectively in case of comparing VTM with VTM + Proposed method. In a similar way, BD-rates for luma were -0.05%, -0.07%, and -0.16% under the CTC AI, RA, and LDB conditions respectively in case of comparing BMS with BMS + Proposed method.

- Acknowledgment

We would like to thank Mediatek and Panasonic for doing crosschecking of our proposal.

References

- A. Said and X. Zhao, “Description of Core Experiment 6: Transforms and transform signalling”, JVET-J1026, San-Diego, US, April 2018.

- K. Kawamura, Y. Kidani and S. Naito, “Description of SDR video coding technology proposal by KDDI”, JVET-J0016, San-Diego, US, April 2018.

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, San-Diego, US, April 2018.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).