| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0398-v1 |
| --- | --- |

| Title: | CE7 : Block size dependent coefficient scanning (CE7.4.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 49 278 7417 yo-kidani@kddi.com ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution presents results of a core experiment CE7.4.1 included CE7 (JVET-J1027). For VTM side, the proposed coefficient scanning obtains BD-rate gains for luma by averagely -0.32%/-0.14%/-0.04% in AI/RA/LDB conditions, respectively without increasing running times. On the other hand, for BMS side, BD-rates for luma are almost same as the anchor by -0.02%/0.01%/-0.10% in AI/RA/LDB conditions. Based on the further investigation, it is clarified that the cause of decrease in BD-rate gain for BMS is conflict with JEM coefficient coding thorough competitive tool analysis.

Introduction

Mode dependent coefficient scanning (MDCS) is introduced in HEVC and JEM for improving coding efficiency. That is applicable for relatively small block by 4x4, 8x8, 4x8, 8x8 and the scanning orders are selected among horizontal, vertical and diagonal scan depends on intra prediction mode. On the other hand, diagonal scan is used fixedly for relatively large blocks whose block size is larger than 8.

|  | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- |
| 4 | M | M | D | D | D |
| 8 | M | M | D | D | D |
| 16 | D | D | D | D | D |
| 32 | D | D | D | D | D |
| 64 | D | D | D | D | D |

Figure 1. Decision table of JEM-MDCS. The vertical line and horizontal line indicate block height and width respectively and M and D indicate MDCS and diagonal scanning.

Block size dependent coefficient scanning

The proposed coefficient scanning is modified the decision table of selecting MDCS based on the ratio of block size [2]. When the ratio of horizontal and vertical size is equal or larger than four, mode-dependent coefficient scanning is applied. The proposed decision maps are shown in the following figure. It is noted that transform block maximum sizes are 64x64 luma and 32x32.

|  | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- |
| 4 | M | M | D | D | D |
| 8 | M | M | D | M | M |
| 16 | D | D | D | D | M |
| 32 | D | M | D | D | D |
| 64 | D | M | M | D | D |

Figure 2. Decision table of proposed method. The vertical line and horizontal line indicate block height and width respectively and M and D indicate MDCS and diagonal scanning.

Experimental Results

Coding conditions

The proposal method was implemented on the top of the VTM 1.0 and BMS 1.0, which can be found at https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-4.1-0, and tested under JVET-J1010 CTC [3].

It is noted that HEVC-MDCS is disabled as an anchor condition of VTM and BMS.

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Objective results

VTM aspects

From Table 1 to Table 3 indicate the objective results of proposed method compared to VTM 1.0. The results show there are averagely -0.33%/-0.13%/-0.09% BD-rates gains in AI/RA/LDB condition without increasing running time due to the proposed method

Table 1 Objective results for AI condition compared with proposed method based on VTM-1.0

| All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Over VTM-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.05% | -0.08% | 102% | 100% |
| Class A2 | -0.18% | 0.00% | -0.07% | 102% | 100% |
| Class B | -0.26% | -0.11% | 0.02% | 101% | 101% |
| Class C | -0.49% | -0.32% | -0.37% | 102% | 102% |
| Class E | -0.63% | 0.09% | -0.06% | 102% | 102% |
| Overall | -0.33% | -0.10% | -0.11% | 102% | 101% |
| Class D | -0.56% | -0.36% | -0.19% | 102% | 104% |

Table 2 Objective results for RA condition compared with proposed method based on VTM-1.0

| Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Over VTM-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.06% | 0.02% | 102% | 100% |
| Class A2 | -0.12% | 0.13% | 0.06% | 102% | 100% |
| Class B | -0.13% | 0.05% | -0.02% | 101% | 100% |
| Class C | -0.21% | -0.10% | -0.10% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | 0.03% | -0.02% | 101% | 100% |
| Class D | -0.23% | 0.06% | -0.01% | 101% | 100% |

Table 3 Objective results for LDB condition compared with proposed method based on VTM-1.0

| Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Over VTM-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.09% | 0.18% | 101% | 100% |
| Class C | -0.06% | 0.03% | 0.09% | 101% | 100% |
| Class E | -0.24% | 0.36% | -0.17% | 101% | 100% |
| Overall | -0.09% | 0.14% | 0.06% | 101% | 100% |
| Class D | -0.06% | 0.35% | 0.07% | 100% | 100% |

BMS aspects

From Table 4 to Table 6 indicate the objective results of proposed method compared to BMS 1.0. The results show there are -0.02 and -0.10% BD-rates gains in AI and LDB but 0.01% BD-rate loss in RA condition due to the proposed method.

Table 4 Objective results for AI condition compared with proposed method based on BMS-1.0

| All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.11% | 0.03% | 101% | 101% |
| Class A2 | 0.02% | 0.11% | 0.05% | 101% | 101% |
| Class B | 0.03% | 0.17% | 0.10% | 101% | 101% |
| Class C | -0.07% | 0.04% | 0.06% | 101% | 101% |
| Class E | -0.15% | 0.21% | 0.02% | 101% | 101% |
| Overall | -0.02% | 0.13% | 0.06% | 101% | 101% |
| Class D | -0.05% | 0.04% | 0.08% | 101% | 101% |

Table 5 Objective results for RA condition compared with proposed method based on BMS-1.0

| Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.00% | 0.07% | 99% | 100% |
| Class A2 | 0.02% | 0.11% | 0.17% | 100% | 101% |
| Class B | -0.04% | 0.15% | 0.07% | 100% | 100% |
| Class C | 0.05% | 0.07% | -0.22% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.09% | 0.01% | 100% | 100% |
| Class D | -0.01% | -0.15% | -0.01% | 100% | 100% |

Table 6 Objective results for LDB condition compared with proposed method based on BMS-1.0

| Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.06% | 0.37% | 99% | 101% |
| Class C | -0.01% | -0.20% | -0.06% | 100% | 100% |
| Class E | -0.36% | 1.03% | 0.57% | 99% | 100% |
| Overall | -0.10% | 0.17% | 0.28% | 99% | 100% |
| Class D | 0.04% | 0.16% | -0.24% | 100% | 100% |

Competitive tool analysis

To verify the cause of the decline in BD-rate gain for BMS, we conducted competitive tool analysis. We compared the results disabling some BMS default tools associated with coefficient coding. From Table 8 to Table 10 show the objective results of proposed method based on BMS 1.0 without NSST only, AltResicomp only, NSST and AltResicom respecitively. It is noted that these are non-full-frame simulation results and they are 32 intra slices.

Simulation results indicates that there are -0.10%/-0.12%/-0.23% BD-rate gain where NSST and AltResiComp tools are disabled, so proposed MDCS is conflicted with them.

Table 8 Objective results of proposed method based on BMS-1.0 without NSST

| All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.16% | 0.00% | 101.9% | 92.3% |
| Class A2 | -0.01% | 0.20% | 0.02% | 100.3% | 96.2% |
| Class B | -0.04% | -0.12% | 0.40% | 98.1% | 107.0% |
| Class C | -0.17% | -0.02% | 0.10% | 102.7% | 83.0% |
| Class E | -0.27% | 0.42% | 0.04% | 100.3% | 94.5% |
| Overall | -0.10% | 0.09% | 0.14% | 100.5% | 95.0% |
| Class D | -0.12% | -0.11% | -0.26% | 90.8% | 114.4% |

Table 9 Objective results of proposed method based on BMS-1.0 without AltResiComp

| All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | -0.02% | 97.8% | 104.4% |
| Class A2 | -0.02% | 0.15% | 0.08% | 98.8% | 96.8% |
| Class B | -0.06% | 0.25% | 0.01% | 97.1% | 105.3% |
| Class C | -0.26% | 0.20% | -0.03% | 96.7% | 104.3% |
| Class E | -0.28% | -0.35% | -0.80% | 99.4% | 103.1% |
| Overall | -0.12% | 0.07% | -0.13% | 97.8% | 103.1% |
| Class D | -0.20% | 0.14% | 0.01% | 101.9% | 112.5% |

Table 10 Objective results of proposed method based on BMS-1.0 without NSST and AltResiComp

| All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Over BMS-1.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.01% | 0.11% | 105.7% | 101.8% |
| Class A2 | -0.08% | 0.03% | 0.01% | 102.1% | 104.0% |
| Class B | -0.11% | 0.16% | 0.07% | 104.9% | 97.9% |
| Class C | -0.43% | -0.11% | -0.07% | 106.0% | 106.3% |
| Class E | -0.49% | -0.32% | 0.12% | 101.9% | 100.7% |
| Overall | -0.23% | -0.03% | 0.04% | 104.3% | 101.9% |
| Class D | -0.56% | -0.06% | -0.23% | 98.7% | 111.8% |

Conclusion

In this contribution, the simulation results by the proposed coefficient scanning were presented. The objective results show proposed method increase BD-rate gains by averagely -0.32%/-0.14%/-0.04% in AI/RA/LDB conditions without increasing running times for VTM though no BD-rates gain for BMS. From competitive tools analysis, it is clarified that the cause of decline in BD-rate gain for BMS is NSST and AltResicomp included BMS default tool sets, so the adoption of proposed MDCS for VTM or further study of competitive tool sets of BMS are recommended.

- Acknowledgment

We would like to thank Huawei and LGE for doing crosschecking of our proposal.

References

- H. Schwarz and M. Coban, “Description of Core Experiment 7 (CE7) on Quantization and coefficient coding”, JVET-J1027, San Diego, US, 10-20, Apr. 2018.

- K. Kawamura, Y. Kidani and S. Naito, “Description of SDR video coding technology proposal by KDDI”, JVET-J0016, San Diego, US, 10-20, Apr. 2018.

- J. Boyce, K. Suehring, X. Li and V. Seregin., “JVET common test conditions and software reference configurations”, JVET-J1010, San Diego, US, 10–20 Apr. 2018.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).