| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0399-v1 |
| --- | --- |

| Title: | CE6-related : Simplification of Shrink Transform (CE6.1.9) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Yoshitaka Kidani Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-5066-9075 ki-kawamura@kddi.com yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes simplification method of shrink transform included in core experiment CE6.1.9 as JVET-J1026. Regarding an up-sampling filter after inverse transform, while JVET-J0016 used 8-tap filter for luma and 4-tap filter for chroma, this contribution replaced it with 1-tap filter (i.e., pixel copy) for luma and chroma. This replacement reduce the burden of both hardware and software implementing. Simulation results show that almost same BD-rates and running times for both VTM and BMS between simplified shirnk transform and (complex) shrink transform.

Problem statement

When the transform block size is equal or larger than 64, inverse transform is realized by two steps; inverse transform with half size and up-sampling. The up-sampling ratio is twice with DCT based coefficient, which is the same as in SHVC. Otherwise (the transform block size is less than 64), inverse transform is the same as in the test model. In the remaining part, this method is called complex shrink transform. It is noted that maximum transform blocks size are 64x64 for luma 32x32 for chroma in VTM and BMS.

Proposed method

The complex shrink transform utilizes 8-tap for luma and 4-tap for chroma as up-sampling filter as follows;

{-1, 4, -11, 40, 40, -11, 4, -1} for luma, and

{-4, 36, 36, -4} for chroma.

Simplified shrink transform replaces the above filter with 1-tap filter (i.e., pixel copy) as follows;

{0, 0, 0, 64, 0, 0, 0, 0} for luma, and

{0, 64, 0, 0} for chroma.

It is noted that phase mismatch can be ignored in the both objective and subjective viewpoints.

Experimental results

Coding conditions

The proposed method was implemented on top of the BMS-1.1 software In order to verify the performance of our proposed method, we have run simulations according to the test conditions of JVET-J1010 [3]. Each of the results are presented in bellow. In the results, “VTM-1.1” means simulation results by using BMS-1.1 software with VTM configuration files.

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Objective evaluation

VTM aspects

Table 1 indicates the objective results of proposed method and CE6.1.9 based on VTM-1.1 compared to VTM-1.1 anchor. The results show that BD-rate gains of proposed method are averagely -0.04%/-0.06%/-0.07% for AI/RA/LDB condition without increasing simulation runtime compared to VTM-1.1 anchor. Moreover, these are the almost same BD-rates as the CE6.1.9 results.

Table 1 Objective results for AI condition compared with proposed method based on VTM-1.1

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Proposed Method over VTM-1.1 |  |  |  |  |  | CE6.1.9 over VTM-1.1 |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 | -0.07% | -0.32% | -0.31% | 100% | 100% |  | -0.17% |  | -0.51% |  | -0.53% |  | 99% |  | 99% |
| Class A2 | -0.02% | -0.12% | -0.05% | 100% | 100% |  | -0.02% |  | -0.13% |  | -0.07% |  | 101% |  | 100% |
| Class B | 0.00% | -0.05% | -0.03% | 101% | 100% |  | -0.01% |  | -0.07% |  | -0.04% |  | 100% |  | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |  | 0.00% |  | -0.01% |  | 0.00% |  | 100% |  | 100% |
| Class E | -0.12% | -0.43% | -0.21% | 100% | 99% |  | -0.11% |  | -0.35% |  | -0.22% |  | 100% |  | 99% |
| Overall | -0.04% | -0.16% | -0.10% | 100% | 100% |  | -0.05% |  | -0.19% |  | -0.15% |  | 100% |  | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 101% |  | 0.00% |  | -0.01% |  | 0.00% |  | 100% |  | 99% |
|  | Random Access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Proposed Method over VTM-1.1 |  |  |  |  |  | CE6.1.9 over VTM-1.1 |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 | -0.25% | -0.26% | -0.49% | 98% | 100% |  | -0.34% |  | -0.32% |  | -0.40% |  | 97% |  | 100% |
| Class A2 | -0.06% | -0.11% | -0.10% | 100% | 100% |  | -0.06% |  | -0.27% |  | -0.05% |  | 100% |  | 100% |
| Class B | 0.01% | -0.15% | -0.03% | 100% | 100% |  | -0.01% |  | -0.17% |  | -0.11% |  | 100% |  | 100% |
| Class C | -0.01% | -0.03% | -0.04% | 100% | 100% |  | -0.02% |  | -0.02% |  | -0.04% |  | 100% |  | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.06% | -0.13% | -0.14% | 99% | 100% |  | -0.09% |  | -0.18% |  | -0.14% |  | 99% |  | 100% |
| Class D | -0.01% | -0.02% | 0.02% | 99% | 100% |  | -0.01% |  | -0.02% |  | 0.03% |  | 100% |  | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Proposed Method over VTM-1.1 |  |  |  |  |  | CE6.1.9 over VTM-1.1 |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.12% | -0.30% | -0.39% | 100% | 100% | -0.11% |  | -0.47% |  | -0.39% |  | 100% |  | 99% |  |
| Class C | 0.00% | -0.10% | -0.09% | 100% | 100% | 0.00% |  | 0.06% |  | 0.10% |  | 100% |  | 100% |  |
| Class E | -0.10% | -1.46% | -0.11% | 100% | 101% | -0.16% |  | -2.08% |  | -1.82% |  | 100% |  | 100% |  |
| Overall | -0.07% | -0.52% | -0.22% | 100% | 100% | -0.08% |  | -0.69% |  | -0.58% |  | 100% |  | 100% |  |
| Class D | -0.03% | 0.09% | -0.02% | 100% | 101% | -0.03% |  | -0.27% |  | -0.22% |  | 100% |  | 101% |  |

BMS aspects

Table 2 indicates the objective results of proposed method and CE6.1.9 based on BMS-1.1 compared to BMS-1.1 anchor. The results show that BD-rate gains of proposed method are averagely -0.04%/-0.05%/-0.16% for AI/RA/LDB condition without increasing simulation runtime compared to BMS-1.1 anchor. Moreover, these are the almost same BD-rates as the CE6.1.9 results.

Table 2 Objective results for AI condition compared with proposed method based on BMS-1.1

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Proposed Method over BMS-1.1 |  |  |  |  |  | CE6.1.9 over BMS-1.1 |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 | -0.05% | -0.37% | -0.33% | 100% | 101% |  | -0.12% |  | -0.49% |  | -0.35% |  | 100% |  | 100% |
| Class A2 | 0.00% | -0.10% | -0.11% | 100% | 101% |  | -0.02% |  | -0.12% |  | -0.08% |  | 101% |  | 100% |
| Class B | -0.01% | -0.02% | -0.01% | 101% | 102% |  | -0.02% |  | -0.07% |  | -0.03% |  | 101% |  | 101% |
| Class C | 0.00% | 0.01% | 0.00% | 100% | 102% |  | 0.00% |  | 0.03% |  | 0.02% |  | 100% |  | 100% |
| Class E | -0.14% | -0.30% | -0.44% | 100% | 101% |  | -0.12% |  | -0.38% |  | -0.40% |  | 100% |  | 100% |
| Overall | -0.04% | -0.13% | -0.15% | 100% | 101% |  | -0.05% |  | -0.18% |  | -0.14% |  | 100% |  | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |  | 0.00% |  | 0.00% |  | -0.01% |  | 100% |  | 101% |
|  | Random Access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Proposed Method over BMS-1.1 |  |  |  |  |  | CE6.1.9 over BMS-1.1 |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 | -0.21% | -0.34% | -0.55% | 99% | 100% |  | -0.29% |  | -0.49% |  | -0.54% |  | 98% |  | 100% |
| Class A2 | -0.03% | 0.09% | -0.04% | 101% | 100% |  | -0.06% |  | -0.04% |  | -0.05% |  | 101% |  | 100% |
| Class B | -0.02% | -0.13% | -0.10% | 101% | 101% |  | -0.02% |  | -0.02% |  | -0.18% |  | 101% |  | 100% |
| Class C | 0.02% | -0.04% | -0.03% | 101% | 101% |  | 0.01% |  | -0.01% |  | 0.03% |  | 100% |  | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.05% | -0.11% | -0.16% | 100% | 101% |  | -0.07% |  | -0.11% |  | -0.17% |  | 100% |  | 100% |
| Class D | -0.01% | -0.09% | -0.03% | 100% | 101% |  | 0.00% |  | -0.09% |  | -0.02% |  | 100% |  | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Proposed Method over BMS-1.1 |  |  |  |  |  | CE6.1.9 over BMS-1.1 |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.15% | -0.79% | -0.76% | 100% | 100% | -0.16% |  | -0.67% |  | -0.70% |  | 100% |  | 101% |  |
| Class C | -0.05% | -0.06% | -0.13% | 100% | 99% | -0.07% |  | 0.10% |  | -0.13% |  | 100% |  | 100% |  |
| Class E | -0.32% | -1.10% | -2.07% | 100% | 98% | -0.27% |  | -0.93% |  | -1.44% |  | 100% |  | 100% |  |
| Overall | -0.16% | -0.63% | -0.88% | 100% | 99% | -0.16% |  | -0.48% |  | -0.70% |  | 100% |  | 100% |  |
| Class D | 0.09% | -0.21% | 0.13% | 100% | 99% | 0.03% |  | 0.12% |  | 0.39% |  | 100% |  | 101% |  |

Conclusion

In this contribution, it was presented simplification method of shrink transform included in core experiment CE6.1.9 as JVET-J1026 [1]. In the complex shrink transform [2], up-sampling filters were 8-tap filter for luma and 4-tap filter for chroma. The proposed simplified shrink transform replaced it with 1-tap filter (i.e., pixel copy) for luma and chroma. It could reduce the burden of both hardware and software implementation. BD-rates for luma were -0.04%, -0.06%, and -0.07% under the CTC AI, RA, and LDB conditions respectively in case of comparing with VTM. In a similar way, BD-rates for luma were -0.04%, -0.05%, and -0.16% under the CTC AI, RA, and LDB conditions, respectively, in case of comparing with BMS.

- Acknowledgment

We would like to thank Panasonic for crosschecking of our proposal.

References

- A. Said and X. Zhao, “Description of Core Experiment 6: Transforms and transform signalling”, JVET-J1026, San-Diego, US, April 2018.

- K. Kawamura, Y. Kidani and S. Naito, “Description of SDR video coding technology proposal by KDDI”, JVET-J0016, San-Diego, US, April 2018.

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, San-Diego, US, April 2018.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).