| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0400-v2 |
| --- | --- |

| Title: | [CE3-related: DC mode with only shift operators based on sub-sampling] |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dongcheol Kim, Geonjung Ko, Ju-Hyung (John) Son, Jin Sam Kwak (WILUS) Jiyeong Seok, Yong-Jae Lee (Humax) | Tel: Email: | +82-31-712-0523 dongcheol.kim@wilusgroup.com jyseok@humaxdigital.com |
| Source: | WILUS Inc., Humax |  |  |

_____________________________

- Abstract

This contribution proposes a DC value calculation method to the rectangular shape of intra prediction block. The proposed method only uses summation and a shift operator without division operator in order to reduce the implementation complexity. Compared with VTM-1.0, simulation results show that the proposed method achieves 0.01% and 0.00% BD-rate for AI and RA configurations respectively while encoding/decoding time remains almost the same as the anchor. Compared with BMS-1.0, simulation results show that the proposed method achieves -0.01% and 0.02% BD-rate for AI and RA configurations respectively while encoding/decoding time remains almost the same as the anchor.

Introduction

In VTM-1.0 [1], a DC value of reference samples of a intra prediction block for DC mode is calculated as follows:

DCval = (Sum(Ref. samples of Width and Height) + ((Width+Height)>>1)) / (Width+Height)

If an intra prediction block is a square block where the sum of block width and height is equal to a power of two, the DCval is calculated by summation and a shift operator. However, for a rectangular block, the shift operation may not be applied if the sum of block width and height is not equal to a power of two.

In [2], a DCval calculation method is proposed and is experimented as the Test 1.2.1 of CE3 [3]. Two separate DC values using the reference samples above the block (DCup) and the reference samples left of the block (DCleft) are calculated separately and combined together by (DCup+DCleft) >> 1. It can use only a shift operator for DC value calculations even when the sum of block width and height is not equal to a power of two.

In this contribution, for the DCval calculation of a non-square block, it is proposed to uniformly exclude reference samples at the longer side of a block where the number of excluded samples is the number of reference samples at the shorter side of a block. Then the number of remaining reference samples is equal to a power of two, therefore it can be summed and divided by using a shift operator.

Technology description

Depending on the shape of an intra prediction block, two kinds of methods are applied to calculate a DC value.

If(a square block){

(Sum(Ref. samples of Width and Height)+((Width+Height) >> 1)) >> log2(Width+Height)

}else {

Proposed method below

}

Fig. 1. Reference sample positions of DCval calculation for non-square blocks 
((a) 8x4 block, (b) 16x4 and (c) 32x4 block examples)

Proposed method is to calculate a DC value without division to the rectangular shape of intra prediction block as follows:

Step 1. Reference samples at the longer side of a block are uniformly sub-sampled. The number of excluded samples at the longer side is the number of samples at the shorter side.

Step 2. The remaining reference samples at both sides are all added.

; Sum(remaining samples)

Step 3. (Sum(remaining samples) + (the length of the longer side >>1)) >> log2(the length of the longer side)

Experimental results

The proposed method has been tested on BMS-1.0 software with VTM and BMS configuration files specified in CTC [4]. Table 1~4 show the summarized results of the proposed method. More detail results can be found in the accompanying Excel tables.

Table 1 Comparison of the proposed method over VTM-1.0 (All Intra)

|  | All Intra (Over VTM-1.0) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.02% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.04% | 99% | 99% |
| Class B | 0.00% | -0.03% | 0.08% | 99% | 101% |
| Class C | 0.02% | -0.04% | -0.14% | 99% | 100% |
| Class E | -0.01% | 0.06% | -0.05% | 99% | 99% |
| Overall | 0.01% | -0.01% | -0.01% | 99% | 100% |
| Class D | 0.01% | 0.00% | 0.09% | 100% | 100% |

Table 2 Comparison of the proposed method over VTM-1.0 (Random Access)

|  | Random Access (Over VTM-1.0) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | 0.00% | 100% | 100% |
| Class A2 | -0.01% | 0.01% | 0.01% | 100% | 100% |
| Class B | 0.00% | 0.01% | -0.05% | 100% | 99% |
| Class C | -0.01% | -0.08% | -0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.03% | 100% | 100% |
| Class D | 0.04% | 0.34% | -0.22% | 100% | 100% |

Table 3 Comparison of the proposed method over BMS-1.0 (All Intra)

|  | All Intra (Over BMS-1.0) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.01% | 0.00% | 0.04% | 100% | 100% |
| Class B | -0.01% | 0.09% | 0.06% | 99% | 100% |
| Class C | -0.01% | 0.02% | 0.05% | 99% | 100% |
| Class E | -0.04% | 0.08% | 0.00% | 99% | 100% |
| Overall | -0.01% | 0.04% | 0.04% | 100% | 100% |
| Class D | 0.02% | 0.07% | -0.01% | 100% | 100% |

Table 4 Comparison of the proposed method over BMS-1.0 (Random Access)

|  | Random Access (Over BMS-1.0) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.08% | 0.00% | 100% | 100% |
| Class A2 | 0.02% | 0.07% | 0.03% | 101% | 101% |
| Class B | 0.00% | 0.12% | -0.07% | 100% | 100% |
| Class C | 0.04% | 0.19% | -0.23% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.12% | -0.08% | 100% | 100% |
| Class D | 0.05% | 0.06% | -0.14% | 100% | 100% |

Conclusions

This contribution proposes a method of DC value calculation without division operation for the rectangular shape of intra prediction block for DC mode. The scheme uses only summation and a shift operator, which reduces implementation complexity. The method proposes to uniformly exclude reference samples at the longer side of a block where the number of excluded samples is the number of samples at the shorter side. The overall luma BD-rates over VTM-1.0 are 0.01% (AI) and 0.00% (RA), and the luma BD-rates over BMS-1.0 are -0.01% (AI) and 0.02% (RA), all without added encoding complexities. It is recommended to adopt the proposed method in the next version of reference software.

References

- J. Chen, E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1),” JVET-J1002, April 2018.

- T. Toma, et al., “Description of SDR video coding technology proposal by Panasonic,” JVET-J0020, April 2018.

- G. Van der Auwera, J. Heo, and A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-J1023, April 2018.

- Jill Boyce, K. Suehring, X. Li, and Vadim Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Humax may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).