| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0404-v1 |
| --- | --- |

| Title: | [AHG8: Selective In-loop filtering for 360 Video Compression] |  |  |
| --- | --- | --- | --- |
| Status: | [Input document to JVET] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Chirag Pujara Sri Nitchith Akula Anubhav Singh Akhil Konda Amith Dsouza Raj Narayana Woongil Choi Kiho Choi | Email: | chirag.p@samsung.com nitchith.a@samsung.com anubhav.23@samsung.com akhil.konda@samsung.com amith.ds@samsung.com raj.gadde@samsung.com woongil.choi@samsung.com kiho14.choi@samsung.com |
| Source: | [Samsung Electronics Co., Ltd., AHG8] |  |  |

_____________________________

# Abstract

This contribution presents an adaptive in-loop filtering for 360◦ video. In case of 360◦ projection, spherical neighbors need to be re-arranged for compact representation and spatial similarity assumption does not hold true near the boundaries of the non-spatial neighbor faces which are placed next to each other. Current in-loop filtering across these boundaries in VTM makes bleed across the seam boundaries causing visible artifacts. In this contribution, avoiding in-loop filtering across these regions is proposed. Results show visible improvement in subjective quality with marginal gain in objective quality improvement due to better reference pictures.

# Introduction

In loop filtering reduces the block artifacts and improves the subjective quality for the compressed natural video contents. 360◦ compression performs re-arrangement of projected faces for compact representation of the projected 2D frame. This would violate the spatial similarity feature of natural video across the non-spherical neighbors. Performing de-blocking across these non-neighboring faces produces visible artifacts due to color bleeding across the seam boundary as shown in Fig.1 for a frame in Harbor sequence.

Fig.1(a) Without deblocking filtering on seam            Fig.1(b) Bleeding due to deblocking filtering

# Subjective Quality

Fig.2(a) shows the viewport from the reconstructed frame without deblocking while fig.2(b) shows the same region with the artifact with deblocking filtering for RSP projection format with VTM software

Fig. 2(a) Without Deblocking             Fig. 2(b) Artifact with Deblocking

# Fig.3 shows the decompressed images with the bleeding artifact with Deblocking and ALF filters ON/OFF for comparing visual quality.

# Fig.3 Effect of bleeding with Deblocking filter and ALF ON/OFF

# Selective Deblocking

# Fig.4 Seam boundary in RSP

Fig.4 shows the boundary (in yellow color) in case of RSP which should be avoided while performing the in-loop filtering like Deblocking and ALF. These boundary may not fall even on CTU boundary so tiles may not be used to stop the in-loop filtering unless suitable padding is used (padding would reduce the coding gain).

# Fig.5 Seam boundary in CMP and its variants

Fig.5 shows the boundary in case of CMP and its variants.

# Fig.6 Pseudo logic for turning off deblocking at seam boundary

Fig.6 shows the block diagram for selectively turning off deblocking filtering for RSP, CMP (and its variants) at the seam boundary. Here y is vertical location of the seam boundary. Here height/2 is at the CTU boundary which would be deblocked.

Currently we have disabled deblocking filtering across the seam and we are working towards disabling ALF.

# Conclusions

Results show that there is a considerable improvement in subjective quality by avoiding in-loop filtering across the seam boundary. We observe a gain in the tune of 0.2% in case of E2E WS-PSNR in the 360lib5.0 for RSP projection format. We expect similar gain in case of VTM/BMS software as well. We shall update the results soon.

# Patent rights declaration(s)

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).