| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0406 |
| --- | --- |

| Title: | Crosscheck of JVET-K0361: CE9-related: Harmonization between CE9.2.6 and CE9.2.9 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-Check |  |  |
| Author(s) or Contact(s): | Li Jingya, Lim Chong Soon 202 Bedok South Ave 1, #02-11 Singapore, 469332 | Email: | jingya.li@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This document cross checks JVET-K0361. JVET-K0361 proposes a harmonization method of CE9.2.6 and CE9.2.9.

BD rate result for CTC RA configuration over VTM1.0 is 2.84% gain with 103% EncTime and 119% DecTime. This BD rate result matches results reported in JVET-K0361.

# Tests description

Source code of JVET-K0361 has been reviewed. Differences between JVET-K0361 and JEM DMVR are highlighted as below:

- bilateral matching method is used with mirroring MVD (no scaling)

- 2 iterations of integer pel search followed by 1 iteration of half-pel cross search are used.

- For integer pel search, adaptive 6 point search pattern is used. (5-point cross search + 1 additional search point determined using the costs of 5 points)

- Search range is [-2, 2] horizontally and vertically.

- 2 tap bilinear filter is used for motion search.

- Mean-removal SAD is used for motion search cost calculation instead of SAD. SIMD is used for Mean-removal SAD.

- Early skip condition is added. First a MVD is calculated between MVP of selected MVP idx and any MVP with smaller MVP idx. If such difference is smaller than a threshold, DMVR process is skipped.

- The refined MV is not used as spatial MV predictor, but it is still used for temporal MV prediction and deblocking filter.

The differences described above matches the technical description of document JVET-K0361.

# Experimental results

Results of JVET-K0361 are summarized in Table 1. It matches with BD results of JVET-K0361.

Table 1 BD-rate results

# Conclusions

This document reports cross-check result of JVET-K0361 source code and simulation results. No mismatch is found.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).