| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0411-v2 |
| --- | --- |

| Title: | AHG11: Palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun, Jicheng An, Jian Lou 969 West Wen Yi Road Yu Hang District Hangzhou 311121 China | Tel: Email: | +1-858-349-2818 yuchen.s@alibaba-inc.com |
| Source: | Alibaba |  |  |

_____________________________

- Abstract

This document proposes a simplified palette mode on top of VTM1.1. For Class F sequences, the results show -2.6%/-1.8%/-0.9% BD-rate luma gain in AI/RA/LDB configuration; for 4:2:0 TGM sequences in CE8, the results show -13.7%/-8.9%/-7.0% BD-rate luma gain in AI/RA/LDB configuration.

Introduction

This document reports a simplified palette mode on top of VTM1.1. The method is similar to the palette mode in HEVC SCC, including palette predictive coding and run-based index map coding. The details of the design are described in Section 2, and the results are summarized in Section 3.

Description of techniques

A flag is transmitted for each CU to signal whether the palette mode is used in the current CU. If the palette mode is utilized, the pixels with pixel values close to the palette colors are represented by the selected palette colors denoted by the palette indices, where the corresponding palette colors are the representative colors in the current CU. The remaining pixels are presented by a special palette index value, denoted as an escape index, and their pixel values are signalled directly

To decode a palette encoded block, the decoder needs to decode palette colors and indices. Palette colors are described by a palette table and encoded by palette table coding tools. Based on the palette table, a pixel of three color components is represented by a palette index. Palette indices of all pixels in a CU form a palette index map and are encoded by palette index map coding tools.

Predictive coding is applied for the palette table. The decoder constructs a palette table predictor to predict the palette table of the current CU. The palettes of the previous palette coded CUs are used as the predictor (palette predictor stuffing). After the palette table is encoded, the pixels in a CU are represented by palette indices corresponding to the colors in the palette. The indices form an index map and are divided into several runs and then encoded in horizontal raster scan order. Figure 1 shows an example of the palette index map coding. In the figure, an index map of 16 indices is encoded by three runs with horizontal raster scan. There are two run modes: copy index mode and copy above mode. For each starting position of a run, a flag is transmitted to indicate which run mode is used.

If the copy above mode is used, a run of pixels will copy the palette indices from the pixels above. In the example in Figure 1, a run, R2, is a copy above run with a run value of 8. If the copy index mode is used, a palette_index_idc syntax element is first signalled, followed by a piece of PaletteIndexRun information. A run of pixels shares the same palette index, where the palette index and the run value are derived from the palette_index_idc and PaletteIndexRun, respectively. For instance, in Fig.1, R0 and R1 are two copy index runs, whose (palette index, run value) are (1, 6) and (2, 2), respectively.

# Figure 1. An example of run-based index map coding.

Simulation results

The proposed method is evaluated according to the CE8 description in [1] and the common test conditions in [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations with the VTM-1.1 software.

Conclusion

This document proposes a simplified palette mode on top of VTM1.1. For Class F sequences, the results show -2.6%/-1.8%/-0.9% BD-rate luma gain in AI/RA/LDB configuration; for 4:2:0 TGM sequences in CE8, the results show -13.7%/-8.9%/-7.0% BD-rate luma gain in AI/RA/LDB configuration. We recommend creating a place holder position for palette mode in the VTM s/w.

Reference

- X. Xu, L. Wang and K. Müller, "Description of Core Experiment 8: Current Picture Referencing, JVET-J1028," San Diego, US, 2018.

- J. Boyce, K. Suehring, X. Li and V. Seregin, "JVET common test conditions and software reference configurations, JVET-J1010," San Diego, US, 2018.

Patent rights declaration(s)

Alibaba Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).