| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0428-v1 |
| --- | --- |

| Title: | Cross-check of JVET-K0072: Non-CE7: Alternative entropy coding for dependent quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Muhammed Coban | Email: | mcoban@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The cross-checker reports that the source code provided by Fraunhofer HHI conforms to the description provided in JVET-K0072. The results provided the proponent can be reproduced by the cross checker. All cases for VTM configuration have been verified. The cross checker has couple of missing simulation points for BMS case. The encoder and decoder runtimes provided by the proponent are consistent with measured run times by the cross-checker.

# Cross-check of JVET-K0072

The document reports the result of a cross check for JVET-K0072 [1]:

- The source code provided by Fraunhofer HHI was studied, the description in JVET-K0072 conforms to the source code provided.

- VTM and BMS configuration test were conducted with transform coefficient coding only (--AltResiComp=2) and transform coefficient coding with dependent quantization (--DepQuant=1).

- Only VTM run time results were measured accurately. BMS cases were run on a cluster utilizing different types of CPUs.

- Runtimes measured are consistent with the numbers reported in JVET-K0072.

- All VTM simulations were completed. BMS case has couple of incomplete simulation results.

- Detailed results are given in the accompanying Excel documents. The average results of the cross check are summarized in Table 1 and Table 2.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.79% | -3.23% | -2.80% | 108% | 99% | 0.00% | 0.00% | 0.00% | 104% | 109% |
| Class A2 | -4.94% | -1.37% | -2.29% | 119% | 100% | 0.00% | 0.00% | 0.00% | 105% | 113% |
| Class B | -4.69% | -1.62% | -0.98% | 117% | 99% | 0.00% | 0.00% | 0.00% | 108% | 113% |
| Class C | -4.90% | -0.51% | -0.79% | 128% | 99% | 0.00% | 0.00% | 0.00% | 105% | 114% |
| Class E | -4.70% | -1.51% | -1.56% | 113% | 99% | 0.00% | 0.00% | 0.00% | 109% | 111% |
| Overall | -4.80% | -1.58% | -1.56% | 117% | 99% | 0.00% | 0.00% | 0.00% | 106% | 112% |
| Class D | -5.00% | 0.30% | -0.22% | 131% | 101% | 0.00% | 0.00% | 0.00% | 103% | 114% |
| Class F (optional) | -4.74% | -0.45% | -2.11% | 123% | 102% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.18% | -1.91% | -1.13% | 105% | 101% | -3.45% | -2.85% | -2.83% | 102% | 103% |
| Class A2 | -2.60% | -0.51% | -2.25% | 110% | 97% | -2.65% | -0.82% | -0.87% | 109% | 103% |
| Class B | -3.58% | -1.73% | -0.87% | 110% | 98% | -2.64% | -2.01% | 0.12% | 139% | 117% |
| Class C | -3.36% | -1.81% | -2.04% | 117% | 100% | -2.32% | -1.84% | 0.90% | 137% | 120% |
| Class E |  |  |  |  |  | -2.19% | -2.04% | 0.57% | 115% | 112% |
| Overall | -3.25% | -1.54% | -1.51% | 111% | 99% | -2.63% | -1.92% | -0.29% | 123% | 112% |
| Class D | -3.35% | -0.28% | -0.89% | 120% | 100% | -2.34% | -0.66% | 1.49% | 120% | 107% |
| Class F (optional) | -3.97% | -1.23% | -2.23% | 112% | 101% | -2.13% | -0.94% | -0.13% | 126% | 112% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.91% | -3.34% | -0.89% | 114% | 100% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -2.66% | -4.63% | -3.01% | 119% | 103% | -1.72% | -3.91% | -2.07% | 151% | 123% |
| Class E | -1.83% | 0.45% | 1.81% | 108% | 103% | -1.10% | 0.18% | 2.09% | 138% | 123% |
| Overall | -2.56% | -2.82% | -0.92% | 114% | 102% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -2.59% | -3.74% | -3.81% | 120% | 99% | -1.53% | -3.01% | -2.74% | 154% | 128% |
| Class F (optional) | -2.72% | -3.29% | -3.62% | 114% | 107% | -1.09% | -2.29% | -1.04% | 144% | 120% |

Table 1: Summary of the cross-check results for the proposed coefficient coding with dependent quantization (JVET-K0072).

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.09% | -1.06% | -1.53% | 105% | 105% | -0.30% | 0.07% | -1.04% | 91% | 106% |
| Class A2 | -1.49% | -0.46% | -2.09% | 105% | 101% | -0.24% | 0.51% | -1.04% | 92% | 108% |
| Class B | -1.43% | -1.72% | -1.19% | 106% | 103% | -0.16% | -1.42% | 0.50% | 118% | 120% |
| Class C | -1.77% | -1.68% | -1.59% | 107% | 103% | -0.19% | -1.29% | 0.46% | 90% | 107% |
| Class E | -2.11% | -2.56% | -2.36% | 105% | 100% | -0.27% | -1.77% | 0.45% | 93% | 109% |
| Overall | -1.57% | -1.53% | -1.68% | 105% | 102% | -0.22% | -0.88% | -0.03% | 98% | 111% |
| Class D | -1.82% | -1.42% | -1.67% | 107% | 104% | -0.12% | -0.84% | 0.21% | 89% | 106% |
| Class F (optional) | -3.19% | -2.55% | -3.01% | 105% | 101% | -1.02% | -1.65% | -1.37% | 108% | 116% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.95% | -0.53% | -0.81% | 103% | 102% | -0.27% | 0.23% | -0.36% | 109% | 108% |
| Class A2 | -0.90% | -0.11% | -1.43% | 103% | 100% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -1.01% | -1.53% | -0.69% | 103% | 101% | -0.10% | -1.27% | 0.77% | 124% | 122% |
| Class C | -1.29% | -1.43% | -1.15% | 105% | 101% | -0.03% | -1.00% | 0.18% | 125% | 125% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.05% | -1.02% | -0.98% | 104% | 101% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -1.25% | -0.61% | -0.96% | 106% | 104% | 0.03% | -0.49% | 0.42% | 125% | 128% |
| Class F (optional) | -2.41% | -1.97% | -2.15% | 104% | 101% | -0.69% | -1.18% | -0.59% | 123% | 123% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.63% | -1.61% | 0.33% | 105% | 101% | -0.04% | -1.53% | 1.25% | 125% | 115% |
| Class C | -0.83% | -1.61% | -0.73% | 105% | 104% | -0.12% | -1.28% | 0.39% | 125% | 122% |
| Class E | -0.58% | -1.40% | -1.09% | 104% | 105% | -0.23% | -2.11% | 0.16% | 126% | 121% |
| Overall | -0.68% | -1.56% | -0.38% | 105% | 103% | -0.11% | -1.59% | 0.69% | 125% | 119% |
| Class D | -0.90% | -1.29% | -1.24% | 106% | 101% | 0.05% | -0.03% | -0.08% | 127% | 129% |
| Class F (optional) | -1.72% | -1.43% | -1.45% | 105% | 100% | -0.28% | -0.84% | -0.18% | 125% | 122% |

Table 2: Summary of the crosscheck results for the proposed coefficient coding without dependent quantization (JVET-K0072).

# References

- H. Schwarz, T. Nguyen, D. Marpe, T. Wiegand, “Non-CE7: Alternative entropy coding for dependent quantization”, JVET-K0072, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubliana, SI, July. 2018.

- J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configuration,” JVET-J1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Diego, US, Apr. 2018.