| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0429-v1 |
| --- | --- |

| Title: | CE6-related: Fast DST-7/DCT-8 with dual implementation support |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Pierrick Philippe Victorien Lorcy | Tel: Email: | pierrick.philippe@orange.com |
| Source: | Orange, b<>com |  |  |

_____________________________

# Abstract

This contribution reports the cross check of JVET-K0291 on a fast implementation of two of the AMT transforms: the DST-7/DCT-8.

The following points are verified: the exactness of the implementation with respect to the description, the amount of operations saved using the fast algorithm, the amount of change in the transform matrices and the performance of the proposal in the BMS1.1 environment.

It is reported that the numbers and description provided by the proponents are correct. The fast algorithm reduces the number of arithmetic operations by approximatively 60% compared to the direct matrix product, leading to 2%(AI) 4% (RA) and 4% (LDB) BMS1.1 encoding time reduction and 3-5% decoding time reduction. The coding performance, through bdrate measurement, is merely not affected.

# Introduction

In AMT of BMS-1.1, DCT-5/8 and DST-1/7 are used as primary separable transform for coding residual blocks. These four additional transform types are implemented as matrix multiplication, it is the purpose of JVET-K0291 to provide a fast algorithm for the DST-7. As the DCT-8 is tied with the DST-7 by the duality operator, the fast algorithm for the DCT-8 inherits from the DST-7 implementation.

Therefore, a fast algorithm is provided for 2 AMT kernels.

# Proposed method

In order to reduce the number of operations, repeating patterns are identified in the transform matrices and are exploited to reduce the processing requirements. 16x16, 32x32 and 64x64 transforms are considered.

DCT-8/DST-7 transform matrices coefficients are slightly modified to provide an mathematical equivalent to the fast algorithm. As such the provided DCT-8/DST-7 transform kernels are not identical to those of BMS.

Few coefficients are affected, for example the table below highlights (in red) the coefficients changed in comparison with the BMS matrix for DST-7 sizes 16 and 32. The level of magnitude change is in the order of +/-1.

DST-7 16x16 10 bits highlighting coefficients changed.

DST-7 32x32 10 bits highlighting coefficients changed.

With the proposed matrices the DST-7/DCT-8 can either be implemented using the matrix form or the provided fast algorithm, both ways lead to identical results.

The number of operations required by the proposed fast algorithms were also verified, our evaluation does not deviate from the proponent’s. The number of shift operations is identical to the BMS1.1 while the number of additions and multiplications is reduced as follows: the number of multiplications represents 50%, 61% and 54% of the initial amount of operations, for sizes 16, 32 and 64, while the number of additions is affected by the ratios of 65%, 72% and 58%. Globally the number of operations is therefore reduced by roughly 40%.

The coding performance has been evaluated in the BMS1.1 environment, corresponding to test #2 in the proponent contribution. The numbers obtained in our independent evaluation are comparable to those of the proponents and are summarised below. DECODING TIMES WILL BE PROVIDED in the next release

The bdrate is merely not affected by the modified coefficients and coding and decoding runtimes are consistently reduced by 1-5%.

# Conclusion

The exactness of the elements brought by the proponents of JVET-K0291 have been reported, no deviation from their proposal have been observed.

# Patent rights declaration(s)

Orange, b<>com may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).