| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0430-v1 |
| --- | --- |

| Title: | CE5-related: State-based probability estimator |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Heiner Kirchhoffer, Jan Stegemann, Detlev Marpe, Heiko Schwarz, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-184 heiner.kirchhoffer@ hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

An extension of the state-based probability estimator of VTM-1.0 to two states per context model is proposed. The transition table size is reduced from 64 to 32 elements and the two states per context model require 8 and 12 bit, respectively. Experimental results for the VTM configuration show overall luma BD rate reductions of -0.67%, -0.45%, and -0.41% for AI, RA, and LB, respectively. Experimental results for BMS will be provided as soon as possible.

# Introduction

This proposal consists of two core elements. First, a state-based probability estimator is presented. The derivation of the subinterval range is the second part.

# Proposed method

The probability estimation method of this proposal stores two signed integer state variables S0 and S1 per context model which require 8 and 12 bit, respectively. The update after encoding or decoding a bin is according to the following pseudo code:

if( bin )

{

S0 += Next[ 16 + (S0 >> 3) ] >> 1;

S1 += Next[ 16 + (S1 >> 7) ];

}

else

{

S0 -= Next[ 16 + (-S0 >> 3) ] >> 1;

S1 -= Next[ 16 + (-S1 >> 7) ];

}

The lookup table ‘Next’ is defined as follows:

Next[32] = { 171, 151, 132, 114, 98, 82, 68, 56, 45, 35, 28, 21, 16, 11, 8, 5, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 0 };

The derivation of the range subinterval of the LPS employs the lookup table ‘rangeTabLps’ of VTM, but with dimensions 32x16 and with adapted values in order to reflect the correct probability values:

R_lps = rangeTabLps[ abs( (S0 << 4) + S1 ) >> 7][(range >> 4) & 15];

Note that other dimensions of the lookup table are possible. It is expected that a 32x8 table only has a small effect on the compression efficiency.

# Design aspects (informal)

The relationship between state variables and the probability of the less probable symbol (LPS) is based on the following equation:

PLPS = 0.5 * a|(S0 << 4)+S1| with a = 0.9989407902

The value of the LPS is 1 if (S0 << 4)+S1 is negative and 0 otherwise. In comparison, VTM uses one 6 bit state variable S and

PLPS = 0.5 * aS with a  0.949217.

The value of the more probable symbol (MPS) is stored using a separate bit so that 7 bit per context model are required in total.

# Experimental results

The probability estimator was implemented in BMS-1.0 and simulations were conducted according to [1]. Runtimes are not accurate due to the simulation conditions (runtimes vary depending on the cluster load).

VTM configuration, AI:

VTM configuration, RA:

VTM configuration, LB:

BMS configuration, AI:

BMS configuration, RA:

BMS configuration, LB:

Missing results for BMS configuration will be provided as soon as possible.

## rangeTabLPS size 32x8 (without cross-check)

In this section shows the effect of reducing the rangeTabLPS size from 32x16 to 32x8.

VTM configuration, AI:

VTM configuration, RA:

VTM configuration, LB:

## Comparison to CE5

The proposed algorithm using the 32x8 rangeTabLPS is compared to a CE5 test using the counter-based probability estimator with 10 and 14 bit per context model and a 32x8 rangeTabLPS without custom window sizes (noCW).

VTM configuration, AI:

VTM configuration, RA:

VTM configuration, LB:

# References

- J.Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, 9th meeting, San Diego, CA, USA, April 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).