| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0439 |
| --- | --- |

| Title: | Crosscheck for CE11-1.1 on composite reference picture (JVET-K0156) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wenhao Zhang F22, Building B, POSCO Center No. 13, Wangjing East Park Chaoyang District, Beijing, China | Email: | wenhao.zhang@hulu.com |
| Source: | Hulu LLC. |  |  |

_____________________________

# Abstract

This contribution is a crosscheck report for CE11-1.1 on composite reference picture, which is proposed in JVET-K0156. By the time when this report is uploaded, it is confirmed the available partial results match with the data provided by the proponents.

# Introduction

The software package provided by the proponents was evaluated under the following testing environment,

- Testing conditions follow the specification of [1]

- RA and LDB configurations were tested for VTM and BMS

- The source code was compiled by using GCC v6.4.0

- The testing jobs were running on Intel(R) Xeon(R) CPU E5-2620 v4 @ 2.10GHz

It’s observed the data for RA configuration has mismatch with proponents’ results slightly. The PSNR values are same, while the bitrates have difference. It may be caused by the accuracy loss during the log calculation, or bitstream header settings.

The available results are shown in the following tables, and detailed data can be found in the attached spreadsheet. It should be noted that the encoding/decoding runtime is not accurate due to the workload imbalance when running the anchor jobs and the crosscheck jobs.

# Experimental results (available)

Table 1 Results of BMS RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.41% | 0.36% | 0.40% | 136% | 116% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.67% | 0.34% | 0.38% | 136% | 131% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 2 Results of BMS LDB configuration

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -2.72% | -5.14% | -5.02% | 141% | 114% |
| Class E | 0.18% | -0.14% | -0.37% | 135% | 105% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.06% | 0.31% | -0.19% | 151% | 140% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 3 Results of VTM RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.83% | 0.54% | 0.68% | 132% | 107% |
| Class A2 | 0.72% | 0.59% | 0.64% | 142% | 107% |
| Class B | 0.38% | -0.08% | 0.10% | 139% | 106% |
| Class C | 0.60% | 0.46% | 0.34% | 175% | 139% |
| Class E |  |  |  |  |  |
| Overall | 0.60% | 0.32% | 0.39% | 147% | 115% |
| Class D | 1.28% | 0.91% | 0.95% | 168% | 132% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 4 Results of VTM LDB configuration

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.76% | -0.86% | -1.14% | 115% | 108% |
| Class C | -2.53% | -4.18% | -3.50% | 159% | 138% |
| Class E | 0.46% | 0.26% | 0.00% | 152% | 151% |
| Overall | -1.05% | -1.69% | -1.64% | 137% | 127% |
| Class D | 0.07% | -0.07% | 0.23% | 142% | 121% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# References

- J. Boyce, K. Suehring, X. Li, V. Seregin, JVET-J1010, “JVET common test conditions and software reference configurations”, Apr. 2018.