| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0441_v1 |
| --- | --- |

| Title: | Crosscheck of JVET-K0175: CE3-related: Advanced MPM based on intra reference line selection scheme |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Liang Zhao | Email: | leolzhao@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the cross-checking of contribution JVET-K0175 “CE3-related: Advanced MPM based on intra reference line selection scheme”.

It is reported that VTM AI and RA results and BMS AI results obtained during this cross-checking match the results described in JVET-K0175.

Introduction

In JVET-K0175 [1], it is proposed to only allow angular most probable modes (MPMs) for non-zero reference line index and reference line index is signaled after MPM flag. If the signaled mpm flag is false and reference line index is not signaled. If the signaled mpm flag is true and current intra prediction is not Planar or DC mode, reference line index is signaled.

Experimental results

The tests were performed on top of BMS 1.0, using either VTM or BMS configuration files. The SIMD level was set to SSE42. The simulations are performed on Linux cluster, using the common test condition described in [2]. The partial results for the test described in [1] are show in the table below. They match the result described in [1].

Table 1: Coding performance of JVET-K0175 over VTM-1.0.

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.38% | -0.11% | -0.05% | 101% | 97% | -0.16% | -0.09% | -0.04% | 100% | 100% |
| Class A2 | -0.35% | -0.02% | -0.01% | 101% | 104% | -0.15% | 0.17% | 0.08% | 97% | 96% |
| Class B | -0.51% | -0.22% | -0.24% | 102% | 97% | -0.22% | -0.07% | -0.09% | 99% | 99% |
| Class C | -0.61% | -0.19% | -0.22% | 100% | 95% | -0.30% | -0.10% | 0.02% | 101% | 100% |
| Class E | -0.38% | -0.29% | -0.06% | 100% | 95% |  |  |  |  |  |
| Overall | -0.46% | -0.17% | -0.14% | 101% | 97% | -0.22% | -0.04% | -0.02% | 99% | 99% |
| Class D | -0.29% | -0.13% | 0.04% | 98% | 93% | -0.09% | 0.16% | 0.00% | 100% | 98% |

Table 2: Coding performance of JVET-K0175 over BMS-1.0.

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.12% |  | 0.00% |  | 0.03% |  | 100% |  | 98% |  | -0.04% |  | -0.06% |  | 0.05% |  | 100% |  | 102% |  |
| Class A2 | -0.19% |  | -0.01% |  | 0.02% |  | 101% |  | 100% |  | -0.12% |  | 0.09% |  | 0.12% |  | 100% |  | 100% |  |
| Class B | -0.37% |  | -0.12% |  | -0.18% |  | 100% |  | 99% |  | -0.19% |  | 0.09% |  | -0.01% |  | 97% |  | 97% |  |
| Class C | -0.42% |  | -0.22% |  | -0.15% |  | 102% |  | 102% |  | -0.14% |  | 0.04% |  | -0.24% |  | 99% |  | 99% |  |
| Class E | -0.25% |  | 0.00% |  | -0.08% |  | 98% |  | 92% |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.29% |  | -0.08% |  | -0.09% |  | 100% |  | 99% |  | -0.13% |  | 0.05% |  | -0.03% |  | 99% |  | 99% |  |
| Class D | -0.10% |  | 0.02% |  | 0.02% |  | 99% |  | 99% |  | -0.01% |  | -0.22% |  | -0.16% |  | 99% |  | 98% |  |

Conclusion

The results obtained in the crosscheck match the results presented in JVET-K0175.

References

- H.-Y. Jiang and Y.-J. Chang, “CE3-related: Advanced MPM based on intra reference line selection scheme,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K0175, Ljubljana, SI, 10–18 July 2018.

- J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J1010, San Diego, CA, USA, 10–20 Apr. 2018.