| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0442 |
| --- | --- |

| Title: | Crosscheck of JVET-K0169: CE3-related: Block Shape Adaptive Intra Prediction Directions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Liang Zhao | Email: | leolzhao@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the cross-checking of contribution JVET-K0169 “CE3-related: Block Shape Adaptive Intra Prediction Directions”.

It is reported that VTM AI results and BMS AI results obtained during this cross-checking match the results described in JVET-K0169.

Introduction

In JVET-K0169 [1], a block shape-based adaptation of the set of intra prediction directions for the QTBT structure is proposed. If the shape of the block is square, then the already defined directions in the JVET VTM/BMS remain unchanged. Otherwise, the searched directions are modified according to the shape of the block. The total number of angular directions for any block remains the same as in VTM/BMS.

Experimental results

The tests were performed on top of BMS 1.0, using either VTM or BMS configuration files. The SIMD level was set to SSE42. The simulations are performed on Linux cluster, using the common test condition described in [2]. The partial results for the test described in [1] are show in the table below. They match the result described in [1].

Table 1: Coding performance of JVET-K0169 over VTM-1.0.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.12% | -0.21% | 98% | 98% |
| Class A2 | -0.17% | -0.09% | -0.16% | 99% | 98% |
| Class B | -0.19% | -0.27% | -0.20% | 101% | 100% |
| Class C | -0.40% | -0.59% | -0.72% | 98% | 98% |
| Class E | -0.25% | -0.17% | -0.18% | 96% | 96% |
| Overall | -0.24% | -0.27% | -0.31% | 99% | 98% |
| Class D | -0.30% | -0.39% | -0.41% | 97% | 96% |

Table 2: Coding performance of JVET-K0169 over BMS-1.0.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.34% | -0.34% | -0.38% | 100% | 97% |
| Class E | -0.29% | -0.22% | -0.31% | 97% | 92% |
| Overall |  |  |  |  |  |
| Class D | -0.24% | -0.04% | -0.17% | 101% | 96% |

Conclusion

The results obtained in the crosscheck match the results presented in JVET-K0169.

References

- G. Rath, F. Urban and F. Racapé, “CE3-related: Block Shape Adaptive Intra Prediction Directions,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K0169, Ljubljana, SI, 10–18 July 2018.

- J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J1010, San Diego, CA, USA, 10–20 Apr. 2018.