| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0445-v1 |
| --- | --- |

| Title: | Compression performance report of VTM/BMS for 8K test sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | +81-3-5494-3353 iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution reports on compression performance of VTM-1.0 and BMS-1.0 associated with 8K test sequences. The clips span a wide range of spatial temporal characteristics and portray content typical of broadcasting programs. Three ITU-R BT.2020 SDR test sequences have been encoded with HEVC test model (HM-16.16) and VVC test model (BMS-1.1 with VTM/BMS configurations). The simulation results show similar tendency of bit rate saving and encoding/decoding complexity for 8K test sequences to those for CTC sequences. It is confirmed that current BMS-1.1 software works for 8K sequences as expected.

Introduction

Versatile Video Coding (VVC) project has been launched jointly by ITU-T SG16 WP3 Q.6 (VCEG) and ISO/IEC JTC 1/SC 29/WG 11 (MPEG) in April 2018. MPEG document N17074 “Requirements for a Future Video Coding Standard v5” addresses the target examples of the existing markets which include 8K UHDTV broadcasting.

Although the performance of VVC Test Model (VTM) is evaluated using a wide range of resolution from 240p to 4K, the performance for 8K test sequences is not evaluated. There might be the case that some unexpected overflow appears in bit budget control of rate control for high resolution sequences.

To verify our test model works as expected for 8K test sequences as well, this contribution presents compression performance for the 8K test sequences using BMS-1.1 with VTM and BMS configurations, assuming HEVC (HM-16.16 with main10 configuration) as the anchor.

Test sequences

The 8K test sequences used for this test are parts of the test materials “Ultra-high definition/wide-color-gamut standard test sequences – Series B” distributed by Japanese academic society, the Institute of Image Information and Television Engineers (ITE). NOTE that these 8K test sequences are only provided to the licensee and not allowed to redistribute.

For more details, readers refer to the following address and contact information.

Web site: http://www.ite.or.jp/content/test-materials/uhdtv_b

Contact: ite@ite.or.jp

The test sequences are captured by 8K cameras. The specifications of the 8K camera used for shooting are listed Table 1.

Table 1. Specifications of 8K camera

| Pickup device | 3-chip 33-M pixel CMOS image sensors |
| --- | --- |
| Sensor size | 29.8mm×16.4mm |
| Image resolution | 7680×4320 |
| Frame rate | 59.94fps, progressive |
| Bit depth | 12 bit |
| Video range | Limited |
| Signal format | R′G′B′ 4:4:4 |
| Color space | ITU-R Rec. BT.2020 |

The captured sequences were converted from R′G′B′ 4:4:4 12bit to Y′C′BC′R 4:2:0 10bit by color space conversion and down-sampling process as shown in Figure 1.

Figure 1. Conversion process of R′G′B′ to Y′C′BC′R

Conversion from 4:4:4 to 4:2:0 was done by HDRTools utility, where {1, 6, 1} filter for horizontal downsampling and {1, 1} filter for upsampling were used.

The thumbnails and brief descriptions of the test sequences are listed in Table 2.

Table 2. Thumbnails and brief description of the 8K test sequences

| Thumbnail | Brief description |
| --- | --- |
| b01_WaterPolo-Goal_8K | Random motion in the water surface. Fixed camera |
| b05_HorseRace-Finish_8K | Panning camera motion Detailed texture on grass |
| b09_Marathon-PanningDown_8K | Complex and colorful texture Panning down camera motion |

Simulation results

This section provides the results of the coding experiments over the 8K test sequences. The content has been compressed by HM-16.16 with randomaccess_main10 configuration and BMS-1.1 with both randomaccess_vtm and randomaccess_bms configurations. Quantization parameter is set equal to {22, 27, 32, 37}. Although the number of frame of the 8K test sequences is all 900, selected 33 frames of each test sequence are compressed since the coding experiments for the while frames of 8K test sequences takes a plenty of time. Table 2 and 3 depict the obtained averaged BD-bitrate values. For more details, the spreadsheet is attached to this contribution.

Table 1. BD bitrate results of BMS-1.1 with VTM configuration over HM-16.16

|  | BMS-1.1 with VTM config over HM-16.16 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| b01_WaterPolo-Goal_8K_from00380_to00412 | -6.61% | -9.38% | -21.89% | 380% | 100% |
| b05_HorseRace-Finish_8K_from00252_to00284 | -9.22% | -23.36% | -31.60% | 140% | 88% |
| b09_Marathon-PanningDown_8K_from00764_to00796 | -5.26% | -9.88% | -13.22% | 266% | 104% |
| All | -7.03% | -14.21% | -22.23% | 242% | 97% |

Table 2. BD bitrate results of BMS-1.1 with BMS configuration over HM-16.16

|  | BMS-1.1 with BMS config over HM-16.16 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| b01_WaterPolo-Goal_8K_from00380_to00412 | -18.75% | -24.00% | -93.16% | 1906% | 223% |
| b05_HorseRace-Finish_8K_from00252_to00284 | -29.82% | -57.83% | -105.45% | 821% | 258% |
| b09_Marathon-PanningDown_8K_from00764_to00796 | -23.32% | -22.57% | -44.60% | 1273% | 235% |
| All | -23.97% | -34.80% | -81.07% | 1258% | 238% |

It was observed that bitrate saving over HM for 8K test sequences denotes similar tendency to that for the CTC sequences. Encoding and decoding complexity increments for the test also show similar tendency to those for the CTC sequences.

Conclusion

This contribution reports on compression performance of VTM-1.0 and BMS-1.0 associated with 8K test sequences. Three ITU-R Rec. BT.2020 SDR test sequences have been encoded with HEVC test model (HM-16.16) and BMS-1.1 with both VTM and BMS configurations. The simulation results show similar tendency of compression performance and encoding/decoding complexity for 8K test sequences to those for CTC sequences. It is confirmed that current BMS-1.1 software works for 8K sequences as expected.

Patent rights declaration(s)

NHK does not have any current or pending patent rights relating to the technology described in this contribution.