| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0450 |
| --- | --- |

| Title: | Current picture referencing for intra pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Wang Fangdong Chen | Email: | wangli7@hikvision.com chenfangdong@hikvision.com |
| Source: | Hangzhou Hikvision Digital Technology Co. Ltd., Hangzhou, China |  |  |

_____________________________

- Abstract

This contribution presents the test results of current picture referencing method (CPR) for intra pictures. In the CE8, the CPR is implemented for the Luma and Chrome comportment as a whole, and the details can be found in Chapter 2.1 of JVET-J0042 [1]. The testing results report that the CPR brings gains on both natural sequences and screen content sequences.

Description of CPR for intra frames

The CPR mode is regarded as inter mode in the intra flames. Thus there are intra blocks and inter blocks in the intra frames. Each block should be signaled to indicate its prediction type. Then the CPR method is basically inherit from the inter frame. Also for blocks predicted by CPR, the residual is transformed using the same transform set as the inter prediction residual.

Fig. 1 The syntax of inter block in proposed CPR method

However, the syntax has been changed. It is no longer efficient to code the skip_flag syntax for each block because the proportion of blocks choosing skip mode in this case is rather low. Therefore, in the proposed method the skip_flag syntax is omitted when encoding intra frame. The details are shown in the Fig. 1.

Experimental Results

Tested under the CE description in [2] and the common test conditions in [3], the proposed method has evaluated with AI and RA configurations.Table 1 presents the performance in VTM configuration and Table 2 presents that in BMS configuration.

The results compared to VTM 1.0 are shown in Table 1. It can be seen that under all intra configuration, the proposed scheme can achieve 1.74% (Y) coding gains in natural sequences with AI configuration and achieve much more gains in SCC sequences (Class F: 13.63%, TGM: 38.33%). The encoding time is high, however the decoding time almost remains the same.

The results compared to BMS 1.0 are shown in Table 2. The gains of nature sequences are not low, and those of screen content sequences are still significant.

Table 1 Comparison of the proposed method with VTM 1.0

| Class | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc T | Dec T | Y | U | V | Enc T | Dec T |
| Class A1 | -0.53% | -0.95% | -0.80% | 216% | 101% | -0.07% | -0.22% | -0.15% | 101% | 105% |
| Class A2 | -2.51% | -4.45% | -3.62% | 219% | 100% | -0.74% | -2.09% | -1.29% | 103% | 102% |
| Class B | -1.82% | -3.00% | -3.30% | 213% | 101% | -0.67% | -1.85% | -1.93% | 106% | 113% |
| Class C | -1.45% | -2.04% | -2.53% | 212% | 102% | -0.37% | -0.83% | -1.01% | 105% | 105% |
| Class D | -0.71% | -1.02% | -1.11% | 197% | 101% | -0.20% | -0.46% | -0.65% | 106% | 104% |
| Class E | -2.43% | -3.03% | -3.50% | 212% | 98% |  |  |  |  |  |
| Class F | -13.63% | -13.85% | -14.25% | 210% | 98% | -9.14% | -9.70% | -9.63% | 115% | 100% |
| Class TGM | -38.33% | -38.52% | -38.60% | 221% | 83% | -12.93% | -13.23% | -13.13% | 108% | 103% |
| CTC All | -1.74% | -2.69% | -2.80% | 214% | 101% | -0.49% | -1.30% | -1.20% | 104% | 107% |

Table 2 Comparison of the proposed method with BMS 1.0

| Class | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc T | Dec T | Y | U | V | Enc T | Dec T |
| Class A1 | -0.15% | -0.22% | -0.24% | 111% | 98% | -0.01% | -0.04% | -0.02% | 99% | 102% |
| Class A2 | -1.86% | -2.27% | -1.80% | 110% | 99% | -0.58% | -1.03% | -0.74% | 101% | 102% |
| Class B | -1.26% | -1.97% | -2.21% | 109% | 98% | -0.48% | -0.98% | -1.34% | 100% | 102% |
| Class C | -0.68% | -0.75% | -0.78% | 108% | 101% | -0.11% | 0.01% | -0.67% | 100% | 104% |
| Class D | -0.46% | -0.47% | -0.64% | 106% | 98% | -0.10% | -0.28% | -0.44% | 100% | 102% |
| Class E | -1.54% | -1.74% | -2.50% | 111% | 99% |  |  |  |  |  |
| Class F | -11.49% | -10.74% | -11.41% | 113% | 97% | -8.00% | -7.30% | -8.13% | 103% | 106% |
| Class TGM | -35.88% | -35.26% | -35.48% | 107% | 85% | -11.91% | -11.38% | -11.71% | 101% | 102% |
| CTC All | -1.10% | -1.42% | -1.54% | 109% | 99% | -0.31% | -0.54% | -0.78% | 100% | 103% |

Conclusion

The CPR in intra frame can achieve obvious coding gains in both VTM and BMS configurations for all kinds of sequences, especially in screen content sequences. Compared to VTM, the complexity is 2 times more. However, compared with BMS, only a little complexity increases. We may study more about CPR in the CE.

References

| [1] | X. Zuo, L. Wang, F. Chen, X. Song " Intra block copy for intra-frame coding", JVET-J0042, San Diego, US, 2018 |
| --- | --- |
| [2] | X. Xu, K. Müller and L. Wang, "Description of Core Experiment 8: Current Picture Referencing", JVET-J1028, San Diego, US, 2018. |
| [3] | J. Boyce, K. Suehring, X. Li and V. Seregin, "JVET common test conditions and software reference configurations", JVET-J1010, San Diego, US, 2018. |

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).