| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-Kxxxx-v1 |
| --- | --- |

| Title: | AHG5: Proposal of template for comparing memory bandwidth |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ryoji HASHIMOTO Seiji Mochizuki | Tel: Email: | ryoji.hashimoto.te@renesas.com seiji.mochizuki.zc@renesas.com |
| Source: | Renesas Electronics Corporation |  |  |

_____________________________

- Abstract

This contribution provides a template to compare memory bandwidth measured with VTM and BMS software. This template makes it easy to check and compare memory bandwidth in the test models.

Introduction

In 10th JVET held in San Diego, the tool to measure the memory bandwidth is adopted in VTM and BMS. A template summarizes the memory bandwidth in CTC helps analysis such as comparison between VTM and BMS. Motivated by this tendency, this contribution proposes a template to check memory bandwidth.

Introduction of a template

A template is created as an Excel file. This template consists from “summary”, “detail” of each condition and “data” sheets. Structure of each sheet is very similar to the template for comparing BD-PSNR. “detail” sheets are prepared for each configuration such as random access and each comparison such as between VTM vs tested.

Figure 1 shows an example of “Summary” sheet. In “Summary” sheet, following information is available.

  - Ratio of average memory bandwidth among tested data for each class and overall

  - Ratio of worst memory bandwidth among tested data for each class and overall

As same as the template for comparing BD-PSNR, much better cell is colored with green, much worse cell is colored with pink automatically. In the current setting, cell whose value is less than 70% becomes green, more than 130% becomes pink. This threshold is tentative so that we need to define this based on tendency of memory bandwidth in its evaluation process.

Figure 2 shows an example of “detail” sheet. This sheet provides memory bandwidth information in each sequence and QP. Color condition is same as “summary” sheet.

Figure 3 shows an example of “data” sheet. Users need to edit only this sheet based on the simulation results. All other sheet is automatically updated based on the values in “data” sheet.

Figure 1 Example of “summary” sheet

Figure 2 Example of “detail” sheet

Figure 3 Example of “data” sheet

Conclusion

This contribution proposes a template to evaluate memory bandwidth. This template helps analysis of memory bandwidth, especially in activity of AHG5.

Patent rights declaration(s)

Renesas Electronics does not have any current or pending patent rights relating to the technology described in this contribution.