| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0454 |
| --- | --- |

| Title: | Cross-check of JVET-K0202: Non-CE2: On SAO Edge Offset classification |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | F. Galpin, P. Bordes | Tel: Email: | franck.galpin@technicolor.com philippe.bordes@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This document reports the cross-check results for the results reported in input document JVET-K0202, non-CE2, on SAO Edge Offset classification [1]. The cross-checked contribution modifies the SAO Edge offset classification and offsets coding. The source code to perform the tests were provided by the proponent Canon.

# Introduction

The tests were performed on top of BMS 1.0, using either VTM or BMS configuration files. The SIMD level was set to SSE42. Spot checking tests were performed in both VTM and BMS configurations.

# Results

## Test 1

In test 1, the proponent changes the coding of the offsets related to EdgeIndex and signals the sign function used for edge offset.

## Over VTM

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class C | -0.08% | -0.08% | -0.12% | 102% | 105% |
| Class D | -0.03% | -0.07% | -0.12% | 102% | 106% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.23% | -0.62% | -0.13% | 100% | 100% |
| Class C | -0.15% | -0.29% | -0.25% | 100% | 102% |
| Class D | -0.05% | -0.22% | -0.17% | 100% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class C | -0.07% | -0.28% | -0.18% | 101% | 92% |
| Class D | -0.11% | 0.35% | 0.23% | 100% | 102% |

## Test 2

In test 2, an additional, signaled parameter, is used to change the sign function used to determine the EdgeIndex.

## Over VTM

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class C | -0.08% | -0.14% | -0.18% | 102% | 105% |
| Class D | -0.03% | -0.11% | -0.15% | 103% | 107% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.28% | -0.71% | -0.27% | 101% | #NUM! |
| Class C | -0.15% | -0.24% | -0.38% | 101% | #NUM! |
| Class D | -0.08% | -0.22% | -0.30% | 101% | #NUM! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class C | -0.14% | -0.89% | -0.59% | 101% | 92% |
| Class D | -0.07% | -0.05% | -0.08% | 101% | 101% |

## Over BMS

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class C | 0.05% | -0.17% | -0.01% | 101% | 102% |
| Class D | 0.01% | -0.06% | -0.09% | 102% | 103% |

# Conclusion

The results obtained in the cross-check match the results presented in JVET-K0202.

# Reference

- G. Laroche, J. Taquet, C. Gisquet, P. Onno, “Non-CE2: On SAO Edge Offset classification” JVET-K0202, Jul. 2018, Ljubljana, SI.

# Patent rights declaration(s)

Technicolor does not have any current or pending patent rights relating to the technology described in this contribution.