| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0459 |
| --- | --- |

| Title: | Crosscheck for CE7-5.1 on residual sign prediction in transform domain (JVET-K0044) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Min Gao, Wenhao Zhang F22, Building B, POSCO Center No. 13, Wangjing East Park Chaoyang District, Beijing, China | Email: | min.gao@hulu.com wenhao.zhang@hulu.com |
| Source: | Hulu LLC. |  |  |

_____________________________

# Abstract

This contribution is a crosscheck report for CE7-5.1 on residual sign prediction in transform domain, which is proposed in JVET-K0044. By the time when this report is uploaded, it is confirmed the available partial results match with the data provided by the proponents.

# Introduction

The software package provided by the proponents was evaluated under the following testing environment,

- Testing conditions follow the specification of [1]

- AI, RA and LDB configurations were tested for VTM and BMS

- The source code was compiled by using GCC v6.4.0

- The testing jobs were running on Intel(R) Xeon(R) CPU E5-2620 v4 @ 2.10GHz

The available results are shown in the following tables, and detailed data can be found in the attached spreadsheet. It should be noted that the encoding/decoding runtime is not accurate due to the workload imbalance when running the anchor jobs and the crosscheck jobs.

# Experimental results (available)

Table 1 Results of BMS AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.55% | -0.32% | -0.42% | 103% | 106% |
| Class A2 | -0.42% | -0.29% | -0.42% | 105% | 112% |
| Class B | -0.43% | -0.30% | -0.35% | 107% | 114% |
| Class C | -0.26% | -0.31% | -0.27% | 129% | 132% |
| Class E | -0.65% | -0.47% | -0.55% | 93% | 103% |
| Overall | -0.45% | -0.33% | -0.39% | 108% | 114% |
| Class D | -0.22% | -0.11% | -0.18% | 124% | 145% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 2 Results of BMS RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | -0.62% | -0.52% | -0.52% | 121% | 129% |
| Class B | -0.66% | -0.35% | -0.57% | 110% | 113% |
| Class C | -0.51% | -0.41% | -0.73% | 113% | 107% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.31% | -0.41% | -0.63% | 119% | 121% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 3 Results of BMS LDB configuration

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.66% | -0.46% | -0.04% | 108% | 120% |
| Class C | -0.62% | -0.28% | -0.51% | 113% | 102% |
| Class E | -0.50% | -0.62% | 0.20% | 112% | 94% |
| Overall | -0.61% | -0.44% | -0.14% | 111% | 107% |
| Class D | -0.35% | 0.05% | -0.44% | 121% | 129% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 4 Results of VTM AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.51% | -1.20% | -1.13% | 127% | 148% |
| Class A2 | -1.20% | -1.07% | -1.14% | 125% | 149% |
| Class B | -1.30% | -1.03% | -0.93% | 135% | 157% |
| Class C | -0.83% | -0.71% | -0.84% | 150% | 169% |
| Class E | -1.81% | -1.31% | -1.38% | 146% | 165% |
| Overall | -1.30% | -1.04% | -1.05% | 137% | 158% |
| Class D | -0.72% | -0.53% | -0.75% | 169% | 225% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 5 Results of VTM RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.11% | -0.94% | -0.74% | 115% | 146% |
| Class A2 | -1.00% | -0.75% | -0.81% | 112% | 125% |
| Class B | -1.13% | -0.75% | -0.89% | 114% | 127% |
| Class C | -0.83% | -0.60% | -0.83% | 125% | 144% |
| Class E |  |  |  |  |  |
| Overall | -1.02% | -0.75% | -0.83% | 117% | 135% |
| Class D | -0.57% | -0.45% | -0.81% | 130% | 137% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 6 Results of VTM LDB configuration

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.86% | -0.36% | -0.45% | 116% | 130% |
| Class C | -0.66% | -0.39% | -0.23% | 131% | 144% |
| Class E | -0.52% | -0.46% | -0.01% | 111% | 120% |
| Overall | -0.70% | -0.39% | -0.27% | 120% | 131% |
| Class D | -0.45% | -0.29% | -0.04% | 120% | 137% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# References

- J. Boyce, K. Suehring, X. Li, V. Seregin, JVET-J1010, “JVET common test conditions and software reference configurations”, Apr. 2018.