| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0462-v1 |
| --- | --- |

| Title: | Cross check of JVET-K0265: CE-6 related: Reduction of the number of core transforms in AMT |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Saurav Bandyopadhyay, Yuwen He, Yan Ye 9276 Scranton Road, Suite 300 San Diego, CA 92121 USA | Tel: Email: | Saurav.Bandyopadhyay@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

This document reports the cross-check for the results reported in input document JVET-K0265: CE-6 related: Reduction of the number of core transforms in AMT [1]. The cross-check contribution reports the results of the proposed transform core reduction on top of BMS software 1.1. with Random configuration. The following section provide results by replacing the implementations of AMT in BMS 1.1. The source code as well as the configuration for performed test were provided by the proponent Technicolor. The cross-check confirm the results reported by the proponents.

# Introduction

The source code as well as the configuration provided by the proponent Technicolor were used for the cross-check. 4 tests were performed to cross-check following AMT replacements in BMS 1.1.: DST-I approximated by DST-II, DCT-V approximated by DCT-II, DCT-VIII by DCT-IV and DST-VII by DST-IV, combining all the previous.

# Simulation results

Simulations results reported in this contribution are based on the Benchmark Set (BMS) configuration as in JVET-J1010 [2]. BMS 1.1. software is used as anchor. The results are generated using configuration for JVET-K0265 provided by the proponents for the following transform replacements.

## DST-I replaced by DST-II

The macro TCH_DST1_DST2 is enabled

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.06% | 0.04% | 102% | 100% |
| Class A2 | 0.06% | 0.25% | 0.17% | 101% | 100% |
| Class B | 0.02% | 0.25% | -0.11% | 101% | 101% |
| Class C | -0.01% | 0.05% | -0.04% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.16% | -0.01% | 102% | 100% |
| Class D | -0.02% | 0.30% | 0.35% | 101% | 100% |

## DCT-V replaced by DCT-II

The macro TCH_DCT5_DCT2_8x8 is enabled

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.17% | 0.08% | 100% | 100% |
| Class A2 | 0.01% | 0.13% | 0.11% | 100% | 100% |
| Class B | 0.03% | 0.15% | 0.04% | 100% | 100% |
| Class C | 0.03% | 0.31% | -0.18% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.19% | 0.00% | 100% | 100% |
| Class D | 0.04% | -0.12% | -0.19% | 100% | 100% |

## DCT-VIII replaced by DCT-IV and DST-VII replaced by DST-IV

The macro TCH_DCT4_DST4_From_DCT2 is enabled

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.23% | 0.35% | 0.21% | 101% | 100% |
| Class A2 | 0.26% | 0.53% | 0.19% | 102% | 100% |
| Class B | 0.09% | 0.05% | 0.21% | 102% | 100% |
| Class C | -0.15% | -0.03% | -0.36% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.19% | 0.06% | 102% | 100% |
| Class D | -0.25% | -0.29% | -0.36% | 102% | 101% |

## Combining 2.1, 2.2 & 2.3 (DST-I replaced by DST-II, DCT-V replaced by DCT-II, DCT-VIII replaced by DCT-IV & DST-VII replaced by DST-IV)

The macros TCH_DST1_DST2, TCH_DCT5_DCT2, TCH_DCT4_DST4 are enabled

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.26% | 0.47% | 0.43% | 101% | 98% |
| Class A2 | 0.36% | 0.68% | 0.34% | 101% | 100% |
| Class B | 0.18% | 0.47% | 0.19% | 101% | 99% |
| Class C | -0.13% | 0.13% | -0.29% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.15% | 0.42% | 0.14% | 101% | 99% |
| Class D | -0.18% | -0.16% | -0.48% | 102% | 100% |

# Conclusion

The results from this cross-check are equivalent to the results provided by proponents in JVET-K0265.

# References

[1] K. Naser, F. Le Léannec, E. François, “CE6-related: Reduction of the number of core transforms in AMT”, JVET-K0265 JVET 11th Meeting: Ljubljana, SI, 10–18 July 2018.

[2] J. Boyce et. al., “JVET common test conditions and software reference configurations”, JVET-J1010, 10th Meeting, April 2018, San Diego, USA.

# Patent rights declaration(s)

InterDigital Communications Inc. does not have any current or pending patent rights relating to the technology described in this contribution.