| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0466 |
| --- | --- |

| Title: | Cross-check of JVET-K0142: AHG8: 360°-based in-loop filters for cubemap projection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Philippe Hanhart 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results for JVET-K0142. The simulation results reportedly match those provided by the proponents.

# Introduction

JVET-K0142 [1] proposes to modify the deblocking and sample adaptive offset (SAO) filters to reduce seam artifacts that occur at some face boundaries in cube-based layouts. Two different methods are proposed. In the first method, the two in-loop filters are disabled across the discontinuous edge boundaries. In the second method, the two in-loop filters are modified to use spherical neighbors instead of frame packed neighbors at face boundaries. Both methods require normative changes. The first method may be better suited for hardware implementations as it does not require to fetch samples from another part of the picture. However, the second method may improve the continuity at face boundaries and further reduce seam artifacts.

# Experimental Results

The simulations were performed using BMS-1.0 and 360Lib-6.0 with the changes proposed in [1]. The encoding was done in the modified cubemap projection (MCP) format using the Random Access configuration under the 360° video CTC [2]. The encoding results are summarized in Table 1 and Table 2 for method 1 (disabled filtering process cross the discontinuous edge) and method 2 (filtering process based on geometric continuity), respectively. Simulation results match those provided by the proponents. More details can be found in the spreadsheets in the contribution.

Table 1. BD-rate performance for method 1: disabled filtering process cross the discontinuous edge

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -0.23% | -0.90% | -1.40% | -0.17% | -0.73% | -0.90% |
|  | ChairLift | -0.32% | -0.11% | -0.25% | -0.26% | -0.03% | -0.10% |
|  | KiteFlite | -0.07% | -0.18% | -0.23% | -0.09% | -0.01% | -0.06% |
|  | Harbor | -0.25% | -0.97% | -1.35% | -0.24% | -0.68% | -0.86% |
|  | Trolley | -0.06% | -0.42% | -0.30% | -0.08% | -0.24% | -0.18% |
|  | GasLamp | -0.09% | -0.58% | -0.47% | -0.11% | -0.40% | -0.29% |
| Class S2 | Balboa | -0.30% | -0.48% | -0.44% | -0.21% | -0.28% | -0.08% |
|  | Broadway | -0.30% | -0.43% | -0.37% | -0.20% | 0.12% | -0.19% |
|  | Landing2 | -0.20% | -1.53% | -1.27% | -0.20% | -1.30% | -1.01% |
|  | BranCastle2 | -0.05% | -0.09% | -0.18% | -0.06% | -0.10% | -0.06% |
| Class S1 |  | -0.17% | -0.53% | -0.67% | -0.16% | -0.35% | -0.40% |
| Class S2 |  | -0.21% | -0.63% | -0.56% | -0.17% | -0.39% | -0.33% |
| Overall |  | -0.19% | -0.57% | -0.62% | -0.16% | -0.36% | -0.37% |

Table 2. BD-rate performance for method 2: filtering process based on geometric continuity

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -0.41% | -1.18% | -1.81% | -0.24% | -0.71% | -1.33% |
|  | ChairLift | -0.46% | -0.38% | -0.52% | -0.39% | -0.18% | -0.37% |
|  | KiteFlite | -0.08% | -0.28% | -0.27% | -0.11% | -0.15% | -0.16% |
|  | Harbor | -0.27% | -1.13% | -1.62% | -0.25% | -0.94% | -1.10% |
|  | Trolley | -0.07% | -0.55% | -0.43% | -0.08% | -0.39% | -0.34% |
|  | GasLamp | -0.08% | -0.66% | -0.54% | -0.12% | -0.57% | -0.39% |
| Class S2 | Balboa | -0.40% | -0.57% | -0.64% | -0.34% | -0.20% | -0.15% |
|  | Broadway | -0.40% | -0.37% | -0.45% | -0.33% | -0.16% | -0.55% |
|  | Landing2 | -0.28% | -1.75% | -1.49% | -0.29% | -1.52% | -1.30% |
|  | BranCastle2 | -0.12% | -0.29% | -0.42% | -0.11% | -0.28% | -0.34% |
| Class S1 |  | -0.23% | -0.70% | -0.87% | -0.20% | -0.49% | -0.62% |
| Class S2 |  | -0.30% | -0.74% | -0.75% | -0.27% | -0.54% | -0.59% |
| Overall |  | -0.26% | -0.72% | -0.82% | -0.23% | -0.51% | -0.60% |

# References

- S.-Y. Lin, L. Liu, C.-H. Shih, J.-L. Lin, H.-C. Lin, S.-K. Chang, C.-C. Ju, “AHG8: 360°-based in-loop filters for cubemap projection”, JVET-K0142, July 2018, Ljubljana, SI.

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-J1012, Apr. 2018, San Diego, USA.

# Patent rights declaration(s)

InterDigital Communications, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.