| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0468 |
| --- | --- |

| Title: | CE12-related: In-loop chroma refinement |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. François, C. Chevance, F. Hiron (Technicolor) | Tel: Email: | edouard.francois@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

Out-of-loop mapping function refinement was proposed for HDR video in JVET-J0022 and JVET-K0298. In this contribution, the technology is evaluated for chroma components when applied in-loop. The reported BD-rate variations in VTM1.0 for RA configuration are as follows. For HLG content, -4.26% for DE100, 0.06% for PSNRL100, 0.06%, -7.49% and -9.90% for wPSNR-Y, U and V. For PQ content, -1.61% for DE100, 0.05% for PSNRL100, 0.05%, -6.94% and -9.89% for wPSNR-Y, U and V. Encoder and decoder runtimes are both evaluated at 101% of the VTM1.0 runtimes.

# Introduction

Out-of-loop luma and chroma mapping refinement was proposed JVET-J0022 and JVET-K0298, in the context of HDR Dynamic Range Adaptation (DRA). The process aims at compensating the distortion resulting from the compression that may impact the inverse DRA process. The process may apply per slice. Refinement applies to the inverse DRA functions (LUTs), that are corrected by the refinement LUTs, modeled by piece-wise linear functions.

In this contribution, the application of the refinement for chroma is investigated when applied in-loop, without any mapping process. The performance on UHD/HD HDR clips for RA is demonstrated.

# Algorithm Description

## Cross-component chroma refinement in decoder

The proposed cross-component chroma refinement process operates as follows. The reconstructed signal after loop filtering (YR, UR, VR) is refined as follows:

  - Urefine = Max(0, Min( 2Bchr – 1, 2Bchr – 1 + LUTrefineU[ YIMcoloc ] * ( UR – 2Bchr – 1 ))

  - Vrefine = Max(0, Min( 2Bchr – 1, 2Bchr – 1 + LUTrefineV[ YIMcoloc ] * ( VR – 2Bchr – 1 ))

  - where YRcoloc is the co-located reconstructed luma sample

The look-up-tables LUTrefineU, LUTrefineV are built from piece-wise linear (PWL) tables, that can be signaled per slice (in the slice header). The pivot points of PWL tables are equi-distant, and the tables size is equal to 2S+1, S being a parameter signaled in the slice header. In practice, a size of 33 was used in this contribution. When tables are not signaled, no refinement applies to the current slice. In practice, in this contribution, the tables are sent only for slices of temporal level 0.

## Refinement tables derivation in encoder

The refinement tables derivation aims at minimizing the distortion of the reconstructed samples, after loop filtering, compared to the encoder input samples. The estimation of the PWL parameters for each table consists in defining the pivot points such as the rate-distortion over the slice is minimized. This is achieved by scanning one-by-one the pivot points (xk,yk)  (k being the index of the point), evaluating the resulting RD cost for a range of possible yk values, and selecting the yk value providing the lowest RD cost. The process can be iteratively repeated (in practice 2 iterations were achieved).

Fast implementations, avoiding repeated full picture scanning, can be used for the optimization process, as explained below.

For the chroma U (the same applies for V), the distortion over the picture I is computed as:

SSE=p in I(Uin(p)-Urefine(p))2

Urefine(p) can be derived as Urefine(p) = off + LUTrefineU[ Ydec(p) ] * ( Urec(p) – off ) (by neglecting the clipping), where off = 2Bchr-1

SSE=p in I(Uin(p)-off)2+LUTrefineUYrec(p)2.p in I(Urec(p)-off)2-2.LUTrefineUYrec(p).p in IUinp-off.(Urec(p)-off)

For the set SY = {p in I such that Ydec(p)= Y}, the cumulated error is

SSEY=p in SY(Uin(p)-off)2+LUTrefineUY2.p in SY(Urec(p)-off)2-2.LUTrefineUY.p in SYUinp-off.(Urec(p)-off)

And the total distortion SSE can be computed as:

SSE=Y=02Bluma-1SSEY

When changing a pivot point of the U (or V) refinement table, it is only required to recompute the new mapping values LUTrefineU[Y] for Y values impacted by the modification of the pivot point, and then to update the SSE for these new mapping values.

# Simulation Results

The in-loop cross-component chroma refinement is implemented in VTM1.0 software. The refinement process at decoder is implemented using fixed-point. The tests are done using the HDR test sequences, in RA configuration. Note that the reference is not the HDR CTC anchor, indeed VTM1.0 is used as in SDR case, not considering the HDR luma and chroma QP adaptation proposed for HDR, in order to also assess the impact on the conventional PSNR-Y, U, V. Related results are reported for the HDR metrics in the table below. Related xls file is attached to the contribution.

Table 1. BD-rate variations versus VTM1.0.

# Conclusion

In this contribution, performance of the cross-component chroma refinement, previously proposed in JVET-J0022 and JVET-K0298, is evaluated when the process is applied in the coding loop. Considering the observed coding efficiency benefits, it is proposed to continue investigating this technology in-loop, and further investigate its application for SDR content.

# Reference

- “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – medium complexity version”, JVET-J0022, San Diego, USA, April 2018.

- E. Francois, D. Rusanovskyy, et al, “CE12: report of dynamic range adaptation and DRA refinement”, JVET-K0298, Ljubljana, SI, July 2018.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).