| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0480-v1 |
| --- | --- |

| Title: | Non-CE9: A computational complexity analysis for DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou and Brian Heng 16340 West Bernardo DR San Diego, CA-92127, USA | Email: | minhua.zhou@broadcom.com brian.heng@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution provides a computational complexity analysis for the DMVR relative to the bi-directional 8x8 motion compensation (MC). Based on the analysis results it is recommended to 1) disable the DMVR for PU sizes smaller than 8x8, 2) make the refinement search range PU size dependent for the DMVR, and 3) study the impact of using short tap filters for the DMVR.

# Introduction

Figure 1: DMVR based on bilateral template matching

Referencing to [1], Figure 1 Illustrates the decoder-side motion vector refinement (DMVR) included in the BMS1.0.   During the bi-prediction operation, for the prediction of a PU, two prediction blocks, formed using a motion vector (MV) of list0 and a MV of list1, respectively, are combined to form a single prediction signal. In the decoder-side motion vector refinement (DMVR) method, the two motion vectors of the bi-prediction are further refined by a bilateral template matching process. The bilateral template matching applied in the decoder to perform a distortion-based search between a bilateral template and the reconstruction samples in the reference pictures in order to obtain a refined MV without transmission of additional motion information. The list0 and list 1 motion vectors, i.e. MV0 and MV1 in Figure 1, can be decoded from either the AMVP mode or the merge/skip mode, depending on the inter prediction mode used by the PU.

In DMVR, a bilateral template is generated as the weighted combination (i.e. average) of the two prediction blocks, from the initial MV0 of list0 and MV1 of list1, respectively, as shown in Figure 1. The template matching operation consists of calculating cost measures between the generated template and the sample region (around the initial prediction block) in the reference picture. For each of the two reference pictures, the MV that yields the minimum template cost is considered as the updated MV of that list to replace the original one. In one of implementations, nine MV candidates are searched for each list. The nine MV candidates include the original MV and 8 surrounding MVs with one luma sample offset to the original MV in either the horizontal or vertical direction, or both. Finally, the two new MVs, i.e., MV0′ and MV1′ as shown in Figure 1, are used for generating the final bi-prediction results. A sum of absolute differences (SAD) is used as the cost measure.

# Computational analysis

The DMVR has the following operation steps:

- Create the extended prediction blocks for list0 and list1 based on the initial motion vectors and the refinement search range

- Produce the bi-directional template by averaging the list0 and list1 prediction blocks

- Compute the SADs for nine locations and for list0 and list1 separately.

- Produce the final bi-directional prediction block based on the refined motion vectors

Assume an N x M PU with a fractional motion vector in both directions, the number of additions and multiplies for the DMVR can be computed as follows:

- Step 1): adds: 2x(ftDMVR-1*N+2*SRx*2*M+4*SRy+ftDMVR-1)

  - mults: 2x(ftDMVR*N+2*SRx*2*M+4*SRy+ftDMVR-1)

- Step 2):  adds:   N*M

- Step 3):  adds:  2*SRx+1*2*SRy+1*2*N*M-1

- Step 4):  adds:  N*M

For an N x M PU with a fractional motion vector in both directions, the number of additions and multiplies for bi-directional MC can be computed as follows:

- adds: 2x(ftMC-1*N*2*M+ftMC-1)

- mults: 2x(ftMC*N*2*M+ftMC-1)

where SRx x SRy are the refinement search range for the DMVR, ftDMVR and ftMC are the filter tap length of the interpolation filters for the DMVR and MC, respectively.

For memory bandwidth analysis, the memory bandwidth consumption of DMVR relative to MC of the same PU size is simply computed by (no memory alignment, burst size and reference block overlapping are considered due to lack of cache model):

2*SRx+N+ftDMVR-1*(2*SRy+M+ftDMVR-1)N+ftMC-1*(M+ftMC-1)

Table 1 summarizes the computational complexity and memory bandwidth consumption of the DMVR relative to 8x8 bi-directional MC for different PU sizes (in the table, the complexity ratio of an addition relative to a multiply is set to 0.25). As shown in Table 1, for the fixed refinement search range both the computational complexity and memory bandwidth consumption increase as the PU size decreases.  The DMVR of PU sizes below 8x8 appears extremely costly.

Table 1. Computational complexity and memory bandwidth consumption of DMVR w.r.t. 8x8 MC with fixed refinement search range

Table 2 summarizes the computational complexity and memory bandwidth consumption numbers of the DMVR relative to 8x8 bi-directional MC when the refinement search range is PU size dependent. As shown in Table 2, the refinement search range can be increased for large PU sizes under a given budget of computational complexity and memory bandwidth consumption.

Table 2. Computational complexity and memory bandwidth consumption of DMVR w.r.t. 8x8 MC with PU size dependent refinement search range

Table 3 summarizes the computational complexity and memory bandwidth consumption numbers of the DMVR relative to 8x8 bi-directional MC when the refinement search range is PU size dependent and the DMVR uses short tap filters. As shown in Table 3, when short tap filters are used for the DMVR the refinement search range can be further increased for different PU sizes under a given budget of computational complexity and memory bandwidth consumption.

Table 3. Computational complexity and memory bandwidth consumption of DMVR w.r.t. 8x8 MC with PU size dependent refinement search range and short tap filters for DMVR

# Recommendations

Based on the analysis above, it is recommended to

- Disable DMVR for PU sizes smaller than 8x8

- Make the refinement search range be PU size dependent

- And study the impact of using short tap filters for the DMVR

# References

[1] S. Esenlik and Y.W.Chen, “Description of Core Experiment 9 (CE9): Decoder side motion vector derivation”, JVET-J1029, the 10th JVET meeting, April 2018, San Diego, USA.

# Patent rights declaration(s)

Broadcom Inc. does not have current or pending patent rights relating to the technology described in this contribution.