| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0482 |
| --- | --- |

| Title: | CE3-related: MPM based multi-line intra prediction scheme |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, and Xiang Li, Shan Liu Hui-Yu Jiang, Yao-Jen Chang | Email: | {leolzhao, shanl, xinzzhao, xlxiangli}@tencent.com {teresa.hy.jiang, morriss.yj.chang}@foxconn.com |
| Source: | Tencent, Foxconn Technology Group |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to signal reference line index before intra prediction mode and only most probable mode (MPM) is allowed for non-zero reference line index. Moreover, Planar and DC modes are excluded from the MPM list derivation for non-zero reference line index. Simulation results reportedly show that, average -0.43% and -0.19% BD-rate is achieved on top of VTM-1.0 under all intra (AI) and random access (RA) configurations, and -0.XX% and -0.XX% BD-rate is achieved on top of BMS-1.0 under AI and RA configurations, respectively. When Intra65Ang is enabled, average -0.61% and -0.xx% BD-rate is achieved on top of VTM-1.0 under AI and RA configurations, respectively.

Introduction

Multi-line intra prediction [1], [2] was proposed to use more reference lines for intra prediction, and encoder decides and signals which reference line is used to generate the intra predictor. The reference line index is signaled before intra prediction modes, and Planar/DC modes are excluded from intra prediction modes in case a nonzero reference line index is signaled. In [3], an advanced MPM (AMPM) is proposed to signal the selected reference line for each CU coded by angular MPM modes.

Proposed MPM based multi-line intra prediction scheme

It is proposed to signal reference line index before intra prediction mode and only angular most probable mode (MPM) is allowed for non-zero reference line index. Moreover, planar and DC modes are excluded from the MPM list generation for non-zero reference line index. The number of MPM keeps the same with VTM1.0.

When non-zero reference line index is signaled, MPM flag of current block is not signaled and the MPM flag of current block is derived as true.

Experimental results

The proposed scheme is implemented on top of BMS-1.0 software [3]. The simulations are performed following AI and RA configurations on Linux cluster, using the common test condition described in [4].

Table 1: Coding performance of proposed method over VTM-1.0.

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.11% | -0.04% | 104% | 97% | -0.11% | -0.04% | 0.07% | 101% | 98% |
| Class A2 | -0.33% | -0.03% | 0.00% | 104% | 99% | -0.16% | 0.07% | -0.07% | 101% | 98% |
| Class B | -0.50% | -0.26% | -0.23% | 104% | 101% | -0.21% | -0.07% | -0.12% | 101% | 98% |
| Class C | -0.59% | -0.28% | -0.31% | 101% | 97% | -0.26% | -0.05% | 0.00% | 102% | 98% |
| Class E | -0.37% | -0.16% | -0.18% | 102% | 97% |  |  |  |  |  |
| Overall | -0.43% | -0.18% | -0.17% | 103% | 99% | -0.19% | -0.03% | -0.04% | 101% | 98% |
| Class D | -0.28% | -0.04% | -0.07% | 100% | 89% | -0.07% | 0.03% | 0.11% | 101% | 98% |

Table 2: Coding performance of proposed method over BMS-1.0.

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.39% | -0.20% | -0.22% | 101% | 99% | -0.19% | 0.10% | 0.07% | 97% | 95% |
| Class C | -0.45% | -0.13% | -0.12% | 101% | 105% | -0.16% | 0.20% | -0.14% | 96% | 93% |
| Class E | -0.29% | 0.09% | -0.10% | 99% | 95% |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.11% | 0.08% | 0.11% | 97% | 95% | 0.00% | 0.08% | 0.08% | 95% | 89% |

Table 3: Coding performance of proposed method over VTM-1.0 when Intra65Ang is enabled.

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -0.04% | 0.07% | 119% | 103% |  |  |  |  |  |
| Class A2 | -0.46% | -0.01% | -0.02% | 117% | 102% |  |  |  |  |  |
| Class B | -0.70% | -0.38% | -0.29% | 117% | 102% |  |  |  |  |  |
| Class C | -0.79% | -0.25% | -0.23% | 112% | 100% |  |  |  |  |  |
| Class E | -0.57% | -0.12% | -0.09% | 114% | 96% |  |  |  |  |  |
| Overall | -0.61% | -0.19% | -0.14% | 116% | 101% |  |  |  |  |  |
| Class D | -0.33% | 0.01% | -0.15% | 110% | 93% |  |  |  |  |  |

Conclusion

In this contribution, it is proposed to signal reference line index before intra prediction mode and only most probable mode (MPM) is allowed for non-zero reference line index. Simulation results reportedly show that, average -0.43% and -0.19% BD-rates are achieved on top of VTM-1.0 under all intra (AI) and random access (RA) configurations. It is proposed to adopt this proposal into VTM.

References

- Y.-J. Chang, P.-H. Lin, C.-L. Lin, J.-S. Tu, and C.-C. Lin, “Arbitrary reference tier for intra directional modes,” JVET-C0043, Geneva, CH, May 2016.

- J. Li, B. Li, J. Xu, R. Xiong, G. J. Sullivan, “Multiple line-based intra prediction,” JVET-C0071, Geneva, CH, May 2016.

- H.-Y. Jiang and Y.-J. Chang “CE3-related: Advanced MPM based on intra reference line selection scheme,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K0175, Ljubljana, SL, 10–20 July 2018.

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J1010, San Diego, CA, USA, 10–20 Apr. 2018.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Foxconn Technology Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).