| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0483-v1 |
| --- | --- |

| Title: | Cross-check of JVET-K0122: CE3-related: Alternative techniques for DC mode without division |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xingchun Qi Li Wang | Tel: Email: | + qixingchun@hikvision.com wangli7@hikvision.com |
| Source: | Hangzhou Hikvision Digital Technology Co. Ltd. |  |  |

_____________________________

- Abstract

This contribution is a cross-check report of JVET-K0122[1]. It is confirmed that the cross-check results are identical with the proponent’s results.

- Introduction

  - Multiplication-based DC mode without division

The proposed mechanism requires only one multiplication per block, some other elementary arithmetic operations and a lookup table (LUT). Multiplication operation and the LUT are not required for square blocks that can be important for the smallest blocks (4x4 for luma and 2x2 for chroma) to avoid such operations as multiplication for the worst case when a picture or its part is subdivided into the smallest blocks. The LUT size (in records) is computed as follows:

Fig. 1 Reference samples used for calculating DC value.

Each record comprises a pair , since the value of  can be different for different values of  to maximize the accuracy of calculations. Here,  is an aspect ratio of a block.

In this technique DC value is calculated as follows:

,

,

,

are width and height (respectively) of the predicted block.

  - DC mode using reference samples only along with a longer side

This technique is close to the one being studied in CE3.1.2.1 [2]. The difference with this technique is that only longest side component is considered when calculating DC value. Specifically:

,

if  and

,

if .

Denotations used here are the same as used previously:,  are width and height (respectively) of the predicted block.

Fig. 2 Reference samples used for calculating DC value.

- Results

Simulations are conducted using two configurations: Test Mode (VTM) and Benchmark Set (BMS) configurations, as described in JVET-J1010 [3].

## Table 1. Experimental results for multiplication-based DC mode without division (CTC)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.01% | 100% | 100% | 0.00% | 0.09% | -0.07% | 97% | 101% |
| Class A2 | 0.00% | 0.01% | -0.01% | 100% | 100% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | -0.03% | 0.04% | 100% | 100% | -0.01% | 0.06% | 0.11% | 98% | 100% |
| Class C | 0.00% | -0.06% | -0.07% | 100% | 101% | 0.00% | 0.01% | 0.08% | 98% | 100% |
| Class E | 0.00% | -0.08% | -0.15% | 100% | 97% | -0.01% | 0.00% | -0.11% | 97% | 101% |
| Overall | 0.00% | -0.03% | -0.03% | 100% | 100% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | 0.05% | 0.06% | 99% | 99% | 0.01% | 0.00% | 0.00% | 97% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.06% | -0.06% | 101% | 108% | 0.02% | 0.11% | 0.02% | 100% | 104% |
| Class A2 | -0.01% | -0.02% | -0.02% | 101% | 107% | 0.02% | 0.01% | 0.05% | 100% | 104% |
| Class B | 0.01% | 0.01% | 0.09% | 100% | 100% | 0.01% | -0.03% | -0.19% | 100% | 106% |
| Class C | -0.03% | -0.07% | -0.10% | 100% | 100% | 0.05% | 0.18% | -0.14% | 100% | 104% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.03% | -0.01% | 100% | 103% | 0.03% | 0.06% | -0.09% | 100% | 105% |
| Class D | 0.01% | -0.02% | -0.10% | 100% | 100% | 0.05% | -0.02% | -0.18% | 100% | 101% |

## Table 2. Experimental results for DC mode using reference samples only along with a longer side (CTC)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | -0.01% | 100% | 100% | 0.01% | 0.02% | -0.08% | 97% | 100% |
| Class A2 | -0.02% | -0.03% | 0.00% | 100% | 100% | 0.00% | -0.03% | -0.02% | 97% | 105% |
| Class B | -0.02% | -0.03% | 0.00% | 100% | 100% | -0.01% | 0.06% | 0.02% | 98% | 101% |
| Class C | -0.03% | -0.13% | -0.09% | 100% | 101% | -0.05% | -0.04% | -0.05% | 98% | 101% |
| Class E | -0.02% | 0.03% | -0.06% | 99% | 97% | -0.03% | 0.00% | -0.13% | 98% | 101% |
| Overall | -0.02% | -0.03% | -0.03% | 100% | 100% | -0.02% | 0.01% | -0.05% | 98% | 102% |
| Class D | -0.03% | 0.05% | -0.08% | 100% | 107% | -0.04% | 0.02% | 0.05% | 98% | 101% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | 0.08% | 100% | 100% | 0.02% | 0.06% | 0.09% | 100% | 105% |
| Class A2 | 0.00% | 0.09% | 0.08% | 100% | 100% | 0.03% | -0.03% | 0.01% | 100% | 105% |
| Class B | -0.01% | 0.07% | -0.15% | 100% | 100% | 0.00% | 0.01% | 0.03% | 100% | 106% |
| Class C | -0.02% | 0.00% | -0.09% | 100% | 100% | 0.05% | 0.10% | -0.13% | 99% | 104% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.04% | -0.04% | 100% | 100% | 0.02% | 0.04% | -0.01% | 100% | 105% |
| Class D | 0.03% | -0.25% | -0.25% | 100% | 102% | 0.02% | -0.05% | 0.13% | 99% | 101% |

- Conclusion

The results obtained in the cross-check match the results presented in JVET-K0122, except one sequence (DaylightRoad2) is still running.

On random access condition, sequence is processed in parallel, so because of the floating-point accuracy, it seems that there is 0.01 percent error.

- Reference

- Alexey Filippov, “CE3-related: Alternative techniques for DC mode without division”, JVET-K0122 JVET 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” April 2018.s

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, JVET 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- Patent rights declaration(s)

Hikvision does not have any current or pending patent rights relating to the technology described in this contribution.