| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0492-v2 |
| --- | --- |

| Title: | Cross-check of JVET-K0369: CE2-related: Longer Tap Deblocking Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Gisquet, J. Taquet | Tel: Email: | christophe.gisquet@crf.canon.fr |
| Source: | Canon Research France |  |  |

_____________________________

# Abstract

This document reports the crosscheck results and technical analysis for input document “JVET- K0369: CE2-related: Longer Tap Deblocking Filter” [1]. The cross-checked contribution reuses some bugfixes, adds new longer in-loop filters with constrain on the filter length on the horizontal border of CTBs to not increase

# Analysis

# Initial comment

There are several bugfixes with normative impact. The corresponding code is identical to CE2-2.1.1 proposal by Ericsson [2]: the JVET-K0369 authors observed similar issues and decided to use code from the above CE2 proposal. The bugfix concerning the implicit TU split was also present in the CE2-2.1.3 proposal by MediaTek [3], as well as a standalone contribution on this bugfix [4].

# Bugfixes

There are three main active bugfixes, the first two likely having normative impact:

- Deblocking of implicitly-split CUs (due to size compared to maximal transform size) under the macro IMPLICIT_TU_DB;

- Restriction on number of filter taps, under macro NON_RECURSIVE_DB, to ensure parallelization and avoid undefined behaviour when filter supports overlap, a problem already known and present in JEM since QTBT addition;

- Deblocking of areas with different motion within a CU (likely due to affine or merge modes such as ATMVP), under macro MV_SUBBLOCK_DB;

- Run block strength only once per direction, under macro IMAGE_EDGE_DB.

It was confirmed that macro IMAGE_EDGE_DB does not change the encoding results.

Another bugfix is present but not active. The JVET-K0369 authors believe the related proposal to be a non-obvious bug requiring discussion. This bugfix is:

- Deblocking of areas with different motion within a CU (likely due to affine or merge modes such as ATMVP), under macro MV_SUBBLOCK_DB.

# Technical proposal on long filters

The implementation matches the description of the proposal. As for several proposals in category 2.1 of CE2, the base addition is longer filters (up to 15 taps, but often shorter), selected under conditions of block size and strong filter conditions. The main features compared to other proposals is the restriction on the causal “P” side of the number of taps and filtered samples, so as to keep the same line buffering requirement as HEVC deblocking filter.

# Results

The rate-distortion points provided by the proponents are matched for the points available to both our cross-check results and the proponents’. Runtime is inaccurate because of the use of an inhomogeneous set of computers.

# Simulation environment

The tests were performed on top of BMS 1.0, using either VTM or BMS configuration files. As an inhomogeneous set of CPUs was used, the SIMD level was either SSE42 or AVX2, and the runtime is indeed inhomogeneous. The cross-check authors believe that the JVET-K0369 proposal however should have little impact, if at all, on runtime.

Spot checking tests were performed in both VTM and BMS configurations.

# Over anchors

# Visual evaluation

Some sequences exhibit disturbing artifacts due to at least the implicit TU bug. It seems ill-advised to conduct such visual evaluation without the anchor and the tested proposal both including some sort of remedy. The proposal in JVET-K0369 does provide better visual quality than the VTM or BMS anchors.

# Conclusion

The results obtained in the crosscheck match the results presented in JVET-K0369 for the available points, and decode correctly.

# Reference

- A.M. Kotra, B. Wang, S. Esenlik, Z. Zhao, J. Chen, “CE2-related: Longer Tap Deblocking Filter” JVET-K0369, Jul. 2018, Ljubljana, SI.

- K. Andersson, Z. Zhang, R. Sjöberg, “CE2-2.1.1: Long deblocking filters and fixes” ” JVET-K0307, Jul. 2018, Ljubljana, SI.

- C.-M. Tsai, T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE2.2.1.3: Long deblocking filters” JVET-K0232, Jul. 2018, Ljubljana, SI.

- C.-M. Tsai, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE2-related: Bugfix for deblocking at maximum transform block boundaries” JVET-K0237, Jul. 2018, Ljubljana, SI.

# Patent rights declaration(s)

Canon Research France does not have any current or pending patent rights relating to the technology described in this contribution.