| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0496-v1 |
| --- | --- |

| Title: | Crosscheck report of JVET-K0154 on encoding distortion evaluation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | +81-3-5494-3353 iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution provides crosscheck results of JVET-K0154 on encoding distortion evaluation of VTM/BMS software. The results matched with those provided by the proponent.

Test results

Test results of VTM configuration for AI and RA cases are shown in Table 1. All the BD-rate results are matched with those provided by the proponent. For encoding/decoding complexity, similar tendency to the provided results by the proponent is observed. Test results of VTM configuration for RA cases are shown in Table 1. Note that for BMS case, encoding/decoding runtime for the anchor is not available, so that only BD-rate matching is confirmed.

Table 1. VTM configuration

|  |  | All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over BMS-1.1 WCG_EXT=1 (VTM) |  |  |  |  |  |
|  |  | BD-rate |  |  |  |  |  |
|  |  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| HDR-A | DayStreet |  |  |  | 0.0% | 0.0% | -0.3% |
|  | FlyingBirds |  |  |  | 0.0% | 0.0% | -0.4% |
|  | PeopleInShoppingCenter |  |  |  | 0.0% | 0.0% | -0.1% |
|  | SunsetBeach |  |  |  | 0.0% | 0.0% | -0.2% |
| HDR-B | Market | 0.0% | -3.3% | -8.6% | 0.0% | -3.0% | -6.9% |
|  | SunRise | 0.2% | -8.0% | -19.7% | 0.3% | -7.4% | -18.8% |
|  | ShowGirl2 | 0.0% | -4.3% | -9.9% | -0.1% | -4.4% | -8.8% |
|  | BalloonFestival | -0.1% | -1.5% | -4.1% | 0.0% | -1.2% | -3.4% |
|  | Hurdles | 0.0% | -2.6% | -3.9% | 0.1% | -2.4% | -3.8% |
|  | Starting | 0.0% | -2.3% | -3.9% | 0.1% | -1.6% | -3.1% |
|  | Cosmos1 | 0.0% | -0.8% | -6.6% | 0.0% | -0.7% | -6.4% |
|  | Average HDR-A |  |  |  | 0.0% | 0.0% | -0.2% |
|  | Average HDR-B | 0.0% | -3.3% | -8.1% | 0.1% | -2.9% | -7.3% |
|  | Average all |  |  |  | 0.0% | -1.9% | -4.7% |
|  | Enc Time[%] | 101% |  |  |  |  |  |
|  | Dec Time[%] | 104% |  |  |  |  |  |
|  |  | Random Access |  |  |  |  |  |
|  |  | Over BMS-1.1 WCG_EXT=1 (VTM) |  |  |  |  |  |
|  |  | BD-rate |  |  |  |  |  |
|  |  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| HDR-A | DayStreet |  |  |  | 0.0% | -0.3% | -0.8% |
|  | FlyingBirds |  |  |  | 0.0% | 0.1% | -0.1% |
|  | PeopleInShoppingCenter |  |  |  | 0.0% | 0.0% | -0.2% |
|  | SunsetBeach |  |  |  | 0.0% | 0.0% | -0.3% |
| HDR-B | Market | 0.1% | -5.0% | -11.0% | 0.0% | -4.4% | -8.9% |
|  | SunRise | 0.3% | -11.8% | -26.5% | 0.3% | -10.6% | -23.3% |
|  | ShowGirl2 | -0.1% | -11.4% | -16.8% | -0.2% | -12.0% | -18.1% |
|  | BalloonFestival | -0.1% | -2.9% | -5.7% | 0.0% | -2.3% | -4.8% |
|  | Hurdles | 0.0% | -3.2% | -4.4% | 0.1% | -2.7% | -4.3% |
|  | Starting | 0.0% | -3.7% | -5.7% | 0.1% | -2.4% | -4.6% |
|  | Cosmos1 | -0.1% | -1.7% | -9.9% | -0.1% | -1.3% | -9.0% |
|  | Average HDR-A |  |  |  | 0.0% | -0.1% | -0.3% |
|  | Average HDR-B | 0.0% | -5.7% | -11.4% | 0.0% | -5.1% | -10.4% |
|  | Average all |  |  |  | 0.0% | -3.3% | -6.8% |
|  | Enc Time[%] | 99% |  |  |  |  |  |
|  | Dec Time[%] | 100% |  |  |  |  |  |

Table 2. BMS configuration

|  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over BMS-1.1 WCG_EXT=1 (BMS) |  |  |  |  |  |
|  |  | BD-rate |  |  |  |  |  |
|  |  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| HDR-A | DayStreet |  |  |  | 0.0% | -0.3% | -0.8% |
|  | FlyingBirds |  |  |  | 0.0% | 0.3% | -1.1% |
|  | PeopleInShoppingCenter |  |  |  | 0.0% | 0.1% | -0.2% |
|  | SunsetBeach |  |  |  | 0.0% | 0.1% | -0.6% |
| HDR-B | Market | 0.0% | -7.1% | -13.4% | 0.1% | -6.1% | -11.4% |
|  | SunRise | 0.2% | -19.3% | -36.8% | 0.4% | -15.8% | -32.5% |
|  | ShowGirl2 | -0.1% | -16.4% | -28.1% | -0.1% | -17.8% | -30.1% |
|  | BalloonFestival | -0.2% | -4.4% | -9.5% | -0.1% | -3.3% | -7.7% |
|  | Hurdles | 0.0% | -5.8% | -8.9% | 0.0% | -4.8% | -8.0% |
|  | Starting | -0.1% | -5.6% | -8.8% | 0.0% | -3.6% | -6.8% |
|  | Cosmos1 | -0.1% | -3.5% | -17.3% | -0.1% | -2.9% | -15.6% |
|  | Average HDR-A |  |  |  | 0.0% | 0.1% | -0.7% |
|  | Average HDR-B | 0.0% | -8.9% | -17.5% | 0.0% | -7.7% | -16.0% |
|  | Average all |  |  |  | 0.0% | -4.9% | -10.4% |
|  | Enc Time[%] | - |  |  |  |  |  |
|  | Dec Time[%] | - |  |  |  |  |  |

Patent rights declaration(s)

NHK does not have any current or pending patent rights relating to the technology described in this contribution.