| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0497 |
| --- | --- |

| Title: | CE1.4 related : Evidence of Split Unit Coding Order |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yinji Piao Jie Chen Chanyul Kim | Tel: Email: | [+82-10-6296-4261] [yj1003.piao@samsung.com] |
| Source: | Samsung Electronics Co.,Ltd |  |  |

_____________________________

# Abstract

This contribution presents the evidence of potential gain of split unit coding order (SUCO) described in CE1[1] subtest 1.4. Since the result of SUCO on VTM in JVET-K0133[3] is not consistent with results on other S/Ws, the evidence of potential gain from SUCO is presented in this contribution for further study of SUCO on VTM. SUCO on HM provides 2.1% and 2.1% BD-rate gains in AI and RA, respectively. SUCO on JEM3.1 provides 2.9% gain in class A2 in RA configuration. On the IFVC s/w of Cfp response JVET-J0024, maximum 1.7% gain is observed with less complexity than those in other s/w because of encoder optimization.

# Introduction

SUCO allows referring to not only left but also right reconstructed pixels and right coded information at leaf node(CU) by supporting more flexible coding order at split unit. A split unit(SU) which is further partitioned into quad-tree or vertically partitioned into Bi- or Triple- tree can be either coded from left to right(L2R) or right to left (R2L) by a flag indicating the coding order. CU-level intra prediction and inter prediction tools which utilize left information previously need to be extended accordingly to the right for more efficient coding based on the availability of neighborhood.

SUCO was first introduced in Cfp response of JVET-J0024[2]. SUCO on VTM was reported in the CE1 subtest4 with description in JVET-K0133[3]. Due to limited time and different platform for implementation, the results of SUCO on VTM is not consistent with results achieved in other s/w. There is still harmonization problem with encoder optimization of VTM s/w which should be solved in further study of SUCO in CE. To show the potential gain from SUCO, results of SUCO on other s/w are presented as evidence.

# Results

SUCO was implemented on top of HM12.1, JEM3.1 and IFVC s/w of JVET-J0024. Tests were conducted under common test condition of each s/w. The applied condition of SUCO in those tests are not exactly same because of different partitioning structures of three S/Ws, but the results to be listed are all in maximum complexity configuration.

Table I. SUCO on HM12.1

Table II. SUCO on JEM3.1

Figure. SUCO on IFVC

# Conclusions

This document presents evidence of potential gain of SUCO for further study in VTM. Due to different s/w platform, block partitioning and encoder optimization from the s/w of CfP response [1], SUCO is not properly implemented and optimized in encoder side in terms of performance and complexity, especially in inter slice. We suggest further study of SUCO for potential improvement in inter slice and complexity reduction of encoder. S/W with SUCO can be found in CE1 repository [4].

# Reference

- JVET, “Description of Core Experiment 1 (CE1): Partitioning”, JVET-J1021, 10th JVET Meeting, San Diego, US, 10-20 Apr., 2018

- JVET, “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario”, JVET-J0024, 10th JVET Meeting, San Diego, US, 10-20 Apr., 2018

- JVET, “CE1.4 : Split Unit Coding Order”, JVET-K0133, 11th JVET Meeting, Ljubljana, SL, 10-20 July., 2018

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE1/CE1-4.0.2/

# Patent rights declaration(s)

Samsung Electronics Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).