| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0505-v2 |
| --- | --- |

| Title: | Cross-check of JVET-K0093 (CE9-Related : Restricted template matching schemes to mitigate pipeline delay) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexander Karabutov, Sergey Ikonin, Roman Chernyak | Tel: Email: | +7-495-660-44-59 karabutov.alexander@huawei.com sergey.ikonin@huawei.com chernyak.roman@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

This contribution is a cross-check report of JVET-K0093 on Restricted template matching schemes to mitigate pipeline delay. Tool’s description and simulation results provided by the proponent were verified.

# Introduction

## Brief description of the tool

Proposal JVET-K0093 [1] have two parts:

- restriction of template region,

- restriction of reordering of merge candidate list.

First part mitigates issue with pipeline delay on template matching over the blocks that was not reconstructed yet. The solution consists in choosing only one region (above or left) for template matching. In other words, template region belonging to the previous block is not considered to calculate the cost.

Second part is an restriction version of reordering mechanism of merge candidate list in [2]. To reduce the memory bandwidth issues it is proposed to divide merge candidate to two groups and reorder only one of them. Index of the group indicated by merge_index.

## Tests

According to [1], following three tests were performed:

- Restricted template region

- Restricted reordering of merge candidate list

- Harmonization of the above two tests

Test #1 and Test #2 evaluates first and second parts separately. Test #3 includes both parts.

## Source code set up

The corresponding branch with source code of [1] includes following defines for control behaviour of proposed tool:

#define RESTRICTED_TEMPLATE_REGION                        1

#define CAND_GROUPING                                       1

Configuration per tests:

| Test # | Parameters name | Parameters value |
| --- | --- | --- |
| 1 | RESTRICTED_TEMPLATE_REGION CAND_GROUPING | 1 0 |
| 2 | RESTRICTED_TEMPLATE_REGION CAND_GROUPING | 0 1 |
| 3 | RESTRICTED_TEMPLATE_REGION CAND_GROUPING | 1 1 |

# Simulation results

## Test #1: Restricted template region

## Test #2: Restricted reordering of merge candidate list

## Test #3: Harmonization of the above two tests

# Conclusion

The results received within this cross-check are the same with provided by proponent.

# References

- N. Park, J. Nam, H. Jang, J. Lee, J. Lim, S. Kim, “CE9-Related : Restricted template matching schemes to mitigate pipeline delay”, JVET-K0093, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 11th Meeting: Ljubljana, SL, 10-20 July 2018

- M. Koo, J. Heo, J. Nam, N. Park, J. Lee, J. Choi, S. Yoo, H. Jang, L. Li, J. Lim, S. Paluri, M. Salehifar, S. Kim, “Description of SDR video coding technology proposal by LG Electronics”, JVET-J0017, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 10th Meeting: San Diego, CA, 10–20 April 2018.