| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0507-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-K0103 (CE4-related: Simplified Affine Prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution is a crosscheck report of JVET-K0103 (CE4-related: Simplified Affine Prediction). The simulations were carried out based on the common test condition (CTC). Based on the partial finished results, the test results are matched.

Introduction

In contribution JVET-K0103, affine prediction in the benchmark set (BMS) is modified in three aspects. First, the sub-block size is fixed to be 4×4, instead of being calculated by an equation with division operations. Second, the constraints of block size for affine merge mode and affine AMVP mode are unified. Both affine merge mode and the affine AMVP mode are applicable when the width and height of a block is greater than or equal to 8. Third, the sub-block size for chroma components is expanded from 2×2 to 4×4. Simulation results reportedly show 0.03% BD-rate increase on the Y component for Random Access (RA) configuration in average compared with BMS-1.1.

Simulation results

Table 1: Test 1 compared with BMS-1.1 (RA)

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.06% | -0.02% | 100% | 100% |
| Class A2 | 0.00% | 0.04% | -0.08% | 100% | 101% |
| Class B | 0.00% | -0.04% | -0.04% | 100% | 100% |
| Class C | 0.00% | 0.00% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.05% | 100% | 100% |

Table 2: Test 1 compared with BMS-1.1 with affine only (RA)

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | 0.03% | 102% | 99% |
| Class A2 | -0.01% | -0.02% | 0.03% | 102% | 100% |
| Class B | -0.01% | -0.04% | 0.00% | 102% | 101% |
| Class C | -0.01% | -0.01% | -0.01% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.03% | 0.01% | 102% | 101% |

Table 3: Test 2 compared with BMS-1.1 (RA)

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | -0.07% | 102% | 100% |
| Class A2 | 0.02% | 0.04% | 0.01% | 102% | 101% |
| Class B | 0.01% | 0.00% | -0.06% | 101% | 100% |
| Class C | 0.02% | -0.10% | 0.02% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.01% | -0.02% | 101% | 100% |

Table 4: Test 2 compared with BMS-1.1 with affine only (RA)

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | 0.06% | -0.05% | 0.01% | 102% | 99% |
| Class B | 0.00% | -0.06% | 0.01% | 102% | 101% |
| Class C | 0.02% | 0.04% | 0.07% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.02% | 0.02% | 102% | 101% |

Table 5: Test 3 compared with BMS-1.1 (RA)

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.12% | 0.02% | 101% | 100% |
| Class A2 | 0.07% | 0.41% | 0.39% | 101% | 99% |
| Class B | 0.01% | 0.18% | 0.25% | 101% | 99% |
| Class C | 0.04% | 0.10% | 0.09% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.14% | 0.19% | 101% | 99% |

Table 6: Test 3 compared with BMS-1.1 with affine only (RA)

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.09% | 0.09% | 100% | 96% |
| Class A2 | 0.09% | 0.52% | 0.46% | 101% | 92% |
| Class B | 0.03% | 0.13% | 0.37% | 101% | 96% |
| Class C | 0.03% | 0.11% | 0.16% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.20% | 0.28% | 101% | 96% |

References

- K. Zhang, L. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong, “CE4-related: Simplified Affine Prediction”, JVET-K0103, July 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, April 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd does not have any current or pending patent rights relating to the technology described in this contribution.