| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0508-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-K0052 (Non-CE4: A study on the affine merge mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution is a crosscheck report of JVET-K0052 (Non-CE4: A study on the affine merge mode). The simulations were carried out based on the common test condition (CTC). Only test 5 specified in JVET-K0052 was crosschecked by simulation. The test results are matched for all points.

Introduction

The contribution JVET-K0052 studied line buffer usage of the affine merge mode in BMS1.0 and advocated the following three changes to simplify the design, namely, 1) directly using 4-parameter affine motion model to derive the seed vectors for the current PU of the affine merge mode; 2) disabling the affine merge mode for PUs whose width is less than 8 to enable motion data line buffer sharing, and 3) having the affine (merge) mode and the regular merge/skip and AMVP mode share the same motion data line buffer. For 4K video, the proposed changes reduce the line buffer size of the affine merge mode from roughly 18,688 bytes to 320 bytes, without comprising compression efficiency.  Relative to the BMS1.0 anchor, the overall BD-rate changes are 0.03% in RA, -0.07% in LD-B and -0.03% in LD-P, respectively.  Compared to the BMS1.0 VTM configurations but with “Affine” and “HighPrecMv” on, the overall BD-rate changes are 0.00% in RA, 0.02% in LD-B and 0.06% in LD-P, respectively. It is recommended to study the simplification of the affine (merge) mode to make it more implementation friendly.

Simulation results

Table 1. Test5 compared to BMS1.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.16% | -0.01% | 102% | 100% |
| Class A2 | 0.03% | -0.01% | -0.02% | 102% | 100% |
| Class B | 0.03% | 0.03% | 0.02% | 102% | 100% |
| Class C | 0.03% | 0.05% | -0.02% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.01% | -0.01% | 102% | 101% |
| Class D | 0.04% | 0.11% | -0.06% | 101% | 102% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low Delay B Main 10 |  |  |  |  |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.33% | 0.04% | 101% | 99% |
| Class C | 0.02% | 0.10% | -0.06% | 101% | 101% |
| Class E | -0.27% | 0.05% | 0.11% | 101% | 100% |
| Overall | -0.07% | -0.09% | 0.03% | 101% | 100% |
| Class D | 0.03% | 0.10% | -0.02% | 100% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

References

- M. Zhou, B. Heng, “Non-CE4: A study on the affine merge mode”, JVET-K0052, July 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, April 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd does not have any current or pending patent rights relating to the technology described in this contribution.