| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0514 |
| --- | --- |

| Title: | CE4-related: Encoder optimization on top of CE 4.2.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu Han, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +1-858-845-1795 yuhan@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution describes an encoder optimization algorithm on top of CE4 test 4.2.3.b which adds non-adjacent spatial merge candidate candidates and enlarge merge candidate list size to 10. For the “Tool on case 1 (VTM as reference)” tests in RA case, this contribution achieved objective gain of 1.12% in terms of average luma BD-rate improvement.

# Technical description

The fast merge algorithm used in VTM-1 and BMS-1 is inherited from JEM. If it is the best mode of the current PU is skip mode in the previous check, it will not check all of the merge candidates in order to save time. Because the merge candidates for the current PU may change, and skip mode would not impact encoding time much, this proposed method suggests checking all of the available merge candidates. The pseudocode is as below: the modified code is highlighted.

if( !bestIsSkip )

{

…

}

else

{

uiNumMrgSATDCand = mergeCtx.numValidMergeCand;

}

# Simulation results

As descripted in CE4 [2], the test specification is as below, and the “Tool on case 1 (VTM as reference)” and “Tool off case 1 (BMS as reference)” with default encoder configuration are used as anchors.

## Proposed method on top of CE4 test 4.2.3.b

Merge candidates list is as below: max number of merge candidates is 11 for BMS test and 10 for VTM test. (new candidates are highlight with underline)

- Spatial candidates for blocks 1-4.

- ATMVP (BMS test).

- Spatial candidate (block 5) (used only when the number of the available candidates is smaller than 6 (or 4 for VTM test)).

- Temporal candidate (derived as in HEVC).

- Non-adjacent spatial candidate (blocks 6 to 49).

- Combined candidates.

- Zero candidates.

Simulation results as below:

## Test4.2.3.b Non-adjacent spatial merge candidate candidates and enlarge merge candidate list size to 10

Merge candidates list is as below: max number of merge candidates is 11 for BMS test and 10 for VTM test. (new candidates are highlight with underline)

- Spatial candidates for blocks 1-4.

- ATMVP (BMS test).

- Spatial candidate (block 5) (used only when the number of the available candidates is smaller than 6 (or 4 for VTM test)).

- Temporal candidate (derived as in HEVC).

- Non-adjacent spatial candidate (blocks 6 to 49).

- Combined candidates.

- Zero candidates.

Simulation results as below:

# Conclusion

This contribution presents improvement on Merge/Skip prediction technologies on top of CE4 test 4.2.3.b. For the “Tool on case 1 (VTM as reference)” tests in RA case, this contribution achieved objective gain of 1.12% in terms of average luma BD-rate improvement.

# References

- JVET-J0021, “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions”, Macau, Apr. 2018.

- H. Yang, S. Liu, K. Zhang, JVET-J1024, “Description of CE4: Inter prediction and motion vector coding”, Apr. 2018.

- J. Chen, E. Alshina, G. Sullivan, J. Ohm, J. Boyce, JVET-G1001, “Algorithm description of Joint Exploration Test Model 7”, July 2017.

- B. Bross, JVET-J1001, “Versatile video coding”, Apr. 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, JVET-J1010, “JVET common test conditions and software reference configurations”, Apr. 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).