| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0518-v1 |
| --- | --- |

| Title: | CE3-related: Interpolation filtering for intra-prediction within rectangular blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy Jianle Chen | Tel: Email: | +7-925-008-23-08 alexey.filippov@huawei.com vasily.rufitskiy@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

In CE3 [1], different intra-interpolation filters were considered to improve the coding efficiency of intra prediction for skew directional modes. In this contribution, we propose an alternative mechanism of selecting interpolation filters. On top of VTM, the simulation results for this technique reportedly show - 0.60%, - 0.44% and - 0.52% BD-rate savings on Y, Cb and Cr components respectively for All Intra (AI) configuration with 105% encoding time and appr. 100% decoding time on average.

# Introduction

In CE3 [1], different intra-interpolation filters were considered to improve the coding efficiency of intra prediction for skew directional modes. The combination of 4-tap Cubic and Gaussian interpolation filters adopted into the JEM software is size-dependent. Initially, the filters were switched by using the width / height of square blocks. After enabling rectangular blocks by introducing the QTBT frameworks, the switching mechanism was modified and become size- and mode-dependent. In JEM-7.2, switching between the filters is performed using both directional mode index and the size of a block as shown in Figure 1. This contribution presents an alternative mechanism of selecting interpolation filters.

Figure 1. Switching between intra interpolation filters in JEM-7.2

# Proposed techniques

The proposed mechanism makes a decision on what interpolation filters should be selected by using directional intra prediction mode index as well as subject to aspect ratio, width and height of a given block. For doing it, the index of a selected directional intra prediction mode is compared with a threshold value that is fetched from a LUT and depends on shape and size of a given block. Particular examples of interpolation filter switch are shown in Figure 2. It could be noticed that prediction from the shorter side uses 4-tap Cubic filter and 4-tap Gaussian filter is applied for the longer side.

Figure 2. Examples of interpolation filter switching

# Experimental Results

The proposed technique is implemented on top of VTM-1.0. The experimental results of All Intra and Random Access configurations are summarized in the Tables 1-2 respectively. The simulations were performed in accordance with the JVET CTC [2].

More detail results can be found in the accompanying Excel tables.

## Table 1. Experimental results on top of VTM-1.0 for AI configuration (CTC)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.48% | -0.44% | -0.64% | 105% | 101% |
| Class A2 | -0.27% | -0.16% | -0.21% | 105% | 100% |
| Class B | -0.50% | -0.33% | -0.43% | 105% | 98% |
| Class C | -0.90% | -0.94% | -1.12% | 104% | 99% |
| Class E | -0.79% | -0.22% | -0.04% | 106% | 99% |
| Overall | -0.60% | -0.44% | -0.52% | 105% | 99% |
| Class D | -0.85% | -0.82% | -0.81% | 104% | 98% |

## Table 2. Experimental results on top of VTM-1.0 for RA configuration (CTC)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.50% | -0.22% | 100% | 100% |
| Class A2 | -0.17% | -0.60% | -0.57% | 99% | 99% |
| Class B | -0.31% | -0.48% | -0.82% | 99% | 99% |
| Class C | -0.38% | -0.69% | -0.38% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.56% | -0.53% | 99% | 99% |
| Class D | -0.42% | -1.16% | -1.29% | 100% | 99% |

# References

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, JVET-B1010, “JVET common test conditions and software reference configurations”, April 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).