| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0520 |
| --- | --- |

| Title: | Cross-check of JVET-K0352 (CE4-related: Encoder optimization on top of CE 4.2.13) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | X. Chen | Email: | anderson.chen@hisilicon.com |
| Source: | HiSilicon Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This document reports the cross-check results for the results reported in input document JVET-K0352: CE4-related: Encoder optimization on top of CE 4.2.13. It is reported that the description provided in the JVET-K0352 appears to match with the implementation. Similarly, simulation results appear to match with the ones provided in JVET-K0352.

Introduction

The tests were performed in VTM (BMS in VTM configuration) with RAconfigurations and LDBconfiguration.

Results

VTM-RA-results

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.92% | -0.92% | -1.11% | 106% | 105% |
| Class A2 | -1.15% | -1.39% | -1.48% | 108% | 108% |
| Class B | -1.50% | -1.23% | -1.10% | 108% | 106% |
| Class C | -0.98% | -1.03% | -1.11% | 111% | 104% |
| Class E |  |  |  |  |  |
| Overall | -1.18% | -1.15% | -1.18% | 108% | 105% |
| Class D | -0.68% | -0.80% | -0.90% | 113% | 100% |

VTM - LDB-results

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.22% | -0.78% | -0.62% | 109% | 108% |
| Class C | -0.61% | -0.69% | -0.63% | 112% | 105% |
| Class E | -0.35% | 0.03% | -0.16% | 108% | 106% |
| Overall | -0.80% | -0.55% | -0.51% | 110% | 107% |
| Class D | -0.46% | -0.67% | -0.83% | 113% | 110% |

Conclusion

The results obtained in the cross-check match theresults presented in JVET-K0352.

Patent rights declaration(s)

HiSilicon may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).