| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0521-v1 |
| --- | --- |

| Title: | BoG Report on ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Li Zhang | Tel: Email: | 1-858-999-7093 lizhang.idm@bytedance.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG has two mandates:

- Assess the implementation complexity (memory accesses, memory for storing predefined filters, operations per sample in terms of mul, add, comp, reload operations, etc.) of the CTU based approach and different aspects of the simplified classification based approaches. This should gives us data to assess the complexity aspect versus the performance.

- Clarification is the signalling of the filter coefficients. The current BMS-GALF solution of signalling between slice header and first CTU is not desirable. Also the case of multiple slices per picture should be supported.

The first meeting was held Thursday 12 July from 9:30am to 11:35am.

# Summary table of coding technology

| Test# | Description | Document# |
| --- | --- | --- |
| BMS-GALF |  |  |
| 2.4.1.1.a | Subsampled sum-modified-Laplacian with 4×4 level classification | JVET-K0164 |
| 2.4.1.1.b | Subsampled sum-modified-Laplacian with 2×2 level classification | JVET-K0164 |
| 2.4.1.2 | Adaptive Loop Filter Simplification | JVET-K0327 |
| 2.4.1.3 | ALF with Multiplication Replaced by Bit-Shifting | JVET-K0215 |
| 2.4.1.4.c | luma 7×7, classifier 4×4, chroma 5×5 | JVET-K0371 |
| 2.4.2.2.c | CTB-based ALF with slice filter sets, only GeometricBA | JVET-K0235 |
| 2.4.2.2.e | CTB-based ALF with slice filter sets, single filter is selected at CTB (including merge with neighbor signalling) | JVET-K0235 |

# Complexity assessment

## Summary of comparisons for all proposals

| Test# | memory for storing predefined filters | Operations per luma | Operations per chroma | Frequency change of filters (Y, Cb, Cr) | Line buffer size (L, C) |
| --- | --- | --- | --- | --- | --- |
| BMS | 400filters*20 coeffs*9bits |  |  |  |  |
| 2.4.1.1.a | 400filters*20 coeffs*9bits |  | 13 add; 1 comp; 7 mult; 1 shift | 4x4, slice, slice | 8 (L) 4(C) |
| 2.4.1.1.b | 400filters*20 coeffs*9bits |  | 13 add; 1 comp; 7 mult; 1 shift | 2x2, slice, slice | 8 (L) 4(C) |
| 2.4.1.2 |  |  |  |  |  |
| 2.4.1.3 |  |  |  |  |  |
| 2.4.1.4.c | 128(filters)*12coefficients*9bits |  | 13 add; 1 comp; 7 mult; 1 shift | 4x4, Slice, Slicee | 6 (L) 4(C) |
| 2.4.2.2.c |  |  |  |  | 8(L) 8(C) |
| 2.4.2.2.e |  |  |  |  | 8(L) 8(C) |

## Detailed analysis of complexity assessment

### BMS-GALF

### CE 2.4.1.1 JVET-K0164

Verbally reported that the calculation of Laplacians for 2.4.1.1.a:

  - Adds: 4blocks * 4directions * 5  = 80

  - Comp: 4blocks * 4directions * 2   = 32

Q: Update the data of using 8x8 block for classification calculations and filter operations based on 9×9 diamond filter shape.

### CE 2.4.1.2 JVET-K0327

Slides presented.

Q: Update the data of using 8x8 block for classification calculations and filter operations based on 9×9 diamond filter shape.

### CE 2.4.1.3 JVET-K0215

No discussion.

### CE 2.4.1.4.c JVET-K0371

Slides presented.

Classification (to be updated):

- 4x4 block with 2x2 Laplacians (reused):

Adds: 4 blocks*4directions*11 = 176;

Comp: 4blocks*4directions*4 = 64

- Activity calculation:

Sum of 16 2x2 sub-block Laplacians:

adds: 2 lower rows*3 adds/row/dir*4direction)+(3adds/dir*4direction)= 36

- direction decision: 3 comp, 2 mults

- average variance: 2 comps, 2shifts:

- Decision of second direction: HV_D_HIGH *2 > 9*HV_D_LOW: 1 COMP, 2 SHIFTS, 2 ADD

Filter (7×7 diamond shape):

- Geometry-based filtered: 4 comps

- Adds: 2*1 + 11*2 + 1 =25

- Multiplications: 13

- Shift: 1

- Comp: 1

Q: Update the data of using 8x8 block for classification calculations

## CE2.4.2.2 JVET-K0235

Summary of BMS-GALF complexity and CE2.4.2.2.e (as presented in the attached slides) are presented. The following questions are raised:

- Remove the memory required by the center filter coefficient (in GALF)

- Missing the number of condition checking for selecting 4 from 6 neighbors

- CTC uses 128x128 LCU, but LCU could be as small as 16x16, what is the memory required for that?

- Question: what’s the performance of just enabling slice filters and LCU on/off flags (i.e., no NewFilterMode)

- A summary of per-pixel operations

    - Without classification (CE2.4.2.2.e)

    - With GALF classification (CE2.4.2.2.c)

# Signalling of ALF related side information

No discussion in the first BoG meeting.

# Recommendation (if any)