| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0521-v2 |
| --- | --- |

| Title: | BoG Report on ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Li Zhang | Tel: Email: | 1-858-999-7093 lizhang.idm@bytedance.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG has two mandates:

- Assess the implementation complexity (memory accesses, memory for storing predefined filters, operations per sample in terms of mul, add, comp, reload operations, etc.) of the CTU based approach and different aspects of the simplified classification based approaches. This should gives us data to assess the complexity aspect versus the performance.

- Clarification is the signalling of the filter coefficients. The current BMS-GALF solution of signalling between slice header and first CTU is not desirable. Also the case of multiple slices per picture should be supported.

The first meeting was held on Thursday 12 July from 9:30am to 11:35am.

The second meeting was held on Saturday 14 July from 9:25am to 12:12pm.

# Summary table of coding technology

| Test# | Description | Document# |
| --- | --- | --- |
| BMS-GALF |  |  |
| 2.4.1.1.a | Subsampled sum-modified-Laplacian with 4×4 level classification | JVET-K0164 |
| 2.4.1.1.b | Subsampled sum-modified-Laplacian with 2×2 level classification | JVET-K0164 |
| 2.4.1.2 | Adaptive Loop Filter Simplification | JVET-K0327 |
| 2.4.1.3 | ALF with Multiplication Replaced by Bit-Shifting | JVET-K0215 |
| 2.4.1.4.c | luma 7×7, classifier 4×4, chroma 5×5 | JVET-K0371 |
| 2.4.2.2.c | CTB-based ALF with slice filter sets, only GeometricBA | JVET-K0235 |
| 2.4.2.2.e | CTB-based ALF with slice filter sets, single filter is selected at CTB (including merge with neighbor signalling) | JVET-K0235 |

# Complexity assessment

## Summary of comparisons for all proposals

| Test# | memory for storing predefined/merge filters | Operations of classification process per luma sample | Operations of filtering process per luma sample | Operations of classification process per chroma sample |  |  |  | Operations of filtering process per chroma sample |  |  | Frequency change of filters (Y, Cb, Cr) |  |  | Line buffer size (L, C) |  | Notes |  |  | Summary (total operations per luma sample) |  | Summary (total operations per chroma sample) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BMS (wo/ data reuse) |  | 400filters*20 coeffs*9bits =72K bits | Add: 33.25 Comp: 10.75 Mult: 0.5 Shift: 3.5 |  |  |  | Add: 41 Mul: 21 Shift: 1 Comp: 4 |  | N/A |  |  |  |  |  | Add 13; Comp 1; Mult: 7; Shift: 1 |  |  |  | 2x2, slice, slice |  |  |  | 8(L) 4(C) |  |  |  |  |  |  |  | Add: 74.25 Comp: 14.75 Mult:21.5 Shift: 4.5 |  | Add: 13 Comp: 1 Mult 7 Shift:1 |  |  |  |
| 2.4.1.1.a (w/ data reuse) |  | 72K bits | Add: 9.625 Comp: 3.4375 Mult: 0.125 Shift: 1.0625 |  |  |  | Add: 41 Mul: 21 Shift: 1 Comp: 4 |  | N/A |  |  |  |  |  | Add 13; Comp 1; Mult: 7; Shift: 1 |  |  |  | 4x4, slice, slice |  |  |  | 8(L) 4(C) |  |  |  |  |  |  |  | Add: 50.625 Comp: 7.4375 Mul: 21.125 Shift: 2.0625 |  | Add: 13 Comp: 1 Mult 7 Shift:1 |  |  |  |
| 2.4.1.1.a (wo/ data reuse) | 72K bits | Add: 15.125 Comp:4.9375 Mult: 0.125 Shift: 1.4375 | Add: 41 Mul: 21 Shift: 1 Comp: 4 | N/A |  |  |  | Add 13; Comp 1; Mult: 7; Shift: 1 |  |  | 4x4, slice, slice |  |  | 8(L) 4(C) |  |  |  |  | Add: 56.125 Comp: 8.9375 Mul: 21.125 Shift: 2.4375 |  | Add: 13 Comp: 1 Mult 7 Shift:1 |  |  |
| 2.4.1.2 (wo/ data reuse) | 72K bits | Highest temporal layer: Add: 13 Comp: 4 Mult: 0.5 Shift: 1.8125 Other cases: Same as BMS-GALF | Add: 41 Mul: 21 Shift: 1 Comp: 4 | N/A |  |  |  | Add 13; Comp 1; Mult: 7; Shift: 1 |  |  | 2x2, slice, slice |  |  | 8(L) 4(C) |  |  |  |  | Add: 74.25 Comp: 14.75 Mult:21.5 Shift: 4.5 |  | Add: 13 Comp: 1 Mult 7 Shift:1 |  |  |
| 2.4.1.3 (wo/ data reuse) | 0 | Add: 33.25 Comp: 10.75 Mult: 0.5 Shift: 3.5 | Add: 41 Mul: 7 Shift: 14+1=15 Comp: 4 | N/A |  |  |  | Add 13; Comp 1; Mult: 7; Shift: 1 |  |  | 2x2, slice, slice |  |  | 8(L) 4(C) |  |  |  |  | Add: 74.25 Comp: 14.75 Mult: 7.5 Shift: 18.5 |  | Add: 13 Comp: 1 Mult 7 Shift:1 |  |  |
| 2.4.1.4.c (w/ data reuse) | 128filters*12coeffs *9bits =13.824k bits Mapping index of fixed filters to each class: 25*16*7bits | Add: 18.625 Comp: 6.4375 Mul: 0.125 Shift: 1.8125 | Add: 25 Mul: 13 Shift: 1 Comp: 4 | N/A |  |  |  | Add 13; Comp 1; Mult: 7; Shift: 1 |  |  | 4x4, Slice, Slice |  |  | 6 (L) 4(C) |  |  |  |  | Add: 43.625 Comp: 10.4375 Mul: 13.125 Shift: 2.8125 |  | Add: 13 Comp: 1 Mult 7 Shift:1 |  |  |
| 2.4.1.4.c (wo/ date reuse) | 128filters*12coeffs *9bits =13.824k bits Mapping index of fixed filters to each class: 25*16*7bits | Add : 28.625 Comp : 9.4375 Mul : 0.125 Shift : 2.5625 | Add: 25 Mul: 13 Shift: 1 Comp: 4 | N/A |  |  |  | Add 13; Comp 1; Mult: 7; Shift: 1 |  |  | 4x4, Slice, Slice |  |  | 6 (L) 4(C) |  |  |  |  | Add: 53.625 Comp: 13.4375 Mul: 13.125 Shift: 3.5625 |  | Add: 13 Comp: 1 Mult 7 Shift:1 |  |  |
| 2.4.2.2.e | ((NumCTUInRow+4)*3comp) filters*10coeffs*8bits (to be confirmed) | N/A | Add: 21 Mult 11 Shift 1 Comp 1 | N/A |  |  |  | Add: 21 Mult : 11 Shift: 1 Comp: 1 |  |  | CTB, CTB, CTB |  |  | 8(L) 8(C) |  | Filter Predicton: - 33 add; 11 shift; - 120 comparisons per CTB |  |  | Add: 21 Comp: 1 Mult: 11 Shift: shift |  | Add: 21 Mult : 11 Shift: 1 Comp: 1 |  |  |

## Detailed analysis of complexity assessment

### BMS-GALF

### CE 2.4.1.1 JVET-K0164

Verbally reported that the calculation of Laplacians for 2.4.1.1.a:

  - Adds: 4blocks * 4directions * 5  = 80

  - Comp: 4blocks * 4directions * 2   = 32

Q: Update the data of using 8x8 block for classification calculations and filter operations based on 9×9 diamond filter shape.

Updated data are presented in the 2nd BoG.

Slides presented on Saturday.

- Adds: 1586

- Comp: 576

- Shift: 2

### CE 2.4.1.2 JVET-K0327

Slides presented.

Q: Update the data of using 8x8 block for classification calculations and filter operations based on 9×9 diamond filter shape.

Updated slides presented on Saturday.

### CE 2.4.1.3 JVET-K0215

Classification: same as BMS-GALF

### Luma filtering process: 9x9 filter support region used as in BMS. 14 of the 21 multiplier coefficients are replaced with shift.

### Chroma filtering process: same as BMS-GALF.

### Q: how to get the pow of 2 values for filter coefficients

### A: After filter merging process, round the real filter coefficients and solve the equation of filter coefficients derivation one more time to update the remaining filter coefficients. Without this additional step, there is 0.1% loss. Filter coefficients signaling method is changed in terms of binarization process.

### Q: apply it to all filter shapes?

### A: 5x5 no changes; 7x7: (6 filter coefficients are restricted to be pow of 2 values); 9x9: like depicted in the first figure of the slides (14 filter coefficients are restricted to be pow of 2 values)

### CE 2.4.1.4.c JVET-K0371

1st version of slides presented on Thursday.

Classification (to be updated):

- 4x4 block with 2x2 Laplacians (reused):

Adds: 4 blocks*4directions*11 = 176;

Comp: 4blocks*4directions*4 = 64

- Activity calculation:

Sum of 16 2x2 sub-block Laplacians:

adds: 2 lower rows*3 adds/row/dir*4direction)+(3adds/dir*4direction)= 36

- direction decision: 3 comp, 2 mults

- average variance: 2 comps, 2shifts:

- Decision of second direction: HV_D_HIGH *2 > 9*HV_D_LOW: 1 COMP, 2 SHIFTS, 2 ADD

Filter (7×7 diamond shape):

- Geometry-based filtered: 3 comps

- Adds: 2*1 + 11*2 + 1 =25

- Multiplications: 13

- Shift: 1

- Comp: 1

Q: Update the data of using 8x8 block for classification calculations

A: updated in the 2nd BoG meeting

2nd version of slides presented on Saturday.

Classification (8x8 block unit) solution 1 (without memory usage):

- 2x2 Laplacians (no reuse):

  - Adds: 8x8/(2x2) blocks*4directions*11 = 176;

  - Comp: 36 blocks*4directions*4 = 576

  - Shift:  36 blocks * 4 = 144

- How to get summation for each 4x4 block

  - 16 blocks of 2x2 Laplacians (additions): 15 add*4dir*4 blocks=240

- Others:

  - 1 add (adding hor/vert Laplacian); 2 comps; 2 shifts (clipping to quantized activities); 1 comps (to set the main direction)

  - Decision of main and second directions: 1 comp; 2 mults, 1 comp

  - 2 comps; 3 shifts; 2 adds

Classification (8x8 block unit) solution 2 (with memory usage, taking one example):

- Memory required: 128bits

- 2x2 Laplacians

  - Add: 1056; comp: 384; shift: 96

- How to get summation for each 4x4 block

  - (2*3+2) * 4dirs * 4blocks = 128

- Others:

  - 1 add (adding hor/vert Laplacian); 2 comps; 2 shifts (clipping to quantized activities); 1 comps (to set the main direction)

  - Decision of main and second directions: 1 comp; 2 mults, 1 comp

  - 2 comps; 3 shifts; 2 adds

Filter (7×7 diamond shape):

- Geometry-based filtered: 3 comps

- Adds: 2*1 + 11*2 + 1 =25

- Multiplications: 13

- Shift: 1

- Comp: 1

## CE2.4.2.2 JVET-K0235

Summary of BMS-GALF complexity and CE2.4.2.2.e (as presented in the attached slides) are presented. The following questions are raised:

- Remove the memory required by the center filter coefficient (in GALF): done

- Missing the number of condition checking for selecting 4 from 6 neighbors: done

- CTC uses 128x128 LCU, but LCU could be as small as 16x16, what is the memory required for that?

- Question: what’s the performance of just enabling slice filters and LCU on/off flags (i.e., no NewFilterMode)

No results yet.

- A summary of per-pixel operations

    - Without classification (CE2.4.2.2.e): provided

    - With GALF classification (CE2.4.2.2.c): withdrawn

Q: how to select four from 6 neighboring blocks’ filters

A: pruning (with 10 comparisons per pruning) is applied. Worst case, it requires (1+2+3+3+3)=12 times.

Q: memory requirement

A: It depends on depending on the pipeline design.

- If frame pipeline is used, then all CTB filters and on/off control flags per CTB should be kept. Considering the current BMS-GALF, the on/off control flags per CTU are also needed to be stored. For the worst case based on the current CTC, the memory requirement is similar. However, for both cases, memory could be reduced since for BMS-GALF, the chance to select 8x8 level on/off control flags is relatively low without expected coding efficiency decrease. For the proposed method, normative changes (e.g., applying subsampling filters) might cause coding performance drop.

# Signalling of ALF related side information

No discussion in the first BoG meeting.

2nd BoG:

Current implementation:

- CE2.4.2.2.e

  - Slice header: slice filters (with VLC)

  - CTB based filter information: before the first CU data, interleaved with CTU data

- Other contributions: same as BMS-GALF

  - After slice header, before the first CTU

Proposed specification suggestions (for all contributions, agreed):

- Temporal prediction: could be implemented in the decoding process. For the specification, a separate sub-clause to describe the updating and selecting process.

- Filter coefficients:

  - Slice level filters: kept in the slice (picture) header

  - CTU level filters: interleaved with CTU

- Block on/off flags: interleaved with CTU

# Conclusions

# Classification complexity is lower than the filtering process.

# The coding gain (roughly 1.5%) and complexity of classification is a good trade-off.

# The number of multiplications and number of line buffers are two of key complexity aspects in ALF filtering process.

# The filter switching frequency should be in a level equal to or higher than 4x4 block.