| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0527-v1 |
| --- | --- |

| Title: | BoG on 360 Video |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce | Tel: Email: | jill.boyce@intel.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met July 13 in two sessions to review CE13 and other 360 Video related contributions identified i the AHG8 report. Informal subjective viewing was performed, and the observations from the viewing discussed in the BoG.

The BoG recommends the following:

- Add the MCP projection format to 360Lib. Should add configuration options for flexible padding, including number of padding pixels, and which locations to apply padding (just between face rows vs. around the face rows including the picture edges.)

- Use the MCP format with padding of 4 samples around face row with blending (PMCP) as in Test 5.2 for coding tool experiments in the CE and the CTC. Replace the ERP and CMP anchors with MCP. Will still ask AHG6 to provide PERP and CMP results for VTM and BMS.

- Define a 360 video Core Experiment be defined to study removing subjective artifacts at face boundaries, and comparing normative decoding tools with non-normative pre- and post-processing (such as padding, cropping, blending, post-filtering). CE coordinators: P. Hanhart and J.-L. Lin.

  - Include in CE: inter and intra prediction using spherical neighbors, deblocking filter disabling and using spherical neighbors, post-filtering, combinations with padding and with other coding tools. Different padding widths.

The BoG encourages study of the following:

- Signaling of projection format parameters, coding tools parameters, and post-filtering hints.

- GPU complexity impact of rendering (including possible post-processing) of projection formats.

The BoG asks the track for guidance on normative tool adoption for 360 video. What complexity impact could be acceptable to remove subjective artifacts that pre- and post-processing alone are unable to resolve?

# Contribution review

## (CE13: Projection formats (8)

Contributions in this category were discussed 13 July 2018 (chaired by J. Boyce).

JVET-K0033 CE13: Summary report on projection formats [P. Hanhart, J.-L. Lin]

This contribution provides a summary report of Core Experiment 13 on projection formats. Seven projection formats are investigated in CE13, 1) padded equi-rectangular projection (PERP), 2) cubemap projection (CMP), 3) equi-angular cubemap projection (EAC), 4) hybrid angular cubemap projection (HAC), 5) modified cubemap projection (MCP), 6) parallel-to-axis uniform cubemap projection (PAU), and 7) rotated sphere projection (RSP). Test results against PERP, CMP, and EAC are provided to show the coding efficiency of each projection format. Test results are provided for both the VTM and BMS configurations. BD-rates are computed based on the end-to-end WS-PSNR metric. Crosschecking results are integrated in this contribution.

EAC was used as an additional anchor.

Comparison of projection formats without padding vs. EAC anchor.

|  | VTM |  |  |  |  | BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| PERP | 13.68% | 8.09% | 8.49% | 94% | 97% | 13.13% | 6.90% | 7.90% | 94% | 98% |
| CMP | 9.40% | 6.15% | 6.43% | 100% | 100% | 9.24% | 6.51% | 6.18% | 100% | 99% |
| HAC | -0.24% | -0.30% | -0.35% | 114% | 100% | -0.33% | -0.36% | -0.40% | 104% | 100% |
| MCP | -0.25% | -0.65% | -0.69% | - | - | -0.63% | -0.92% | -1.06% | - | - |
| PAU | -0.15% | -0.12% | -0.18% | - | - | -0.30% | -0.23% | -0.46% | - | - |
| RSP | 1.39% | 0.32% | 0.21% | - | - | 0.74% | -0.94% | -0.79% | - | - |
| EAC + AFP | -0.31% | -1.09% | -1.14% | 101% | 103% | -0.32% | -1.28% | -1.06% | 100% | 100% |
| HAC + AFP | -0.46% | -1.32% | -1.41% | 116% | 102% | -0.56% | -1.63% | -1.45% | 104% | 101% |
| CMP + AFP | 9.06% | 5.16% | 5.23% | 96% | 105% | 8.90% | 5.29% | 5.24% | 93% | 99% |
| MCP + AFP | -0.53% | -1.66% | -1.82% | 99% | 102% | -0.90% | -2.14% | -2.17% | 102% | 101% |
| PAU + AFP | -0.32% | -1.02% | -1.07% | 101% | 105% | -0.44% | -1.33% | -1.31% | 102% | 101% |

JVET-K0131 CE13: Modified Cubemap Projection in JVET-J0019 (Test 5) [Y.-H. Lee, J.-L. Lin, S.-K. Chang, C.-C. Ju (MediaTek)]

Discussed as part of the CE review.

JVET-K0182 CE13: Parallel-to-Axis Uniform cubemap projection (PAU) in JVET-J0033 (Test 7) [Y. Sun, X. Huangfu, B. Wang, L. Yu (Zhejiang Univ.)] [late]

Discussed as part of the CE review.

JVET-K0328 CE13: Cubemap projection (Tests 2.1 and 2.2) [P. Hanhart, Y. He, Y. Ye (InterDigital)]

Discussed as part of the CE review.

JVET-K0329 CE13: Equi-angular cubemap projection (Tests 3.1 and 3.2) [P. Hanhart, Y. He, Y. Ye (InterDigital)]

Discussed as part of the CE review.

JVET-K0330 CE13: Hybrid angular cubemap projection (Tests 4.1 and 4.2) [P. Hanhart, Y. He, Y. Ye (InterDigital)]

Discussed as part of the CE review.

JVET-K0331 CE13: Adaptive frame packing (Tests 4.3 and 4.4) [P. Hanhart, Y. He, Y. Ye (InterDigital)]

Discussed as part of the CE review.

JVET-K0387 CE13: Rotated Sphere Projection (Tests 8.1, 8.2 and 8.3) [C. Pujara, A. Singh, A. dddKonda (Samsung)] [late]

Discussed as part of the CE review.

## Normative coding tools for 360 video

Contributions in this category were discussed in the BoG 13 July 2018 (chaired by J. Boyce).

JVET-K0141 AHG8: 360°-based inter/intra prediction for cubemap projection [C.-H. Shih, J.-L. Lin, H.-C. Lin, S.-K. Chang, C.-C. Ju (MediaTek)]

This contribution proposes a 360°-based inter/intra prediction for cubemap projection. This proposed method modifies the referenced pixels in inter and intra prediction by using the geometric neighboring pixels when those referenced pixels are outside the frame boundary (layout boundary) or across the discontinuous edge(s) in the projection layout. As compared to the conventional inter and intra prediction applied to the cubemap layout, the experimental results reportedly demonstrate that the proposed method, in terms of end-to-end WS-PSNR-Y/U/V, achieves -0.97%/-1.26%/-1.28% BD-rate reduction, under VTM-1.0, respectively.

Has a 30% decoder runtime increase. For inter, requires decoder to do interpolation for geometric padding. Had previously proposed in JVET-E0057 a simpler padding approach for CMP with copying, similar to K0333 for ERP.

For intra prediction, just do copy with 90 degree rotation.

Don’t have a comparison showing proposal vs. padding.  The CE used evil viewports with rotation encoding for subjective viewing, while this contribution did not do the pre-rotation for evil viewports before encoding.

Suggest further study in a CE, where would want to compare against an anchor using conventional padding.

It was suggested that decoder complexity could be reduced by using bilinear interpolation for the inter-prediction across cube faces.

Need to upload slides.

JVET-K0142 AHG8: 360°-based in-loop filters for cubemap projection [S.-Y. Lin, L. Liu, C.-H. Shih, J.-L. Lin, H.-C. Lin, S.-K. Chang, C.-C. Ju (MediaTek)]

This contribution proposes two 360°-based in-loop filtering methods that modify the conventional in-loop filters for appropriately filtering on the cubemap layout. One of them is to disable in-loop filtering processes at the discontinuous edge, while the other one uses the property of geometric continuity in 360° video to properly filter the pixel data located at the geometric positions. As compared to the conventional in-loop filters applied to the cubemap layout, the experimental results demonstrate that the two proposed methods under VTM-1.0, in terms of end-to-end WS-PSNR-Y, achieve 0.19% and 0.26% BD-rate reduction, respectively. In addition, the proposed methods noticeably enhance subjective quality by alleviating the seam artifacts.

Method 1 disables filters at face seam edges, both deblocking and SAO.

Method 2 changes the filtering process to use spherical spatial neighbors (with copy and rotate). Will have some memory access differences, since can’t just use line buffers for above spatial neighbors in coded frame.

Method 2 has a slight coding gain and a small subjective improvement over Method 1.

No comparison is made to conventional padding. In the CE, should study conventional padding with Method 1 (disabling filtering), and use blending.

Need to upload slides.

JVET-K0466 Cross-check of JVET-K0142: AHG8: 360°-based in-loop filters for cubemap projection [P. Hanhart (InterDigital)] [late] [miss]

Noted.

JVET-K0183 AHG8: Face boundary filtering for 360° video [Xuchang Huangfu, Yule Sun, Bin Wang, Lu Yu (Zhejiang Univ.)] [late] [miss]

This contribution proposes a face boundary filtering method for 360 video to reduce the seam artifacts in reconstructed viewports. Blocks which are adjacent on the sphere but nonadjacent in the frame-packed image are filtered. The subjective quality of reconstructed viewports has been improved significantly after filtering. The performance of in-loop filtering and post-filtering is similar with each other objectively and subjectively.

Need to upload slides.

First proposed method is similar to K0142. Only impacts deblocking filter, and not SAO.

Used PAU with padding. Used 3 samples * 2 of padding, in just the middle section between the face rows, and not on the top and bottom.

Studied two methods – inloop or post filtering. Recommends the post filtering approach to avoid changing the core decoder.

Very small objective gains, using an anchor with padding. Claims subjective improvement, which should be checked during subjective viewing.

Suggest to further study in a CE.

JVET-K0333 AHG8: Horizontal geometry padding for PERP [P. Hanhart, Y. He, Y. Ye (InterDigital)]

This proposal investigates the performance of horizontal geometry padding (JVET-E0065) over conventional repetitive padding at the left and right boundaries for the padded equirectangular projection (PERP). Similar to geometry padding (JVET-D0075), the horizontal geometry padding scheme considers the intrinsic cyclic property of 360° content in the horizontal direction when a reference sample is outside of the picture in motion compensated prediction, but reuses the repetitive padding method in HEVC in the vertical direction. Experimental results show that horizontal geometry padding achieves average BD-rate reduction of 0.16% and 0.19% for VTM and BMS, respectively, at a similar complexity as repetitive padding. More importantly, it is reported that horizontal geometry padding significantly reduces the seam artifact issue for the Chairlift sequence.

Proposes motion compensation prediction for PERP across seam edge. Largest gain on moving sequences, 0.41% on SkateboardInLot and 0.72% on ChairLift. Can be implemented by a decoder on a frame basis or on the fly, as an implementation choice.

Asserts subjective quality improvement on seam, for ChairLift sequence. Sequence dependent results – for other moving sequences padding in PERP was already sufficient. Will do informal subjective viewing.

Wasn’t tested for CMP formats, but could be applied.

Suggest to study in a CE.

JVET-K0404 AHG8: Selective In-loop filtering for 360 Video Compression [C. Pujara, S.N. Akula, A. Singh, R. Narayana, W. Choi (Samsung)] [late]

This contribution presents an adaptive in-loop filtering for 360◦ video. In case of 360◦ projection, spherical neighbors need to be re-arranged for compact representation and spatial similarity assumption does not hold true near the boundaries of the non-spatial neighbor faces which are placed next to each other. Current in-loop filtering across these boundaries in VTM makes bleed across the seam boundaries causing visible artifacts. In this contribution, avoiding in-loop filtering across these regions is proposed. Results show visible improvement in subjective quality with marginal gain in objective quality improvement due to better reference pictures.

Only disabled deblocking filter, not SAO. Implemented on VTM, not BMS, so didn’t study ALF.

Similar to K0142 Method 1.

Suggest to study in a CE.

JVET-K0332 CE13-related: Adaptive frame packing on top of CMP, MCP, and PAU [P. Hanhart, Y. He, Y. Ye (InterDigital)]

This contribution reports simulation results for adaptive frame packing (AFP, JVET-K0331) on top of CMP, modified cubemap projection (MCP, JVET-K0131), and parallel-to-axis uniform cubemap projection (PAU, JVET-K0182). Results based on end-to-end WS-PSNR show that AFP provides luma BD-rate savings of about 0.3%, 0.3%, and 0.15% on top of CMP, MCP, and PAU, respectively. There is no significant encoding or decoding complexity increase due to AFP.

Motion compensated prediction can be done in 2-step process, fetching out-of-face samples before the sub-pel interpolation. Requires normative decoder change.

Suggest to study in a CE.

# Observations from informal subjective viewing

Seam artifacts for EAC were slightly more visible than for CMP, when no padding or tools were applied.

RSP padding wasn’t enough to eliminate visible line at apex point.

HAC with adaptive packing w/o any padding or tools had more visible seam than CMP.

Moving sequences had more visible subjective artifacts at face seams than static scenes.

Padding was insufficient to remove subjective artifacts, even if blending was used.

For MCP w/o padding (from K0142), for moving camera scene, disabling deblocking (Method 1) or using spherical neighbors for deblocking (Method 2) was insufficient. Method 2 had a different type of subjective artifact around seam for moving sequences (especially Chairlift). Both methods removed the visible line on the seam, but didn’t fully eliminate the seam discontinuity artifact.

Using spherical neighbors for inter/intra prediction (from K0141) reduced subjective artifact, but didn’t always completely eliminate it.

PERP inter-prediction using horizontal geometry padding (from K0333) improved subjective quality by removing artifact in QP=37 Chairlift sequence. Also improvement for QP=32 but artifact was less visible anyway in the anchor.

For PAU w/ padding (from K0183), in-loop deblocking with spherical neighbors was better than post-filter deblocking with spherical neighbors at reducing subjective artifact. Both methods did improve quality.

RSP w/o blending (from K0404) with disabling of deblocking filter reduced subjective artifact. the apex artifact was still visible, but less so.

Will be interesting in a CE to combine some of the techniques to see if seam subjective artifacts can be fully eliminated.

Subjective viewing of the CEs was limited to the 5 sequences used in the CfP, which seemed to be reasonable for subjective viewing. Landing and BranCastle were used for the non-CE tests, but were not good for use with subjective viewing because of unsteady motion.

Subjective viewing was only done for the middle vertex for evil viewports.

# Discussion

The CE results show that get higher gains for different projection formats vs. PERP for BMS than for VTM, which is opposite that generally observed for coding tools. Perhaps this is because the more advanced prediction tools of BMS can take better advantage of the less distorted representation of EAC vs. ERP.

How to signal projection format? SPS VUI or SPS? Depends on if normative decoder changes are adopted or not.

Would be interesting to study GPU complexity implications of projection mapping. Should be an AHG mandate. How to assess the post-decode rendering complexity of different projection formats? We don’t report runtimes of 360Lib conversions.

EAC has some industry support from Google, announced on Google blog, may be used in YouTube. Was originally proposed by Minhua, because of industry support.

The tested projection format with the largest coding gain is MCP, which is 0.63% better than EAC for BMS.

Pre-rotation was tested for RSP, 1.4% gain on average.  Potential gain for EAC-type formats is lower than for ERP. Could do SEI message or VUI to signal rotation, doesn’t impact decoding process.

Padding amounts tested in CE:

- Test 5.2 used blending w/ MCP padding. 4 pixels on face row boundaries. 0.2% bitrate penalty for padding + blending.

- Test 7.2 used padding w/o blending. Only padded in between the face rows, and not on the picture edges. Lower penalty because fewer extra padding pixels. 3 pixels * 2 on face row boundary.

- Test 8.3 used 8 pixels of padding on edges with RSP.

- Test 4.3 and 4.4 used adaptive face packing, which requires normative decoder change on IRAP pictures.

For subjective viewing, want to evaluate padding vs. no padding. Focus on vertex between face rows.

Interest in defining a CE. Perhaps should define an anchor with projection format with better coding efficiency and w/ padding and blending. Want to demonstrate how coding tools perform relative to approaches that don’t require decoder changes such as conventional padding.

For evil viewports, now that we are comparing cube-type projections, rather than ISP, OHP, etc., pre-rotation is not necessary, and the test conditions can reflect that. May need to redefine the RSP evil viewport vertices to align with cube formats.

What would it mean to “adopt” a projection format in JVET?  Would need some syntax to signal projection format.

MCP, PAU, and HAC are not currently in 360Lib. EAC and RSP are in 360Lib.

Should we address the seam subjective artifact problem before we try to adopt projection formats? Probably yes.

Should a particular projection format be used for coding tool experiments? EAC and MCP are the leading candidates.

Padding should be configurable – amount of padding, and whether padding is just in the center between the face rows, or also above and below the face rows on the picture edges.

BoG recommends to add MCP to 360Lib. Should add configuration options for flexible padding, including number of padding pixels, and which locations to apply padding (just between face rows vs. around the face rows including the picture edges.)

BoG recommends to use the MCP format with padding of 4 samples around face row with blending (PMCP) as in Test 5.2 for coding tool experiments in the CE and the CTC. Replace the ERP and CMP anchors with MCP. Will still ask AHG6 to provide PERP and CMP results for VTM and BMS.

BoG recommends a 360 video Core Experiment be defined to study removing subjective artifacts at face boundaries, and comparing normative decoding tools with non-normative pre- and post-processing (such as padding, cropping, blending, post-filtering). CE coordinators: P. Hanhart and J.-L. Lin.

Include in CE: inter and intra prediction using spherical neighbors, deblocking filter disabling and using spherical neighbors, post-filtering, combinations with padding and with other coding tools. Different padding widths.

BoG encourages study of signaling of projection format parameters, coding tools parameters, and post-filtering hints. Include as an AHG mandate.

BoG encourages study of GPU complexity impact of rendering (including possible post-processing) of projection formats.

Ask track for some guidance on normative tool adoption for 360 video. What complexity impact could be acceptable to remove subjective artifacts that pre- and post-processing alone are unable to resolve?